3GPP TSG-SA WG6 Meeting #66	S6-251408

Gothenberg, Sweden 7th – 11st April 2025	(revision of S6-251101)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0381 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the procedure of SCAI provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add a note to explain how the VAL UE benefits the SCAI info provisioning from the application enabled layer. And correct the number of NOTE in clause 21.3.2.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a NOTE in clause 21.2.2.1 - Correct the number of NOTE in clause 21.3.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specific will not have a good quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21.3.2.3	NRM server provisioning the S&F configuration procedure

Figure 21.3.2.3-1 shows how the NRM Server provides the S&F configuration to the NRM Client based on the service request from VAL server and the VAL UE reported satellite information.

This procedure also applies when the NRM server provides the updated S&F configuration to the NRM client.

Pre-condition:

-	The NRM client and the NRM server are deployed on the ground.

-	The PGW-U in 3GPP CN is deployed on the satellite.

-	The NRM Client has reported the UE satellite information to the NRM server as described in clause 21.3.2.2.

Figure 21.3.2.3-1: NRM Server providing the S&F configuration to the NRM Client procedure

1.	The NRM server takes advantage of the service request initiated by the VAL server, the reported UE satellite information (e.g. serving satellite ID, the maximum data storage quota per UE) as described in clause 21.3.2.2 and its pre-configuration, etc. to determine the on-board S&F parameters for the application layer (e.g., maximum S&F data storage quota per UE/service, maximum S&F data retention per UE/service, etc.).

2.	After determined the satellite S&F parameters, the NRM server provides such information to the NRM client via sending a satellite S&F configuration request.

3.	The NRM client/VAL UE stores the received satellite S&F parameters and may take them into account when the VAL UE starts to operate the S&F operations based on its satellite S&F configuration per service.

NOTE:	The VAL UE may consider the pre-configuration, the received parameters from the application enabled layer to determine/update its satellite S&F configuration per service.

4.	The NRM client sends the satellite S&F configuration response to the NRM server.

* * * End of Changes * * * *