3GPP TSG-SA WG6 Meeting #66	S6-251412

Sweden, Goteborg 7th – 11th April 2025	rev of S6-251215

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0384 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update satellite coverage availability information by also considering UE's mobility. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_APP |  |  |  |  |  | Date: |  |  | 2025-3-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, the configuration management server provides the satellite coverage information based on the current UE location. This has limitation due to: - UE may move to different location due to its mobility and the provided satellite coverage information may not be valid due to the mobility and the UE needs to request the information again. - Configuration management server may provide all the satellite coverage information that may not be required for the UE. To overcome this limitation, it is proposed that the configuration management server receive UE mobility analytics from the NWDAF (which is already well defined) and consider this information to derive the satellite coverage information applicable to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 21.2.2.1 is updated to clarify that configuration management server may receive UE mobility analytics information from the NWDAF to provide a more relevant satellite coverage information to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This leads to inefficient operations, resources wastage and unnecessary signalling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 21.2.2.1	Procedures of the satellite coverage availability information provisioning

The procedure for VAL UE obtaining the satellite coverage availability information configuration for a particular location of UE is illustrated in figure 21.2.2.1-1.

Pre-conditions:

-	The VAL UE has registered to the 3GPP network via satellite access as defined in clause 5.3.2.1 of 3GPP TS 23.401 [9] or clause 4.2.2.2.2 of 3GPP TS 23.502 [11].

Figure 21.2.2.1-1: The satellite coverage availability information provisioning

1.	The configuration management server obtains the satellite coverage availability information locally or from the satellite service provider, and may interact with NWDAF to obtain the target VAL UE’s mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [34] and may interact with the SEAL location management server to obtain the VAL UE’s information (e.g., location) in order to provide accurate information to the UE. If the configuration management server receives the different kinds of satellite information from different satellite information providers for the same satellite, it may consolidate them in a unified format. The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [10].

2.	The configuration management server sends the satellite coverage availability information configuration request to the configuration management client to indicate whether the coverage is available for a particular location and time when the VAL UE uses the satellite access.

3.	The configuration management client acknowledges the configuration management server if it supports the satellite access.

* * * End of Changes * * * *