3GPP TSG-SA WG6 Meeting #66	S6-251417

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251238)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0290 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group user consent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Proposes solution as per conclusion of Key Issue#1 in TR 23.700-22. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TR 23.700-22, regarding solution#18 conclusions of Key Issue #1 states: Solution#18 can be considered as candidate for normative work for enabling resource owner’s consent in the context of group. The CR proposes new procedure, to enable resource owner’s consent in the context of group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for resource owner’s consent in the context of group will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-250576, S6-250471, S6-250247 |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

### 8.34.3		Procedure

Figure 8.34.3-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs that belong to the same group.

Pre-condition:

1.	The network hosts individual resources for a group of UEs which can be accessed via AEF provided the necessary security conditions are met.

2.	UE 1 and UE 2 belong to the same group and the same VAL service provider (IoT case) or the same user.

3.	CCF is provisioned with the information of a Group Resource Owner (GRO) which indicates UE 1 as the authorized RO (e.g. by business relationship between the group of UEs) to provide resource owner authorization for the resources belonging to the UEs in this group.

NOTE:	In this release of the specification group context information is provisioned by the CAPIF administrator as specified in Annex E and the other mechanisms are out of scope of this release.

Figure 8.34.3-1: UE-deployed API invoker accessing other UEs’ resources of a group

1.	The API invoker (e.g., in UE 2) sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API for other UE's resources hosted in the network (e.g., location) by including the API invoker identity information and any information required for authentication of the API invoker. The request includes API invoker information, the group identifier, and the UE in a group whose resources are to be accessed. Alternatively, the request shall include the API invoker information (identity of UE2). Additionally, the API invoker may include in the request information about the purpose and operations (scope) to be performed on the targeted resources.

Editor’s note: How the alternatives can be chosen by API invoker is FFS.

2.	CCF performs authentication of the API invoker (using authentication information) as specified in 3GPP TS 33.122 [12].

3.	CCF, based on the group identifer determines that RO authorization is provided by a GRO. The CCF then resolves the identity of the GRO responsible for the group of UEs based on the provisioned group context information related to the GRO, as specified in Annex E. If the UE2 identity (API invoker) was received in step 1, the CCF shall additionally check whether UE2 belongs to the group.

4.	If step 3 is successful, the CCF performs the resource owner authorization check using the GRO as the RO for the requested resources of other UE(s) belonging to the group.

Editor’s note:	The authorization details for step 4 of how CCF interacts with GRO to obtain authorization which also includes the group identification is FFS and will be specified by SA3.

5.	Based on the group resource owner authorization received in step 4, the CCF sends an Obtain service API authorization response to the API invoker. The authorization response indicates that the access to UE (resource owner) network resource(s) is authorized for an indicated scope.

NOTE:	In this release of the specification, the authorization revokation due to the removal of any of the UE(s) from the group is triggered by the GRO.

6.	The API invoker sends service API invocation request to the API exposing function with the RO authorization information received in step 5.

7.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the RO authorization information

# /************************ End of changes ******************************/