3GPP TSG-SA WG6 Meeting #66	S6-251424

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-250519, S6-250513)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0373 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consolidation of Reserve Network Resource and Request Unicast Resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL, TEI19 |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Network Resource Adaptation and Request Unicast Resource service operations are intended to allocate resources in the network according to the requested capabilities, being the main difference between them the target users of a request. SA6 has discussed and agreed that both operations can be consolidated into one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Consolidate Network Resource Adaptation and Request Unicast Resource operations into the Network Resource Adaptation operation. - Keep the semantics of the existing procedures where the Request Unicast Resource operation was used, i.e: - limit the target UEs of the request to a single VAL UE ID or VAL User ID, - Indicate that during the modification of the requested unicast resource, the VAL server needs to indicate the VAL UE ID or VAL User ID whose resources need to be modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear procedures to manage the request for allocation of network resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.1, 14.3.2.2, 14.3.2.2a (new), 14.3.2.2b (new), 14.3.2.6 (void), 14.3.2.7 (void), 14.3.2.8 (void), 14.3.2.9 (void), 14.3.3.1, 14.3.3.2.1.2, 14.3.3.2.2.1, 14.3.3.2.2.2, 14.3.3.3.1.2, 14.3.3.3.2.2, 14.3.3.3.3.1, 14.3.3.3.3.2, 14.4.2.2a (new), 14.4.2.3 (void), 14.4.2.4 (void) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Resubmission of SA6#65 agreed CR due to being omitted from SA#107 plenary cycle |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.1	Network resource adaptation request

Table 14.3.2.1-1 describes the information flow network resource adaptation request from the VAL/SEALDD server to the NRM server.

Table 14.3.2.1-1: Network resource adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs for whom the network resource adaptation occurs. |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource adaptation occurs. |
| Resource adaptation requirement | M | The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource, VAL service ID). |
| BDT Reference ID | O | Indicates the Background data transfer Reference ID provided by the 3GPP network |
| NOTE: Either of the information elements should be present. |  |  |

* * * Next Change * * * *

#### 14.3.2.2	Network resource adaptation response

Table 14.3.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server.

Table 14.3.2.2-1: Network resource adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource adaptation with the underlying network. The response may also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering) |

* * * Next Change * * * *

#### 14.3.2.2a	Network resource adaptation modification request

Table 14.3.2.2a-1 describes the information flow for the network resource adaptation modification request for a VAL UE ID or VAL user ID from VAL server to NRM server for unicast resources.

Table 14.3.2.2a-1: Network resource adaptation modification request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL user IDs or VAL UE IDs | O (see NOTE) | List consisting of one or more VAL user IDs or VAL UE IDs for whom the network resource adaptation modification occurs. |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource adaptation modification occurs. |
| Resource adaptation requirement | O | VAL service requirements for unicast resource (e.g. VAL sevice ID, Bitrate) |
| BDT Reference ID | O | Indicates the Background data transfer Reference ID provided by the 3GPP network |
| NOTE: Either of the information elements should be present. |  |  |

* * * Next Change * * * *

#### 14.3.2.2b	Network resource adaptation modification response

Table 14.3.2.9-1 describes the information flow for the network resource adaptation modification response from NRM server to VAL server for unicast resources.

Table 14.3.2.2b-1: Network resource adaptation modification response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource modification operation. |

* * * Next Change * * * *

#### 14.3.2.6	void

* * * Next Change * * * *

#### 14.3.2.7	Void

* * * Next Change * * * *

#### 14.3.2.8	Void

* * * Next Change * * * *

#### 14.3.2.9	Void

* * * Next Change * * * *

#### 14.3.3.1	General

The following subclauses specify the procedures for unicast resource management for vertical application layer. The NRM server sets up bearers and may need to modify the bearers for an already established VAL service communication.

Characteristics that may need to be modified include:

-	activation and deactivation of the bearer;

-	modification of the QoS characteristics of the bearer (e.g. bearer priority adjustment); and

-	modification of GBR due to application requirement

NOTE:	A VAL service communication can consist of both unicast and multicast bearers which can all need modification due to the same event.

Vertical application layer specific pre-requisites and resultant behaviour by functional entities in performing these procedures are specified in the respective vertical application layer TS (e.g. for V2X application layer, see 3GPP TS 23.286 [7]).

Unicast resource management is supported with NRM server interactions with SIP core and NRM server interactions with PCC. The PCC procedures for EPS are specified in 3GPP TS 23.203 [18] and the PCC procedures for 5GS are specified in 3GPP TS 23.503 [19].

* * * Next Change * * * *

###### 14.3.3.2.1.2	Procedure

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 14.3.3.2.1.2-1 are the signalling procedures for the requesting resource at session establishment.

Pre-condition:

-	The VAL client has requested VAL service communication with the VAL server.

Figure 14.3.3.2.1.2-1: Resource request at VAL service communication establishment

1.	The VAL server sends request for network resource adaptation to the NRM server. The request includes a VAL UE ID or a VAL user ID.

2.	The NRM server evaluates the need for network resources and use of resource sharing.

3.	The NRM server sends a session progress request containing request for resources.

4.	PCC procedures are initiated from SIP core local inbound/outbound proxy.

5.	The SIP core local inbound / outbound proxy sends a OK message to the NRM server.

6.	The NRM server sends a network resource adaptation response to the VAL server.

7.	The VAL service communication is established, and resources have been allocated.

* * * Next Change * * * *

###### 14.3.3.2.2.1	General

To modify unicast bearers, the NRM server shall send a network resource adaptation request containing the parameters to be modified for the UE.

Possible scenarios when this procedure may be used are:

-	Modify the allocation and retention priority for unicast resources;

-	Release and resume resources in-between VAL service communications; or

-	Release and resume resources when a UE is able to receive the VAL service communications over multicast transmission

###### 14.3.3.2.2.2	Procedure

Procedures in figure 14.3.3.2.2.2-1 are the signalling procedures for the modification of a unicast:

Pre-condition:

-	A VAL service communication is already in progress;

-	The VAL server requested network resources for the VAL UE ID, VAL User ID.

Figure 14.3.3.2.2.2-1: Bearer modification request

1.	The VAL server sends a network resource adaptation modification request to the NRM server. The request includes a VAL UE ID or a VAL user ID whose resources need to be modified.

2a.	The NRM server decides to modify the parameters of a unicast bearer.

2b.	If the media bearer modification is not required, the NRM server sends a network resource adaptation modification response to the VAL server.

3.	The NRM server sends a session update which includes a resource modification request containing the modified parameters of the unicast bearer.

4.	PCC procedures are initiated from SIP core local inbound/outbound proxy.

5.	The SIP core local inbound / outbound proxy sends a OK message to the NRM server.

6.	The NRM server sends a network resource adaptation modification response to the VAL server.

7.	The VAL service communication continues with the modified unicast resources.

NOTE 1:	If the VAL service communication is transferred to multicast transmission, the unicast resources could be temporarily released.

NOTE 2:	If multiple VAL service communication streams are sent to the UE, additional bearer resources could be required during an established VAL service communication. Pre-allocation of additional bearer resources already at VAL service communication establishment could be useful.

* * * Next Change * * * *

###### 14.3.3.3.1.2	Procedure

Figure 14.3.3.3.1.2-1 illustrates the procedure for the network resource adaptation.

Figure 14.3.3.3.1.2-1: Network resource adaptation

1.	The VAL server (or SEALDD server) sends a network resource adaptation request to the NRM server for one or more users belonging to one or more VAL services, and may comprise one or more VAL UEs that will have updated resource requirement. This requirement may be in the form of exact resources /resource pools to be used or indication of bandwidth increase/decrease for the corresponding VAL UEs or groupof VAL UEs. The BDT Reference ID may also be included for setting BDT policies.

NOTE:	The SEALDD server can use the NRM netwok resource adaption service as per the procedure defined in 3GPP TS 23.433 clause 9.11 SEALDD Background data transfer.

2.	The NRM server processes the request and applies / enforces the resource adaptation per VAL UE.

3.	The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status) and initiates the PCC procedures for each VAL UE.

4.	The NRM server provides a network resource adaptation response to the VAL server, providing information on the fulfilment of the network resource adaptation request. This will include information either per VAL UE or per set of VAL UEs, as indicated by the request of the VAL server in step 1.

* * * Next Change * * * *

###### 14.3.3.3.2.2	Procedure

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 14.3.3.3.2.2-1 are the signalling procedures for the requesting resource at session establishment.

Pre-condition:

-	The VAL client has requested VAL service communication with the VAL server.

Figure 14.3.3.3.2.2-1: Resource request at VAL service communication establishment

1.	The VAL server sends a network resource adaptation request to the NRM server for a VAL UE or a VAL user ID.

2.	The NRM server evaluates the need for network resources and use of resource sharing.

3.	The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status), and then PCC procedures are initiated from NRM server.

4.	The NRM server sends a network resource adaptation response to the VAL server.

5.	The VAL service communication is established, and resources have been allocated.

* * * Next Change * * * *

###### 14.3.3.3.3.1	General

To modify unicast resources, the NRM server shall send a network resource adaptation modification request containing the parameters to be modified for the UE.

Possible scenarios when this procedure may be used are:

-	Modify the allocation and retention priority for unicast resources;

-	Release and resume resources in-between VAL service communications; or

-	Release and resume resources when a UE is able to receive the VAL service communications over multicast transmission

###### 14.3.3.3.3.2	Procedure

Procedures in figure 14.3.3.3.3.2-1 are the signalling procedures for the modification of a unicast:

Pre-condition:

-	A VAL service communication is already in progress;

-	The VAL server requested network resources for the VAL UE ID, VAL User ID.

Figure 14.3.3.3.3.2-1: Bearer modification request

1.	The VAL server sends a network resource adaptation modification request for a VAL UE ID or VAL user ID to the NRM server.

2a.	The NRM server decides to modify the parameters of a unicast bearer.

2b.	If the media bearer modification is not required, the NRM server sends a network resource adaptation modification response to the VAL server.

3.	PCC procedures are initiated from NRM server.

4.	The NRM server sends a network resource adaptation modification response to the VAL server.

5.	The VAL service communication continues with the modified unicast resources.

NOTE 1:	If the VAL service communication is transferred to multicast transmission, the unicast resources could be temporarily be released.

NOTE 2:	If multiple VAL service communication streams are sent to the UE, additional bearer resources could be required during an established VAL service communication. Pre-allocation of additional bearer resources already at VAL service communication establishment could be useful.

* * * Next Change * * * *

#### 14.4.2.2a	Reserve_Network_Resource_Modify operation

API operation name: Reserve_Network_Resource Modify

Description: Requesting for the modification of network resource adaptation.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.2a

Outputs: See subclause 14.3.2.2b

See subclause 14.3.3 for the details of usage of this API operation.

* * * Next Change * * * *

#### 14.4.2.3	Void

* * * Next Change * * * *

#### 14.4.2.4	Void

* * * END of Change * * * *