3GPP TSG-SA WG6 Meeting #66	S6-251447

Göteborg, Sweden, 07th – 11th April 2025	(revision of S6-251070, S6-251391)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0729 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correstions for Eees_EASInformationProvisioning and Eees_EASDiscovery APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Misalignments exist in the specification in the service description clauses and the procedures: - In clause 8.15.2.2, only EEC is as consumer of the Eees_EASInformationProvisioning API, the same also in clauses 8.15.4.1, 8.15.4.2, 8.15.3.2, 8.15.3.3, and 8.15.2.2. EES as consumer of this service API is introduced in clause 8.18.2.4.5, but missing in clause 8.15. - In clauses 8.8.1.1, 8.8.2.4, 8.8.3.2, and 8.8.4.2, it described that EAS sends requests to EES for EAS discovery, reuse the mechanism introduced in clause 8.5. Thus, EAS is also consumer of the Eees_EASDiscovery API. - In clauses 8.8.2A.1 and 8.8.2A.6, CAS may send EAS discovery requests tot EES. Thus, CAS is also consumer of the Eees_EASDiscovery API. This paper proposes to align the consumer of the Eees_EASInformationProvisioning API and the Eees_EASDiscovery API, in the service description clauses and the procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Add description about EES as consumer of Eees_EASInformationProvisioning API to clause 8.15. - Added EAS and CAS to the consumer list of Eees_EASDiscovery API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Descripiton about EES as consumer of Eees_EASInformationProvisioning API is missing. Misalignments exist in the specification in the service description calsues and the procedures for the Eees_EASDiscovery API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.5.1, 8.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES, CES | 8.8.3.3 |
| Eecs_ECSRegistration | ECS | 8.17.2.2 |
| Eecs_ECSDiscovery | ECS | 8.17.2.3 |
| Eecs_ECSServiceProvisioning | ECS | 8.17.2.4 |
| Eecs_EASInfoManagement | EES | 8.20.2 |
| Eecs_ACREvents | EES | 8.8.3.11 |

Table 6.7.2-2 summarizes the APIs exposed by the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC, EAS, CAS | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS, EES, CAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | EAS | 8.6.7 |
| Eees_TargetEASDiscovery | EAS, EES, CAS | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS, EES | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EES, EAS | 8.8.3.6 |
| Eees_SelectedTargetEAS | EAS, CAS, EES | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS, CAS, EES | 8.8.3.8 |
| Eees_ACRParameterInformation | EES, CES | 8.8.3.9 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES, CES | 8.9.4.3 |
| Eees_EASInformationProvisioning | EEC, EES | 8.15 |
| Eees_CommonEasAnnouncement | EES | 8.19 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

Table 6.7.2-3 summarizes the APIs exposed by the CAS.

Table 6.7.2-3: APIs provided by the CAS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Ecas_SelectedEES | EES | 8.8.3.10 |

Table 6.7.2-4 summarizes the APIs exposed the EEC.

Table 6.7.2-4: APIs provided by the EEC

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eeec_ACRegistration | AC | 8.14.4.2 |
| Eeec_EASDiscovery | AC | 8.14.4.3 |
| Eeec_ACRTrigger | AC | 8.14.4.4 |
| Eeec_Services | AC | 8.14.4.5 |
| Eeec_UEId | AC | 8.14.4.6 |

Table 6.7.2-5 summarizes the EES APIs re-used by the CES.

Table 6.7.2-5: APIs re-used by the CES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EASRegistration | CAS | 8.4.3 |
| Eees_UELocation | CAS | 8.6.2 |
| Eees_ACRManagementEvent | CAS | 8.6.3 |
| Eees_AppClientInformation | CAS | 8.6.4 |
| Eees_UEIdentifier | CAS | 8.6.5 |
| Eees_SessionWithQoS | CAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | CAS | 8.6.7 |
| Eees_TargetEASDiscovery | EES | 8.8.3.2 |
| Eees_AppContextRelocation | CAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_SelectedTargetEAS | CAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | CAS | 8.8.3.8 |
| Eees_ACRParameterInformation | EES | 8.8.3.9 |
| Eees_EECContextPush | EES | 8.9.4.3 |

* * * Next Change * * * *

### 8.5.1	General

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest.

EAS discovery enables the EEC, EAS, and CAS to obtain information about available EASs of interest (e.g. instantiated EASs registered with the EES and instantiable EASs that may be created when needed). The discovery of the EASs is based on matching EAS discovery filters provided in the request.

When multiple EASs are discovered for a specific AC, the EEC may select one or more EASs to enable AC communication with one of the selected EASs. The selection algorithm is outside the scope of this specification. Once the EAS is selected, the EEC may subscribe for the ACR event notifications at the EES of the selected EAS, as described in clause 8.8.3.5.2. The EDN configuration information received from ECS may be used for establishing a connection to EAS(s).

EAS discovery may be initiated by the EEC when a certain trigger condition at the UE is met. Some examples are as follows:

-	AC related updates available at the EEC (e.g. due to AC installation/re-installation/activation), AC requesting application server access;

-	Lifetime received via EAS discovery response specified in clause 8.5.3 is expired; or

-	EEC detects the need of application context relocation as in clause 8.8.

NOTE:	When the EEC decides to perform EAS discovery is up to EEC implementation.

EAS discovery may also be initiated by the EAS or CAS when a certain trigger condition met, as described in clasues 8.8.3.2 and 8.8.2A.1.

If the EASs consisting of the EAS bundle register to different EESs and if the Bundle Type of the  EAS bundle is set to Direct bundle, then the EEC sends the EAS discovery request message to the associated EES for the bundle EAS information.

* * * Next Change * * * *

### 8.15.1	General

EAS information provisioning procedure allows the EEC to exchange information with the EES about selected EAS and/or ACR scenario selection, and allows the EES exchange informaiton with another EES about selection of the EAS for the service consumption as described in clause 8.18.2.4.5.

When service continuity is required, ACR scenarios may be combined to perform ACR detection in one or more of the EEC, the EES and the EAS; the related procedures are specified in clauses 8.15.2, 8.6.3 and referred to in clause 8.8.2. The selection of ACR scenario(s) may be performed by the EEC or the EES for a given AC and the selected EAS from the common supported ACR scenarios of AC, EEC, selected EES and selected EAS. The selection of ACR scenario(s) for each EAS in EAS bundles may be performed separately by the EEC or the EES (or the main EES in case of bundle EAS served by multiple EESs, which is the EES registering the main EAS) for a given AC and the selected EAS(s) in a bundle, based on the service continuity support by AC, EEC, EES(s) and EAS(s) when the coordinated ACR need to be performed for the bundle EAS. In addition, the EEC or EES can further determine the one or more ACR scenario(s) for the selected EAS based on the AC service KPI.

NOTE:	How to select ACR scenario(s) at the EEC or EES considering AC service KPI is implementation specific.

Instantiable EAS Information may be provided to the EEC in EAS discovery response and EAS discovery notification, as specified in clause 8.5.3.3 and 8.5.3.6. Triggering the instantiation of an EAS by the EEC may be announced to the EES by including the selected EASID in the EAS information provisioning request without including the selected EAS endpoint.

The EAS information provisioning request types supported are:

-	"ACR scenario selection announcement". Inform the EES about the EAS that has been selected by the EEC and may provide the selected ACR scenario list to the EES. For the EAS bundles scenario, the selected bundled EAS(s) and the selected ACR scenario list for EAS bundles by EEC are included and notified to the EES.

-	"ACR scenario selection request". Inform the EES to perform ACR scenario selection. For the EAS bundles scenario, the request may inform the EES to determine the ACR scenario list for EAS bundles.

-	"EAS selection request". Inform the EES of EAS selection, e.g. for Application Group.

* * * END of Changes * * * *