3GPP TSG-SA WG6 Meeting #66	S6-251453

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251428, S6-251337, S6-251164)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0277 | rev | 3 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to update API invoker’s API list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | “Partial success and complete success result will include APIs information that the API invoker can access.” When API invoker updates the API for enrollment, the CCF returns partial success. The behavior is not alignining with onboarding API invoker procedure where the API invoker provides the first list of APIs for enrollment. - Further, partial success/failure was bit confusing where we give list of APIs which API invoker successfully enrolled along with railure reason. - From protocol perspective too, HTTP patch method should be atomic – that is either full change or none. Partial success reponse creates complexity which may not be required for this operation. Partial update may be useful when very large amount of data is being transmitted – which is not the case here. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing partial success response |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure/specification is not aligned with existing procedures (e.g. onborad API invoker) and also confusing with both success and failure information in the response. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.26.2.2, 8.26.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.26.2.2	Update API invoker API list response

Table 8.26.2.2-1 describes the information flow update API invoker API list response from the CAPIF core function to the API invoker.

Table 8.26.2.2-1: Update API invoker API list response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the update operation |
| API information | O (see NOTE 1) | List of APIs and the categories of service APIs that the API invoker can access |
| Reason | O (see NOTE 2) | This element indicates the reason for failure of the operation |
| NOTE 1: Information element shall be present when update API invoker API list status is successful. At least one API successfully enrolled is considered as success response. NOTE 2: Information element shall be present when update API invoker API list has failed . |  |  |

* * * Next Change * * * *

### 8.26.3	Procedure

Figure 8.26.3-1 illustrates the procedure for updating the API invoker API list on the CAPIF.

Pre-conditions:

1.	The API invoker has been onboarded as a recognized user of the CAPIF and associated API invoker profile is provisioned.

2.	The API invoker has visibility to new APIs information (e.g. updates on API catalogue or dashboard, API discovery).

Figure 8.26.3-1: Procedure for updating the API invoker profile on the CAPIF

1.	For updating of the API invoker API list on the CAPIF, the API invoker triggers update API invoker API list request towards the CAPIF core function, providing the information to be updated (e.g. enroll new APIs, disenroll APIs).

2.	The CAPIF core function updates the API invoker API list of the requesting API invoker, according to the grant from the CAPIF administrator or the API management.

NOTE 1:	Completion of updating process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

3.	The update API invoker API list response provides a success or failure indication. When not all the APIs included in the request are enrolled, the success result include information about the APIs that the API invoker can access.  When the update status is failure, the reason for failure is included.

* * * End of Change * * * *