3GPP TSG-SA WG6 Meeting #66	S6-251454

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-251342)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0284 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AEF1 interaction with CCF for token exchange |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.32.3 needs to be updated to clarify the specified mechanisms for token exchange in nested API interactions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of clause 8.32.3 to clarify the AEF-1 interaction with CCF for obtaining AEF-2 authorization information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete procedure for nested API invocations that may lead to misinterpretations and implementation mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.32.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************/

### 8.32.3		Procedure

Figure 8.32.3-1 illustrates the procedure to obtain authorization information in a nested API invocation, in which an API exposing function receiving the service API invocation request interacts with another API exposing function to provide the service.

Pre-conditions:

1.	The resource owner function can communicate with the API invoker.

2.	The API exposing functions 1 and 2 are in the same trust domain.

Figure 8.32.3-1: Procedure for obtaining authorization information in a nested API invocation

1.	The API invoker requests authorization information to invoke the service API exposed by API exposing function 1.

NOTE 1:	This step may use either the existing procedure to obtain authorization to access service API specified in clause 8.11 or the procedure that involves the resource owner function to get authorization information. For the latter case, the mechanisms to support interactions with the Resource owner are specified in 3GPP TS 33.122 [12], with further possible CAPIF support for these mechanisms being out of scope of the current release.

2.	The API invoker sends a service API invocation request to API exposing function 1 with the authorization information received in step 1. The API exposing function 1 verifies the API invoker request.

3.	Based on the service API invocation request, API exposing function 1 decides to invoke another service API exposed by API exposing function 2.

4.	API exposing function 1, acting as an API invoker, before interacting with API exposing function 2, shall obtain from the CCF the authorization information to access the service API exposed by API exposing function 2.

NOTE 2:	To obtain the authorization information, the API exposing function 1 exchanges the authorization information provided in the API invoker's request (in step 2) with authorization information for accessing the service API exposed by API exposing function 2. The security mechanism involved in this exchange are out of scope of the present document.

5.	API exposing function 1, acting as an API invoker sends a service API invocation request to API exposing function 2 with the authorization information received in step 4.

6.	The API exposing function 1 receives the service API invocation response resulting from the service API invocation once API exposing function 2 has checked whether the API exposing function 1, acting as API invoker, is authorized to invoke that service API based on the authorization information.

7.	The API invoker receives the service API invocation response resulting from the service API invocation.

# /******************************* First Change *****************************/