3GPP TSG-SA WG6 Meeting #66	S6-251455

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-251346)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0283 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Revocation on CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following ENs still remain in the specification. Editor's note: It is FFS whether additional IEs need to be included e.g. API invoker information based on SA3 progress on RNAA authorization token revocation. However, there is no dependency with SA3 progress: - SA3 does not specify authorization revocation procedures, but token revocation procedures. - Token revocation procedures do not depend on authorization revocation procedures. Mind that while a token may be valid (i.e. the permissions delegated to the application may still be valid), the API invoker operation on the service API might not be valid (e.g., the API invoker is surpassing the access control policy) - If an authorization token is revoked, the access to the service API may become not authorized, and in this case, the service API authorization revocation procedure may be triggered, which may include as cause authorization token revocation. Therefore, the identity information required in the revocation procedure can be agreed. ENs in clauses 8.25.2.11 and 8.25.2.13 can be rephrased to indicate that whether the identity information is offuscated/masked between CAPIF provider domains is to be discussed in SA3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The ENs in clauses 8.25.2.11 and 8.25.2.13 are rephrased to indicate whether the API invoker identity information is offuscated/masked is to be defined by SA3. - The API invoker identity information is added in the flows for interconnect revoke API invoker authorization response and notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API invoker cannot be identified in the revocation of API invoker authorization in interconnection scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.2.11, 8.25.2.13, 8.25.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.25.2.11	Interconnection revoke API invoker authorization request

Table 8.25.2.11-1 describes the information flow interconnection revoke API invoker authorization request from one CCF to another CCF.

Table 8.25.2.11-1: Interconnection revoke API invoker authorization request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the requesting CAPIF core function, may include identity, authentication and authorization information. |
| API invoker related information | M | Information related to the API invoker, e.g. information that determines the identity of the API invoker. |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

Editor's note:	It is FFS the additional security aspects for the protection of the identity information of the API invoker.

* * * Next Change * * * *

#### 8.25.2.13	Interconnection revoke API invoker authorization notify

Table 8.25.2.13-1 describes the information flow interconnection revoke API invoker authorization notify from one CCF to another CCF.

Table 8.25.2.13-1: Interconnection revoke API invoker authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the notifying CAPIF core function, may include identity, authentication and authorization information. |
| API invoker related information | M | Information related to the API invoker, e.g. information that determines the identity of the API invoker. |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

Editor's note:	It is FFS the additional security aspects for the protection of the identity information of the API invoker.

* * * Next Change * * * *

#### 8.25.3.8	Procedure for CAPIF revoking API invoker authorization in CAPIF interconnection

Pre-conditions:

1.	The API invoker is authenticated by CCF-B and authorized by CCF-B and CCF-A to use the service API. The API invoker has obtained authorization to use the service API as specified in clause 8.25.3.7.

2.	The CCF-B may have available access control policies corresponding to the service API.

3.	The CCF-A is triggered to revoke API invoker authorization for service API access, e.g. AEF requested the revocation of API invoker authorization.

Figure 8.25.3.8-1: Procedure for revoking API invoker authorization in CAPIF interconnection

1.	If the CCF-B keeps service API authorization information for the API invoker, the CCF-A sends an interconnection revoke API invoker authorization request to the CCF-B with the details of the API invoker, the AEF and the service API.

2.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CCF-B invalidates the API invoker authorization corresponding to the service API.

3.	The CCF-B sends an interconnection revoke API invoker authorization response to the CCF-A.

4.	Instead of step 1 to 3, if the CCF-A keeps service API authorization information for the API invoker, the CCF-A sends an interconnection revoke API invoker authorization notify to the CCF-B.

5.	The CCF-A invalidates the API invoker authorization corresponding to the service API.

6.	The CCF-B sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * END of Change * * * *