3GPP TSG-SA WG6 Meeting #66	S6-251461

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251326, S6-251192)

Source:	China Mobile

Title:	New KI on end to end support for multiplexed data flows

Spec:	3GPP TR 23.700-51 v0.0.0

Agenda item:	9.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

Add new KI on end to end support for multiplexed data flows.

2. Reason for Change

XR and interactive media services can send data traffic of different media components with different QoS requirements. Several media flows could be multiplexed on the same end-to-end transport layer connection.

To achieve differentiated QoS for multiplexed media flows, SA2 assumes UE and/or UPF support the IP Packet Filter with (S)RTP Multiplexed Media Identification Information, based on AF provided per media flow the (S)RTP Multiplexed Media Identification Information and the specific QoS requirements. In addition to the (S)RTP,  there is also traffic in the network that uses other protocols for media stream transmission(e.g., SDP ). A more flexible solution can be implemented from the application enablement layer to provide end-to-end QoS guarantees for multiplexed data flows.

This paper proposes to study how to provide end to end services for multiplexed data flows in application enabler layer. It is proposed to study the open issues:

-	Whether and how to identify, provide multiplexed media flows related infoamation adapt for more protocol.

-	Whether and how to provide application enabler layer differentiated QoS service for multiplexed data flows.

----------------changes in 1461----------------

To ensure end-to-end QoS guarantees for multiplexed data flows, an AF session with the required QoS based on Flow Description Information, QoS Reference, or individual QoS parameters provided by the application enabler layer, as defined in TS 23.501, can be used to guarantee QoS in the 5GC for the identified traffic flow. The Flow Description Information is determined by application enabler layer based on negotiation with VAL server. Additionally, SEALDD services, as defined in TS 23.433 (such as end-to-end redundant transmission and SEALDD-enabled data transmission quality guarantees), can be used to ensure QoS from the SEALDD client to the SEALDD server.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 v0.0.0.

* * * First Change * * * *

4.x	Key issue #x: End to end support for multiplexed data flows

XR and interactive media services can send data traffic of different media components with different QoS requirements. Several media flows could be multiplexed on the same end-to-end transport layer connection.

To achieve differentiated QoS for multiplexed media flows, SA2 assumes UE and/or UPF support the IP Packet Filter with (S)RTP Multiplexed Media Identification Information, based on AF provided per media flow the (S)RTP Multiplexed Media Identification Information and the specific QoS requirements. In addition to the (S)RTP,  there is also traffic in the network that uses other protocols for media stream transmission. For example, RTMP and HLS may also be used for live stream transmission. When these protocols are used, SA2’s solution fails to provide multiplexed data flows related service. If the multiplexed media flow can be identified in the application enablement layer, based on negotiation with VAL server, a more flexible solution could be implemented to provide end-to-end QoS guarantees for multiplexed data flows. For example, services like, AF session with required QoS as defined in TS 23.501, SEALDD services as defined in TS 23.433 (e.g., SEALDD enabled rate control and SEALDD enabled E2E redundant transmission, SEALDD enabled data transmission quality guarantee, etc.), could be used to provide end-to-end QoS guarantees for multiplexed data flows.

This Key issue studies how to provide end to end services for multiplexed data flows in application enabler layer. It is proposed to study the open issues:

-	Whether and how to identify, provide multiplexed media flows related information adaptable to more protocols.

-	Whether and how to provide application enabler layer differentiated QoS service for multiplexed data flows.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>