3GPP TSG-SA WG6 Meeting #66	S6-251465

Göteborg, Sweden, 07th – 11th April 2025	(revision of S6-251378,S6-251075)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0025 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor distance compute service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The WID for Metaverse phase 2 S6-250534 mentions Enhancements to Spatial anchor service. Currently in the discovery procedure defined in clause 8.3.4, the response includes list of discovered spatial anchor location information(3D coordinates). For a given range for the spatial anchor discovery in the discovery request, the discovery response could include huge list of spatial anchors. It becomes important for the users to understand how far(in terms of distance) the discovered spatial anchors are, for navigation purpose or user just want to select the closest(e.g. coffee shop spatial anchor). For example, if a user discovers coffee shop spatial anchor in the 2Km radius, user will get all the available coffee shop spatial anchors in the 2Km. If the user wants to understand which coffee shop is closest from its location, as per the discovery procedure the user cannot. There are two ways to find the distance: One method is that the client device can calculate the distance of all the spatial anchors from its location. Just comptuing for one spatial anchor is not a problem. Given the large number of spatial anchors, this could be compute-heavy as the distance calculation would grow linearly, and it negatively impacts the low-end or low compute devices like AR glasses due to low compute resources and may drain battery faster. It also degrades the quality of experience for the users due to the waiting period involved for the calculation of the distances. Another method is to offload the distance calculation to the Spatial anchor server. The spatial anchor server can calculate the distance and send in the discovery response. In this approach, the client can just sort out the list from the discovery response, which is lightweight as compared to computing the distance of all spatial anchors. It saves battery, fast, and enhances the quality of experience for the users. Therefore, this proposal provides the enhancement to the spatial anchor discovery procedure to include the distance information in the discovert response. The proposal also proposes a generic procedure to calculate the distance of the spatial anchors. With the distance information the user can filter out the nearest spatial anchor in the given range. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A generic procedure to compute distance and enhancements to provide distance between the client and the available spatial anchors in the discovery response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users cannot find its distance with the spatial anchors and negative impact on the performance of the user’s device side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.2, 8.3.4.3.2, 8.3.x(new), 8.3.x.1(new), 8.3.x.2(new), 8.3.x.3(new), 8.3.x.3.1(new), 8.3.x.3.2(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.4.2	Procedure

Figure 8.3.4.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.4.2-1: Spatial anchor discovery

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor discovery request message to the SAn server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and can include spatial anchor discovery filters as described in clause 8.3.4.3.1. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request can additionally include a VAL client identifier and location of the client.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies spatial anchor(s) that satisfies the spatial anchor discovery filters.

NOTE 1:	User preferences in the spatial anchor discovery filters can be compared with spatial anchor specific information.

For each matching spatial anchor, the SAn server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the access control rules. For each matching spatial anchor, the SAn server calculates the distance between the requestor’s location and the spatial anchor location information. It can also use the procedure defined in clause 8.3.x to calculate the distance with spatial anchor(s).

To determine the location of interest of the requestor or location of the requestor(client), the SAn server may use location information provided in the request. If location information is not provided in the request, the SAn server may obtain location information by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [6] and 3GPP TS 23.502 [5] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

3.	The SAn server sends a spatial anchor discovery response to the requestor. If the SAn server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors and may includes the recommended list of spatial anchors. Otherwise, the response includes an indication of failure and may include a reason for failure.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the SAn client, the SAn client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

#### 8.3.4.3	Information flows

##### 8.3.4.3.1	Spatial anchor discovery request

Table 8.3.4.3.1-1 describes information elements for the spatial anchor discovery request from the SAn client or VAL server to the SAn server.

Table 8.3.4.3.1-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Location information | O | Indicates the location information of the VAL client when the requestor is a SAn client. |
| Discovery filters | O | The filters used for determining the spatial anchors. |
| > List of spatial anchor ID | O | The list of spatial anchor identifiers to discover. |
| > Area of interest | O | The area of interest (e.g., topological area and/or geographical area). |
| > Service ID | O | The identifier of a VAL service. |
| > Spatial anchor specific information | O | A container to contain application specific information. |

##### 8.3.4.3.2	Spatial anchor discovery response

Table 8.3.4.3.2-1 describes information elements for the spatial anchor discovery response from the SAn server.

Table 8.3.4.3.2-1: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor discovery request was successful. |
| List of spatial anchors | O | List of discovered spatial anchors. |
| > Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| > Position | M | The 3D position of the spatial anchor in space (x, y and z coordinates). |
| > Spatial anchor distance | O | Indicates the computed distance between the spatial anchor location and the requestor location |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > VAL service information | M | Information about the application service associated with the spatial anchor. |
| >> Service ID | M | The identifier of the application service. |
| >> Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| Failure response | O | Indicates that the spatial anchor discovery request has failed. |
| > Cause | O | The cause for the request failure. |

* * * Next Change * * * *

### 8.3.x	Spatial anchor distance compute service

#### 8.3.x.1	General

Spatial anchor get operation enables a SAn client or VAL server to get distance between the spatial anchor(s) from the SAn server. This procedure enables calculation of spatial anchor distance for a moving UE.

#### 8.3.x.2	Procedure

Figure 8.3.x.2-1 illustrates the procedure for Spatial anchor distance compute service following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server;

3.	The SAn client or VAL server has discovered the list of spatial anchors as per the procedure defined in clause 8.3.4.

Figure 8.3.x.2-1: Spatial anchor distance compute service

1.	The requestor (e.g., SAn client or VAL server) sends a Spatial anchor distance compute service subscribe request message to the SAn server to calculate the distance between spatial anchors. The request includes a spatial anchor identifier, location information and a type indicating the source or target spatial anchor. If the requestor is SAn client then the source location spatial anchor may represent UE location.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server sends the subscription response

3.	The SAn server identifies whether a spatial anchor exists or not based on the identity or location information of the spatial anchor. The SAn server may obtain location information of the spatial anchor by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [6] and 3GPP TS 23.502 [5] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

The SAn server tracks the UE location using the procedure defined in clause 9.3.7 TS 23.433 [4] to determine the UE location within the area of interest. Based on the information, the SAn server iteratively calculates the distance between the updated source UE location and target spatial anchor to send in the notification message. If the request includes one or more than one source and target spatial anchor, the SAn server calculates the distance from each source location to all target spatial anchor locations.

Based on the area of interest or spatial anchor location information, the SAn server discovers and identifies the associated spatial map and augments the spatial map with source and target spatial anchor(s) and its distance information.

4.	The SAn server sends a Spatial anchor distance compute service notification to the requestor. The notification includes the spatial anchor distance information between the source and target spatial anchors. It may also include the augmented spatial map grid information with the source and target spatial anchor and its distance information.

#### 8.3.x.3	Information flows

##### 8.3.x.3.1	Spatial anchor distance compute service subscribe request

Table 8.3.x.3.1-1 describes information elements for the Spatial anchor distance compute service subscribe request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor distance compute service subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor |
| > Spatial anchor distance calculation | M | Indicates the details of the source and target spatial anchor. |
| > Spatial anchor identifier(NOTE) | O | Identifier of the spatial anchor or UE. |
| > Spatial anchor location information(NOTE) | O | Indicates the location information of the spatial anchor or UE. |
| > Spatial anchor type | M | Indicates whether the spatial anchor is the source UE/ spatial anchor or target spatial anchor |
| >Area of interest | O | The area of interest (e.g., topological area and/or geographical area). |
| NOTE: Atleast one of the IE shall be present |  |  |

##### 8.3.x.3.2	Spatial anchor distance compute service subscription response

Table 8.3.x.3.2-1 describes information elements for the Spatial anchor distance compute service subscription response from the SAn server.

Table 8.3.x.3.2-1: Spatial anchor distance compute service subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor distance compute service subcription request was successful. |
| > Cause | O | The cause for the request failure. |

##### 8.3.x.3.3	Spatial anchor distance compute service notification

Table 8.3.x.3.3-1 describes information elements for the Spatial anchor distance compute service notification from the SAn server.

Table 8.3.x.3.3-1: Spatial anchor distance compute service notification

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor retrieve request was successful. |
| > Spatial anchor distance information | M | Indicates the calculated distance between source spatial anchor(s) and target spatial anchor(s) |
| >> Source spatial anchor | M | Indicates the source spatial anchor location or identifier. The source spatial anchor may represent UE location. |
| >> Target spatial anchor | M | Indicates the target spatial anchor location or identifier |
| >> Calculated distance | M | Indicates the distance between the source and target spatial anchor location. |
| > Augmented Spatial map grid | O | Indicates the spatial map containing the source and target spatial anchor with its calculated distance information |
| Failure response | O | Indicates that the spatial anchor retrieve request has failed. |
| > Cause | O | The cause for the request failure. |

* * * END of Changes * * * *