3GPP TSG-SA WG6 Meeting #66	S6-251490

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-251476)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0288 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Obtain service API authorization (without RNAA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.11.3 describes the Obtain service API procedure for an API invoker requesting authorization to access a service API when RNAA does not apply. In this procedure, the following aspects need to be corrected/clarified: - The obtain service API authorization request indicates that the API invoker may include, besides the API invoker identity, service information as application service provider and application identifier. Considering that the binding between the API invoker identity and the related application information is permanent and unique, it is not needed to include the application related information in every authorization request, but to keep it in the API invoker’s profile created during onboarding. - The obtain service API authorization request must include information identifying the service API, and this aspect is not clear in the current wording. Additionally, the request may include finer granularity information about the operation(s) and resource(s) for which authorization is requested, being this aspect also unclear. - The CAPIF core function, to return authorization information to the API invoker, checks the restrictions to access to the requested service API, which may include operation(s) and resource(s) restrictions. This aspect is not clear either. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 8.1.3 is updated to clarify that the API invoker’s profile can contain API invoker’s application information. - Clause 8.11.3 is updated to remove API invoker’s application information from the Obtain Service API authorization request. - Clause 8.11.3 is updated to clarify the possibility to include in the request and return in the reply finer granularity than service API, such as operation and resource. - Annex E is updated to include the CCF configuration information required to apply finer granularity in service API authorization information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Obtain service API authorization procedure is incorrect. Setting unnecessary requirements may create misunderstandings and misuse of the specified functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3, 8.11.3, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.1.3	Procedure

Figure 8.1.3-1 illustrates the procedure for onboarding the API invoker to the CAPIF. The security aspects of this procedure are specified in subclause 6.1 of 3GPP TS 33.122 [12].

Pre-conditions:

1.	The API invoker is not a recognized user of the CAPIF.

2.	The API invoker has visibility to APIs information (e.g., API catalogue or dashboard - central place for the API provider to manage which APIs are displayed, giving API invokers the ability to enroll for).

Figure 8.1.3-1: Procedure for onboarding the API invoker to the CAPIF

1.	For enrollment of the API invoker to be a recognized user of the CAPIF, the API invoker triggers onboard API invoker request towards the CAPIF core function, providing the information as required for the API management and criteria information as required to onboard the API invoker.

2.	The CAPIF core function begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker, and further initiates a grant process. If the onboard API invoker request message includes onboard criteria information, then the CCF verifies whether the CCF can support the onboard criteria information. The CCF further proceeds with API invoker onboarding procedure, only if the CCF supports the onboard criteria information from the API Invoker. Successful onboarding results in provisioning API invoker profile which includes identity for the API invoker. The authorization information and the list of APIs and the categories of APIs that the API invoker can access subsequent to successful onboarding may also be created. The CAPIF core function may create access control policy (see Table E-1) for the onboarded API invoker considering the network slice information.

NOTE 1:	Completion of onboarding process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

NOTE 2:	The API invoker profile consists of at least the identity information for the API invoker, information required for the authentication and authorization by the CAPIF (e.g., application service information such as application service provider and application identifier) and the CAPIF identity information.

3.	If the API invoker has triggered the onboard API invoker request and is granted permission, the onboard API invoker response provides success indication including information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

4.	As a result of successful onboarding process, the CAPIF core function is able to authenticate and authorize the API invoker.

* * * Next Change * * * *

### 8.11.3	Procedure

Figure 8.11.3-1 illustrates the procedure for obtaining authorization to access the service API.

Pre-condition:

1.	The API invoker is onboarded and has received an API invoker identity.

Figure 8.11.3-1: Procedure for the API invoker obtaining authorization for service API access

1.	The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API by including the API invoker information required for authorization, the information identifying the service API, and any information required for authentication of the API invoker. The request may include desired Network Slice Info of the service API. In addition to the information identifying the service API, the request may include finer granularity level for service API  authorization e.g., service API operation and  resource.

2.	The CAPIF core function validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API, considering also service API operation(s) and resource(s) restrictions locally configured, if available. The CAPIF core function may additionally verify the Network Slice Info, e.g., check that the desired Network Slice Info by the API invoker is included in the supported Network Slice Info for the indicated service API.

Editor's Note:		Authorization related to network slice are to be detailed by SA3.

NOTE 1:	The authorization of Network Slice Info is performed by the CAPIF core function within the PLMN trust domain.

NOTE 2:	The authentication process is specified in subclause 6.5.2 of 3GPP TS 33.122 [12].

3.	Based on the API invoker's subscription information available in the CCF, the authorization information to access the service APIs, which may include access authorization per service operation, resource, optionally including access authorization per service operation and, optionally, resource, is sent to the API invoker in the obtain service API authorization response.

NOTE 3:	How the finer service API authorization granularity is indicated in the request from API invoker and is returned to the API invoker in the CCF authorization reply is specified in 3GPP TS 33.122 [12].

NOTE 4:	The mechanism for distribution of the authorization information for the API invoker to the API exposing function over CAPIF-3 reference point is specified in subclause 6.5.2 of 3GPP TS 33.122 [12].

* * * Next Change * * * *

Annex E (normative):
Configuration data for CAPIF

The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator.

The configuration data for CAPIF is specified in table E-1.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE) |
| Subclause 4.10 | List of access control policy per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| NOTE: If no value is set for the duration, the duration is assumed to be unlimited. |  |

Editor's Note:		The per resource, operation authorization information to authorize the API invoker request is FFS and depends on the progress of SA3.

* * * END of Changes * * * *