3GPP TSG-SA WG6 Meeting #67	S6-252014

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-250xyz)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0037 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to SM data source events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The consumers like VAL server can discover SM data source UEs to build their Spatial maps using the procedure defined in clause 9.5.5. These SM data source UEs provide information like sensor data to the VAL server. Since SM data source play a very critical role for spatial maps, it is important for the VAL server to be aware of any changes in the SM data source. The proposed solution enables the VAL server to subscribe for the event changes in the SM data source. This helps the VAL server to receive notification for any change in the SM data source and accordingly VAL server application logic can handle the changes. For example, VAL server can discover new set of SM data source in case of non availability of SM data source event. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enabling VAL server to subscribe for changes of SM data source event and receive notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VAL server cannot receive notification regarding any change in SM data source and this has negative impact to the VAL server services or applications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.6.2.1, 9.3.6.2.2, 9.3.6.3.1, 9.3.6.3.3, 9.5.x, 9.5.x.2, 9.5.x.3, 9.5.x.3.1, 9.5.x.3.2, 9.5.x.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-250280 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 9.3.6.2.1	Subscribe spatial map event

Figure 9.3.6.2.1-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to subscribe the spatial map event. The service is provided by SEAL SM server. Spatial map consumer can get notification information when the the SM server detects subscribed event.

Figure 9.3.6.2.1-1: Subscribe spatial map event procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, a spatial map ID, event to subscribe which triggers the notification. The request message may also include a list of spatial map layers with its corresponding layer IDs to subscribe spatial map layering information related events. The requestor (VAL server or SEAL SM client) may subscribe to one or more of the following events:

a)	"changes of objects" events to indicate the SM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for "changes of objects" event may include added or removed objects, position or direction changes of the spatial map objects.

b)	"layers modification" events to indicate the SM server to notify the requestor when any layer in the spatial map is added or updated or removed.

c)	"spatial map ready" events to indicate the SM server to notify the requestor when the SEAL SM server has sufficient information to map all the objects related to area of interest the spatial map and the spatial map is ready to provide service (e.g. employed for localization). This event is used when the result IE in the Create spatial map response indicates "in-progress".

d)	"changes of SM data source" events to indicate the SM server to notify the requestor when any changes in SM data source elements described in Table 7.3.3.2-1 are detected.

Editor's Note: Objects are limited to UEs and objects other than UE is FFS.

2)	The SEAL SM server authorizes the VAL server (or SEAL SM client), and validates the request. Then server add the requestor into the subscriber list of the event. SM server starts monitoring the events. The SM server monitors the elements described in Table 7.3.3.2-1 for "changes of SM data source" events. The SM server also subscribes to the SEAL Location Management Server services in 3GPP TS 23.434 or NEF services specified in 3GPP TS 23.502 to monitor the location of the SM data source UE. The SM server also monitors any explicit update in the SM data source done via the procedure defined in clause 9.5.3 TS 23.437.

3)	The SEAL SM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.

##### 9.3.6.2.2	Notify spatial map event

Figure 9.3.6.2.2-1 depicts the notify spatial map event operation between the authorized spatial map consumer (VAL server or SEAL LM client) and SEAL SM server.

Figure 9.3.6.2.2-1: Notify spatial map event

1)	The SEAL SM server detects an event that triggers the notification to the subscribers for the corresponding event.

a)	If the subscribed event is "changes of objects", the SM server detects any changes of the spatial map objects.

b)	If the subscribed event is "Layers modification", the SM server detects any changes of the spatial map layers information.

c	If the subscribed event is "spatial map ready", the SM server detects when the SEAL SM server has sufficient information to map all the objects related to area of interest the spatial map and the spatial map is ready to provide service (e.g. employed for localization).

d)	If the subscribed event is "changes of SM data source", the SM server detects any changes of the SM data source UE like changes in SM data type, SM data format, SM data area, changes in location, availability state, data source update interval.

SM server determines whether the detected event matches triggering criteria and notifies subscribers accordingly.

2)	The SEAL SM server notifies the VAL servers (or SEAL SM clients) which subscribed the corresponding event. The notification includes a subscription ID and event-specific information. If the event is "changes of objects", spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map etc. If the event is "Layer modification", the notification message includes the list of all layers for the spatial map. If the event is "Changes of SM data source", the notification message includes the list of all SM data sources identifiers that are updated and the updated elements (e.g. changes in SM data type, SM data format, SM data area, location, availability state, data source update interval).

##### 9.3.6.3.1	Subscribe spatial map request

Table 9.3.6.3.1-1 shows the subscribe spatial map event request sent by VAL server (or SEAL SM client) to SEAL SM server to subscribe for spatial map event(s).

Table 9.3.6.3.1-1: Subscribe spatial map event request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL user) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Expiry time | O | Proposed subscription expiry time |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to subscribe |
| List of events | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | List of events to subscribed. Possible values are: - Change of objects - Layers modification - spatial map ready - Changes of SM data source |
| > specified objects | O | List of specified objects for "change of objects" event |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g. Spatial map is employed and ready for localization service. |
| >> Changes in objects | O (NOTE) | Triggers notification if any or specified (or specified type of) objects are added, removed, etc. |
| >> Layer IDs | O (NOTE) | List of layers. Triggers notification if any or specified spatial map layer change is detected |
| >> Changes in SM Data source | O (NOTE) | List of SM Data sources with IDs. Triggers notification if any or specified element described in Table 7.3.3.2-1 change is detected |
| > Notification intervals | O | Specifies when to notify the detected event, Possible values are: - Event based (e.g., promptly); Or - Periodically (with minimum time between consecutive notifications) - intervals (i.e. minimum time between consecutive notifications) |
| > interval time | O | Time between consecutive notifications (applicable only if notification intervals is set to intervals. |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

Editor's Note:	Details about "layers" event is FFS.

Editor's Note:	Additional events (e.g. change of access control rules) is FFS.

##### 9.3.6.3.3	Notify spatial map event

Table 9.3.6.3.3-1 shows the spatial map notification sent by SEAL SM server to VAL server (or SEAL SM client) to notify about spatial map event(s).

Table 9.3.6.3.3-1: Spatial map event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| > Spatial map ID | M | ID of the spatial map for which the event is detected |
| > Event type | M | Type of event detected. One of the event as specified in Table 9.3.6.3.1-1. |
| >> Changed object | O (NOTE 1) | Information on the object which triggered the notification |
| >>> Object ID | O | ID of the changed object |
| >>> Type of change | O | Specifies the detected type of change for the object (e.g., added, removed, or changed position or direction) |
| >>> Changed object information | O | A container to carry the changed object specific information |
| >> list of layers (NOTE 2) | O | Information about layers for the spatial map. |
| >> list of SM data source (NOTE 3) | O | SM data source ID and information about changes in the SM data source. |
| NOTE 1: This IE is applicable for "Change of objects" event type. NOTE 2: This IE is applicable for "Layer modification" event type. NOTE 3: This IE is applicable for "Changes of SM data source" event type. |  |  |

Editor's Note: Details on "Changed object information" is FFS.

* * * Next Change * * * *

### 9.5.x	SM data source event subscription

#### 9.5.x.1	General

The SM data source event subscription procedure enables consumers to subscribe to any changes in the SM data sources and receive notifications about it.

#### 9.5.x.2	Procedure

Figure 9.5.x.2-1 depicts the procedure for a consumer to subscribe to the SM data source events.

Pre-conditions:

- The SEAL SM client has already registered the SM data source with the SEAL SM server.

- The consumer has already discovered the SM data source.

Figure 9.5.x.2-1: SM data source subscribe event procedure

1.	The consumer(e.g.,VAL server) sends a SM data source event subscribe request to the SEAL SM server to subscribe for the changes of SM data source event. The request includes a requestor ID and a list of SM data source IDs.

2.	Upon receiving the request, the SEAL SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SEAL SM server sends the SM data source event subscription response to consumer. The SM server monitors the elements described in Table 7.3.3.2-1 for "changes of SM data source" events. The SM server also subscribes to the SEAL Location Management Server services in 3GPP TS 23.434 or NEF services specified in 3GPP TS 23.502 to monitor the location of the SM data source UE. The SM server also monitors any explicit update in the SM data source done via the procedure defined in clause 9.5.3 TS 23.437.

3.	The SM server detects any changes of the SM data source UE like changes in SM data type, SM data format, SM data area, changes in location, availability state, data source update interval. And sends the notification message includes the list of all SM data sources identifiers that are updated and the updated elements (e.g. changes in SM data type, SM data format, SM data area, location, availability state, data source update interval).

#### 9.5.x.3	Information flows

##### 9.5.x.3.1	SM data source event subscribe request

Table 9.5.x.3.1-1 describes information elements in the SM data source event subscribe request from the SEAL SM client to the SEAL SM server.

Table 9.5.x.3.1-1: SM data source event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identity of the requestor (e.g., VAL server) |
| List of SM Data source IDs | M | Identifiers of the list of SM data sources requested to subscribe the changes in SM Data source |
| >SM Data source ID | M | Identity of the SM data source |

##### 9.5.x.3.2	SM data source event subscribe response

Table 9.5.x.3.2-1 describes information elements in the SM data source event subscribe response from the SEAL SM server to the consumer.

Table 9.5.x.3.2-1: SM data source event subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.5.x.3.3	SM data source event notification

Table 9.5.x.3.3-1 describes information elements in the SM data source event notification from the SEAL SM server to the consumer

Table 9.5.x.3.3-1: SM data source event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| List of SM Data source IDs | M | List of SM data sources |
| Changes in SM Data source | M | Indicates list of SM data source ID and information about changes in the SM data source. |
| > SM data source ID | M | Identity of the SM data source |
| > Updated elements | M | The updated elements include changes in the elements mentioned in Table 7.3.3.2-1. e.g., changes in SM data type, SM data format, SM data area, location, availability state, data source update interval |
| >> Changes in SM data type | O (NOTE) | Indicates the Changes in SM data type |
| >> Changes in SM data format | O (NOTE) | Indicates the Changes in SM data format |
| >> Changes in SM data area location | O (NOTE) | Indicates the Changes in SM data area location |
| >> Changes in availabaility state | O (NOTE) | Indicates the Changes in availabaility state |
| >> Changes in data source update interval | O (NOTE) | Indicates the Changes in data source update interval |
| NOTE: Atleast one of the information elements shall be present |  |  |

* * * END of Change * * * *