3GPP TSG-SA6 Meeting #67	S6-252015

# Fukuoka, Japan, 19th May 2025 - 23th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0242 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A.3 MCVideo user profile configuration data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wrong reference point mentioned in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change REC-5 to REC-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.3	MCVideo user profile configuration data

The general aspects of MC service user profile configuration data are specified in 3GPP TS 23.280 [6]. The MCVideo user profile configuration data is stored in the MCVideo user database. The MCVideo server obtains the MCVideo user profile configuration data from the MCVideo user database (MCVideo-2). The recording server obtains the MCVideo user profile configuration data from the configuration management server (REC-3 in 3GPP TS 23.280 [16]).

Tables A.3-1 and A.3-2 contain the MCVideo user profile configuration required to support the use of on-network MCVideo service. Tables A.3-1 and A.3-3 contain the MCVideo user profile configuration required to support the use of off-network MCVideo service. Data in table A.3-1 and table A.3-3 can be configured offline using the CSC-11 reference point.

Table A.3-1: MCVideo user profile configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | Rec. admin UE & Replay UE (NOTE 8) | MCVideo Server | Configuration management server | Rec. server | MCVideo user database |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo identity (MCVideo ID) | Y | Y | Y | Y | Y | Y |
| 3GPP TS 33.180 [14] | KMSUri for security domain of MCVideo ID (see NOTE 1) | Y |  | Y | Y |  | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | Pre-selected MCVideo user profile indication (see NOTE 2) | Y |  | Y | Y |  | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo user profile index | Y |  | Y | Y |  | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo user profile name | Y |  | Y | Y |  | Y |
| [R-5.17-007], [R-6.13.4-002] of 3GPP TS 22.280 [2] | User profile status (enabled/disabled) |  | Y | Y | Y | Y | Y |
| [R-5.7-001] [R-6.9-003] of 3GPP TS 22.280 [2] | Authorised to create and delete aliases of an MCVideo user and its associated user profiles. |  |  |  | Y |  | Y |
| [R-5.7-002], [R-6.9-003] of 3GPP TS 22.280 [2] | Alphanumeric aliases of user | Y |  | Y | Y |  | Y |
| [R-5.1.1-005], [R-5.9-001] of 3GPP TS 22.280 [2] | Participant type of the user | Y |  | Y | Y |  | Y |
| [R-5.1.8-006], [R-5.3-002], [R-5.9-001], [R-5.16.2-001], [R-5.16.2-002] of 3GPP TS 22.280 [2] | User's Mission Critical Organization (i.e. which organization a user belongs to) | Y | Y | Y | Y | Y | Y |
| [R-5.2.2-003] and [R-6.6.3-002] of 3GPP TS 22.280 [2] | Authorisation to create a group-broadcast group (see NOTE 7) |  |  |  | Y |  | Y |
| [R-5.2.2-003] and [R-6.6.3-002 of 3GPP TS 22.280 [2] | Authorisation to create a user-broadcast group (see NOTE 7) |  |  |  | Y |  | Y |
| [R-5.6.2.4.1-002] of 3GPP TS 22.280 [2] | Authorised to activate MCVideo emergency alert | Y |  | Y | Y |  | Y |
| [R-5.6.2.4.1-013] of 3GPP TS 22.280 [17] | Automatically trigger a MCVideo emergency communication after initiating the MCVideo emergency alert | Y |  | Y | Y |  | Y |
| [R-5.6.2.4.1-004] [R-5.6.2.4.1-008] [R-5.6.2.4.1-012] of 3GPP TS 22.280 [2] | Group used on initiation of an MCVideo emergency group call (see NOTE 5) | Y |  | Y | Y |  | Y |
| [R-5.6.2.4.1-004], [R-5.6.2.4.1-008], [R-5.6.2.4.1-012] of 3GPP TS 22.280 [17] | Recipient for an emergency private MCVideo call (see NOTE 5) |  |  |  |  |  |  |
|  | > MCVideo ID | Y |  | Y | Y |  | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCVideo ID (see NOTE 1) | Y |  | Y | Y |  | Y |
| [R-5.6.2.4.2-002] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCVideo emergency alert | Y |  | Y | Y |  | Y |
| [R-6.15.6.2-002] of 3GPP TS 22.280 [2] | Authorised to activate an MCVideo ad hoc group emergency alert | Y |  | Y | Y |  | Y |
| [R-6.15.6.2-006] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCVideo ad hoc group emergency alert | Y |  | Y | Y |  | Y |
| [R-6.15.6.2-007] of 3GPP TS 22.280 [2] | Authorised to set up an MCVideo group call using the ad hoc group used for the alert | Y |  | Y | Y |  | Y |
|  | Authorised to receive the participants information of an MCVideo ad hoc group emergency alert | N |  | Y | Y |  | Y |
|  | Authorised to modify the list of participants and criteria for an MCVideo ad hoc group emergency alert | Y |  | Y | Y |  | Y |
| [R-5.1.2.1.2-004] of 3GPP TS 22.281 [3] | Authorisation to modify the video settings of the transmitted video stream of another MCVideo User | Y |  | Y | Y |  | Y |
| [R-5.1.2.1.2-006] of 3GPP TS 22.281 [3] | Authorisation to renegotiate a codec during a video transmission. | Y |  | Y | Y |  | Y |
| [R-5.1.2.1.2-007] of 3GPP TS 22.281 [3] | Authorisation to remotely control the video capabilities or parameters for a camera on an MCVideo UE | Y |  | Y | Y |  | Y |
| [R-5.1.2.2.2-001] of 3GPP TS 22.281 [3] | Authorisation to remotely control the video capabilities or parameters of a remote MCVideo UE | Y |  | Y | Y |  | Y |
| [R-5.1.2.2.2-004] of 3GPP TS 22.281 [3] | Authorisation to receive and display the capabilities of a remote MCVideo UE | Y |  | Y | Y |  | Y |
| [R-5.1.3.1.2-004] of 3GPP TS 22.281 [3] | Authorisation to remotely activate another MCVideo User's camera | Y |  | Y | Y |  | Y |
| [R-5.1.9.2.2-002] of 3GPP TS 22.281 [3] | Authorisation to push a video to another MCVideo user . | Y |  | Y | Y |  | Y |
| [R-5.1.9.2.2-003] of 3GPP TS 22.281 [3] | Authorisation to enable and to disable the automatic sending of notification to a second MCVideo User that a video is being pushed to a third MCVideo User | Y |  | Y | Y |  | Y |
| [R-5.1.9.2.2-004] of 3GPP TS 22.281 [3] | List of MCVideo users for whom to receive notifications about video being pushed to them | Y |  | Y | Y |  | Y |
|  | > MCVideo IDs |  |  |  |  |  |  |
| [R-5.1.9.2.2-005] of 3GPP TS 22.281 [3] | List of specific video categories to receive (see NOTE 3) | Y |  | Y | Y |  | Y |
|  | > Video categories |  |  |  |  |  |  |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | List of user(s) who can be called in MCVideo private call |  |  |  |  |  |  |
|  | > MCVideo ID | Y |  | Y | Y |  | Y |
|  | > User info ID | Y |  | N | Y |  | Y |
|  | > ProSe discovery group ID | Y |  | N | Y |  | Y |
|  | > Presentation priority relative to other users and groups (see NOTE 4) | Y |  | Y | Y |  | Y |
| 3GPP TS 33.180 | > KMSUri for security domain of MCVideo ID (see NOTE 1) | Y |  | Y | Y |  | Y |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | Authorised to make a private video call towards users not included in "list of user(s) who can be called in MCVideo private call" | Y |  | Y | Y |  | Y |
| [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of category tags |  |  |  |  |  |  |
|  | > Video category tag | Y |  | Y | Y |  | Y |
| [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | Authorization to query MCVideo client | Y |  | Y | Y |  | Y |
| [R-5.1.3.2.2-004] [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of category tags that authorized to query MCVideo client |  |  |  |  |  |  |
|  | > Video category tag for query | Y |  | Y | Y |  | Y |
| [R-5.1.3.2.2-004] [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of geography areas that authorized to query MCVideo client |  |  |  |  |  |  |
|  | > Geography area to query | Y |  | Y | Y |  | Y |
| [R-5.1.1.1-015] of 3GPP TS 22.281 [3] | Authorization to perform video adaptation | Y |  | Y | Y |  | Y |
| [R-5.1.7-002] and [R-6.8.7.2-007] and [R-6.8.7.2-008] of 3GPP TS 22.280 [2] | Priority of the user (see NOTE 6) |  |  | Y | Y |  | Y |
| [R-6.15.4-001…010] and [R-5.1.10.2-008] of 3GPP TS 22.280 [17] | User is a target for recording | Y (NOTE 10) | Y | Y | Y | Y | Y |
| [R-6.15.4-001…010] and [R-5.1.10.2-008] of 3GPP TS 22.280 [17] | Recording server address (URI) (NOTE 9) | Y (NOTE 10) | Y | Y | Y |  | Y |
| NOTE 1: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: As specified in 3GPP TS 23.280 [6], for each MCVideo user's set of MCVideo user profiles, only one MCVideo user profile shall be indicated as being the pre-selected MCVideo user profile. NOTE 3: If this list is blank then this implies that all video categories are acceptable for the MCVideo user. NOTE 4: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 5: This parameter is used for the emergency communication and also used as a target of the emergency alert request. At most one of them is configured; i.e. emergency communication will go to either a group or a user. If both are not configured the MCVideo user's currently selected group will be used. NOTE 6: The use of the parameter is left to implementation. NOTE 7: This parameter applies to temporary broadcast groups built from regrouping mechanism. This authorisation automatically sets the originator of the temporary group as the only transmitting party. NOTE 8: This is the recording admin UE and/or replay UE of a user who is authorized to set this MCVideo user as target for recording and/or authorized to replay this MCVideo user’s recordings. NOTE 9: If this parameter is absent, the recording server URI shall be that identified in the initial MC service UE configuration data (on-network, 3GPP TS 23.280 table A.6-1) or in the initial recording and replay UE configuration data (on-network, 3GPP TS 23.280 table A.10-1) NOTE 10: These parameters, when provided to an MCVideo UE, are relevant only for recording administrators and replay users. |  |  |  |  |  |  |  |

Table A.3-2: MCVideo user profile configuration data (on network)

| Reference | Parameter description | MCVideo UE | Rec admin UE & Replay UE (NOTE 9) | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- | --- |
| [R-5.1.5-001], [R-5.1.5-002], [R-5.10-001], [R-6.4.7-002], [R-6.8.1-008], [R-6.7.4-002] of 3GPP TS 22.280 [2] | List of on-network MCVideo groups for use by an MCVideo user |  |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |  |
|  | >> Server URI | Y | Y | Y | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |  |
|  | >> Server URI | Y | Y | Y | Y | Y |
| 3GPP TS 33.180 [14] | > KMSUri for security domain of group (see NOTE 4) | Y |  | Y | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y |  | Y | Y | Y |
| Subclause 5.2.5 of 3GPP TS 23.280 [6] | List of groups user implicitly affiliates to after MCVideo service authorization for the user |  |  |  |  |  |
|  | > MCVideo Group ID | Y |  | Y | Y | Y |
| [R-6.4.2-006] of 3GPP TS 22.280 [2] | Authorisation of an MCVideo user to request a list of which MCVideo groups a user has affiliated to |  |  | Y | Y | Y |
| [R-6.4.6.1-002], [R-6.4.6.1-003] of 3GPP TS 22.280 [2] | Authorisation to change affiliated groups of other specified user(s) |  |  | Y | Y | Y |
| [R-6.4.6.2-001], [R-6.4.6.2-002] of 3GPP TS 22.280 [2] | Authorisation to recommend to specified user(s) to affiliate to specific group(s) |  |  | Y | Y | Y |
| [R-6.6.1-004] of 3GPP TS 22.280 [2] | Authorisation to perform regrouping | Y |  | Y | Y | Y |
| [R-6.7.2-001] of 3GPP TS 22.280 [2] | Presence status is available/not available to other users | Y |  | Y | Y | Y |
| [R-6.7.1-002], [R-6.7.2-002] of 3GPP TS 22.280 [2] | List of MCVideo users that MCVideo user is authorised to obtain presence of |  |  |  |  |  |
|  | > MCVideo IDs | Y |  | Y | Y | Y |
| [R-6.8.7.4.2-001], [R-6.8.7.4.2-002] of 3GPP TS 22.280 [2] | Authorisation of a user to cancel an emergency alert on any MCVideo UE of any user |  |  | Y | Y | Y |
| [R-6.13.4-001] of 3GPP TS 22.280 [2] | Authorisation for an MCVideo user to enable/disable an MCVideo user |  |  | Y | Y | Y |
| [R-6.13.4-003], [R-6.13.4-005], [R-6.13.4-006], [R-6.13.4-007] of 3GPP TS 22.280 [2] | Authorisation for an MCVideo user to (permanently /temporarily) enable/disable a UE |  |  | Y | Y | Y |
| [R-7.14-002], [R-7.14-003] of 3GPP TS 22.280 [2] | Authorization for manual switch to off-network while in on-network | Y |  | Y | Y | Y |
| [R-5.1.5-004] of 3GPP TS 22.280 [2] | Limitation of number of affiliations per user (Nc2) | N |  | Y | Y | Y |
| [R-6.4.6.1-001], [R-6.4.6.1-004] of 3GPP TS 22.280 [2] | List of MCVideo users whose selected groups are authorized to be remotely changed |  |  |  |  |  |
|  | > MCVideo ID | Y |  | Y | Y | Y |
| [R-5.2.3.2-002], [R-5.2.3.2-003] of 3GPP TS 22.281 [3] | Period after which MCVideo data on a MCVideo UE is to be deleted if no action is taken by an authorized MCVideo user | Y |  | Y | Y | Y |
| [R-5.2.6.2.2-004] of 3GPP TS 22.281 [3] | Maximum number of simultaneous video streams that can be received (see NOTE 3) | Y |  | Y | Y | Y |
| [R-5.2.6.2.2-005] of 3GPP TS 22.281 [3] | Authorisation to automatically receive video communications | Y |  | Y | Y | Y |
| [R-5.2.6.2.2-006] of 3GPP TS 22.281 [3] | Authorisation to automatically receive emergency video streams | Y |  | Y | Y | Y |
| [R-5.2.6.2.2-007] of 3GPP TS 22.281 [3] | Authorisation to automatically receive imminent peril video streams | Y |  | Y | Y | Y |
| [R-5.2.6.2.2-008] of 3GPP TS 22.281 [3] | List of MCVideo groups for which video can be automatically/mandatorily received |  |  |  |  |  |
|  | > MCVideo group IDs | Y |  | Y | Y | Y |
| [R-5.2.7.2-001] of 3GPP TS 22.281 [3] | Authorisation to request to override an active MCVideo transmission | Y |  | Y | Y | Y |
| [R-5.2.7.2-002] of 3GPP TS 22.281 [3] | Authorisation to select MCVideo transmissions that can be overridden | Y |  | Y | Y | Y |
| [R-5.2.7.2-004] of 3GPP TS 22.281 [3] | Authorisation to allow MCVideo private communications to override or not to override active MCVideo group communications | Y |  | Y | Y | Y |
| [R-5.2.8-005], [R-5.2.8-006] of 3GPP TS 22.281 [3] | Maximum length of time of a single video transmission | Y |  | Y | Y | Y |
| [R-6.7.3-007a] of 3GPP TS 22.280 [2] | List of user(s) from which MCVideo private calls can be received |  |  |  |  |  |
|  | > MCVideo ID | Y |  | Y | Y | Y |
|  | > Presentation priority relative to other users and groups | Y |  | Y | Y | Y |
| 3GPP TS 33.180 [18] | > KMSUri for security domain of MCVideo ID | Y |  | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [2] [R-5.9a-013] of 3GPP TS 22.280 [2] | Authorised to request association between active functional alias(es) and MCVideo ID(s) |  |  | Y | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | List of partner MCVideo systems in which this profile is valid for use during migration |  |  |  |  |  |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCVideo system | Y |  | N | Y | Y |
| Subclause 10.1.1 of 3GPP TS 23.280 [16] | > Access information for partner MCVideo system (see NOTE 5) | Y |  | N | Y | Y |
| [R-6.6.4.2-002a] and [R-6.6.4.2-002b] of 3GPP TS 22.280 [2] | List of groups the client affiliates/de-affiliates when criteria is met |  |  |  |  |  |
|  | > MCVideo Group ID | Y |  | Y | Y | Y |
|  | >> Criteria for affiliation (see NOTE 7) | Y |  | Y | Y | Y |
|  | >> Criteria for de-affiliation (see NOTE 7) | Y |  | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y |  | Y | Y | Y |
| [R-6.6.4.2-002] of 3GPP TS 22.280 [2] | List of groups the client affiliates after receiving an emergency alert |  |  |  |  |  |
|  | > MCVideo Group ID | Y |  | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y |  | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [2] | Authorised to take over a functional alias from another MCVideo user | N |  | Y | Y | Y |
|  | List of functional alias(es) of the MCVideo user |  |  |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | > Functional alias | Y |  | Y | Y | Y |
| [R-5.9a-018] of 3GPP TS 22.280 [2] | >> MCVideo server trigger criteria for activation (see NOTE 6) | N |  | Y | Y | Y |
| [R-5.9a-017], [R-5.9a-018] of 3GPP TS 22.280 [2] | >> MCVideo server trigger criteria for de-activation (see NOTE 6) | N |  | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [16] | >> MCVideo client trigger criteria for activation (see NOTE 6) | Y |  | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [16] | >> MCVideo client trigger criteria for de-activation (see NOTE 6) | Y |  | Y | Y | Y |
|  | >> Manual de-activation is not allowed if the criteria are met (see NOTE 6) | Y |  | Y | Y | Y |
| [R-5.10-001b] of 3GPP TS 22.280 [16] | Maximum number of successful simultaneous MCVideo service authorizations for this user (see NOTE 8) | N |  | Y | Y | Y |
| [R-5.9a-020] of 3GPP TS 22.280 [17] | List of functional aliases to which private calls are allowed when using a certain functional alias |  |  |  |  |  |
|  | > Used functional alias | Y |  | Y | Y | Y |
|  | >> List of functional aliases which can be called |  |  |  |  |  |
|  | >>> Functional alias | Y |  | Y | Y | Y |
| [R-5.9a-021] of 3GPP TS 22.280 [17] | List of functional aliases from which private calls can be received when using a certain functional alias |  |  |  |  |  |
|  | > Used functional alias | N |  | Y | Y | Y |
|  | >> List of functional aliases from which calls can be received |  |  |  |  |  |
|  | >>> Functional alias | N |  | Y | Y | Y |
|  | Ad hoc group call authorizations |  |  |  |  |  |
| [R-6.15.5.3-001] of 3GPP TS 22.280 [17] | > Authorised to initiate ad hoc group call | Y |  | Y | Y | Y |
| R-6.15.5.3-003] of 3GPP TS 22.280 [17] | > Authorised to participate in ad hoc group call | Y |  | Y | Y | Y |
|  | > Authorised to initiate emergency ad hoc group call | Y |  | Y | Y | Y |
|  | > Authorised to initiate imminent peril ad hoc group call | Y |  | Y | Y | Y |
|  | > Authorised to receive the participants information of ad hoc group call | N |  | Y | Y | Y |
|  | > Authorised to modify the list of participants and criteria for an ad hoc group call | Y |  | Y | Y | Y |
|  | > Authorised to release ongoing ad hoc group calls | Y |  | Y | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 3: The parameter can be set to an unlimited number of simultaneous streams received that can be received. NOTE 4: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 5: Access information for each partner MCVideo system comprises the list of information required for initial UE configuration to access an MCVideo system, as defined in table A.6-1 of 3GPP TS 23.280 [16] NOTE 6: The criteria may consist of conditions like the location of the MCVideo user, local time etc. NOTE 7: The criteria may consist of conditions such as the location of the MCVideo user or the active functional alias of the MCVideo user. NOTE 8: If configured, this value has precedence over the system level parameter "maximum number of successful simultaneous service authorisations" in table A.5-2. If not configured, the corresponding parameter from table A.5-2 shall be used. NOTE 9: This is the recording admin UE and/or replay UE of a user who is authorized to set this MCVideo user as target for recording and/or authorized to replay this MCVideo user’s recordings. |  |  |  |  |  |  |

Table A.3-3: MCVideo user profile configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| [R-7.2-003], [R-7.6-004] of 3GPP TS 22.280 [2] | List of off-network MCVideo groups for use by an MCVideo user |  |  |  |  |
|  | > MCVideo Group IDs | Y | N | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
| 3GPP TS 33.180 [14] | > KMSUri for security domain of group (see NOTE 3) | Y | N | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | N | Y | Y |
| [R-7.12-002], [R-7.12-003] of 3GPP TS 22.280 [2] | Authorization for off-network services | Y | N | Y | Y |
| Subclause 7.18.1 | User info id (as specified in 3GPP TS 23.303 [7]) | Y | N | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. |  |  |  |  |  |

# * * * * End of changes * * * *