3GPP TSG-SA6 Meeting #67	S6-252022

# Fukuoka, Japan, 19th May 2025 - 23th May 2025 	                   (revision of S6-251029)

Source:	Airbus

Title:	Analysis of TR 23.784 for FS_MCLOG_Ph2

Spec:	3GPP TR 23.700-39

Agenda item:	9.2

Document for:	Discussion

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

Abstract:

This discussion paper provides an analysis of what can be utilized from TR 23.784 (Rel-16) in the Rel-20 study item FS_MCLOG_Ph2 (TR 23.700-39).

This DP can be used as one source when creating pCRs - scenarios, KIs, solutions - for the Rel-20 study. However, please note that this DP represents the opinions of the author at the time of presenting this in SA6.

This DP may be updated during the study phase (by adding color coding as shown below) to help estimating the progress of the study item. However, no further analyses of the TR 23.784 should be done in this DP – all further development/analysis etc shall be added to the new Rel-20 TR 23.700-39.

Note also that TR 23.784 (or this discussion paper) does not cover all scenarios/KIs/solutions that are required for the Rel-20 FS_MCLOG_Ph2 study.

Green highlight = pCR(s) approved for at least one KI and/or solution

Yellow highlight = pCR(s) available, but not yet approved, or a topic that requires discussion in SA6

# General observations of TR 23.784 and proposals for the Rel-20 study work

General:

- The assumption when starting this Rel-16 study obviously was that the two features covered in this study – “Discreet listening” and “Logging” - share the same architecture, similar procedures and configurations.

  - Now in Rel-20, when we have separate study items for these two features, we should not make this assumption at the start. It is naturally ok if that is the end result but such assumptions should not limit the content or set any biases on these studies.

Scenarios:

- TR 23.784 does not contain recording (or logging) specific scenarios. However, all call related discreet listening scenarios in the TR are relevant also for recording as written in clause 4.8:

  - Clause 4.8 / Scenario 7: Logging scenarios – where it says: “The call related scenarios can be considered to be the same as those described in scenarios 1 to 6 related to discreet listening, except that the recipient of the information is a logging client or interface instead of the authorized user”.

- All the “scenarios 1 to 6” are still valid but before importing them into the new Rel-20 TR, some regrouping and changes are proposed:

  - Scenarios that are already supported in the Rel-19 TSs can be ignored.

  - All indications of solutions shall be removed. E.g. clauses like “…the request for discreet listening (or recording) is sent by MC service client A to MCData server A…”.

  - All figures and text shall be aligned with the (recording) functional architecture in Rel-19 TSs.

  - A lot of text in the NOTEs belong to KIs and - for clarity - shall be removed from the scenarios.

  - S6-251028 proposes a structure for the new Rel-20 TR 23.700-39. In this discussion paper the scenarios from TR 23.784 are analyzed one by one and mapped to the proposed structure.

    - NOTE 1: TR 23.784 does not contain any scenarios for “non-communication sessions related user activities” (location signaling, emergency alert, failed authorization attempts etc) or “off-network recording”.

    - NOTE 2: In this discussion paper, only the scenarios are categorized according to S6-251028. KIs and solutions are listed in the order they were found from TR 23.784.

- Mapping between scenarios and SA1 user requirements is missing from TR 23.784. This should be added to the Rel-20 TR. Note that this mapping can be many-to-many.

Key Issues:

- TR 23.784 has a mixture (in no order) of KIs for recording and discreet monitoring. Some of the KIs are only for recording, some for discreet monitoring and some are common for both. They can be used as one source for KIs to the Rel-20 TR but probably very little can be copied without a deep review and revisioning.

Solutions:

- Solutions may contain ‘extra’ text (explanations, different options, …) in the first phase of the study, but before TR approval, there should not be any overlapping solutions or ‘options’ inside the preferred solution.

- To speed up the normative phase, the proposal is to go one step ‘deeper’ (from TR 23.784) in the Rel-20 TR. At least each preferred solution should clearly indicate which TSs and clauses etc will be impacted and how. The final version of this TR should provide clear (unambiguous) starting point for normative CRs, not just long verbose explanations how the solution ‘could’ generally look like in configurations and procedures etc…

# Scenarios (Clause 4 of TR 23.784)

## Recording of SIP session-based MC services

### MCPTT and MCVideo private calls and group calls

- TR23.784:

  - Clause 4.2.1 / Scenario 1.1: Discreet listening of MCPTT and MCVideo private calls in the same MC service server

  - Clause 4.3 / Scenario 2: Discreet listening of MCPTT and MCVideo group calls

- Analysis:

  - Recording solutions for these scenarios already exist in Rel-19 SA6 TSs i.e. there is no need to add them into TR 23.700-39.

- Action:

  - No action needed

### One to one MCVideo pull and push

- TR23.784:

  - Clause 4.4.1 / Scenario 3.1 Discreet listening of one to one MCVideo pull

  - Clause 4.5.1 / Scenario 4.1 Discreet listening of one to one MCVideo push

- Analysis:

  - As described in these clauses (in TR 23.784), one to one Video pull/push is a MCVideo private call with specific parameter values.

  - A key issue related to this scenario for Rel-20 is to study if the Rel-19 solution for MCX server – Recording server interface (REC-4 reference point / SIPREC) can be used also for MCVideo one to one pull/push. The solution is most probably that nothing needs to be added, except some text to the TS(s) saying that recording of MCVideo one to one push/pull is supported. See e.g. the NOTE in TS23280/7.5.2.34.

- Action:

  - Update the scenarios (see clause 1 of this DP) and add to TR 23.700-39.

### Remotely initiated MCVideo push

- TR23.784:

  - Clause 4.5.3 / Scenario 4.3: Discreet listening of remotely initiated MCVideo push

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

- Action:

  - Update the scenario (see clause 1 of this DP) and add to TR 23.700-39.

### MCData SDS (using MSRP/SIP session)

- TR23.784:

  - Clause 4.6 / Scenario 5: Discreet listening of MCData SDS

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

- Action:

  - Update the scenario (see clause 1 of this DP) and add to TR 23.700-39.

### MCData one to one file distribution (using MSRP/SIP session)

- TR23.784:

  - Clause 4.7.2 / Scenario 6.2: Discreet listening of one to one file distribution

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

- Action:

  - Revise the scenario (see clause 1 of this DP) and add to TR 23.700-39.

## Recording of non-SIP-session-based MC services

### One from server MCVideo pull and one to server MCVideo push

- TR23.784:

  - Clause 4.4.2 / Scenario 3.2 Discreet listening of one from server MCVideo pull

  - Clause 4.5.2 / Scenario 4.2 Discreet listening of one to server MCVideo push

- Analysis:

  - These are valid scenarios for FS_MCLOG_Ph2.

- Action:

  - Revise the scenarios (see clause 1 of this DP) and add to TR 23.700-39.

### MCData SDS (using SIP MESSAGE)

- TR23.784:

  - Clause 4.6 / Scenario 5: Discreet listening of MCData SDS

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

  - Note that this is the same scenario as 2.1.4 with different transport mechanism.

- Action:

  - Revise the scenario (see clause 1 of this DP) and add to TR 23.700-39.

### MCData file upload and file download

- TR23.784:

  - Clause 4.7.1 / Scenario 6.1: Discreet listening of file upload and file download

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

- Action:

  - Revise the scenario (see clause 1 of this DP) and add to TR 23.700-39.

### MCData one to one file distribution (using SIP MESSAGE)

- TR23.784:

  - Clause 4.7.2 / Scenario 6.2: Discreet listening of one to one file distribution

- Analysis:

  - This is a valid scenario for FS_MCLOG_Ph2.

  - Note that this is the same scenario as 2.1.5 with different transport mechanism.

- Action:

  - Revise the scenario (see clause 1 of this DP) and add to TR 23.700-39.

## Recording of migrated users / interworking scenarios

### MCPTT and MCVideo private calls and group calls with target receiving service from different MC service server

- TR23.784:

  - Clause 4.2.2 / Scenario 1.2: Discreet listening of MCPTT and MCVideo private calls with target receiving service from different MC service server

  - Clause 4.3 / Scenario 2: Discreet listening of MCPTT and MCVideo group calls / NOTE 3

- Analysis:

  - These are valid scenarios. A better title could be e.g. “recording of migrated users”.

- Action:

  - Revise the scenarios (see clause 1 of this DP) and add to TR 23.700-39.

# Key Issues (Clause 5 of TR 23.784)

## Selection of MC service

- TR23.784:

  - Clause 5.1 / Key Issue 1: Selection of MC service for discreet listening

- Analysis:

  - In TR 23.784 this KI is for discreet listening only, but it is a valid question also for recording

  - Granularity with the Rel-19 solution is already at a service level i.e. MCPTT / MCData / MCVideo recording can be set independently for each user.

  - If granularity within each MC service (PTT/Video/Data) is needed, that should be a requirement from SA1.

- Action:

  - To be decided: either no action or LS to SA1?

## End to end encrypted calls

- TR23.784:

  - Clause 5.5 / Key Issue 5: Logging and replay of end to end encrypted calls

- Analysis:

  - During Rel-19 work for recording, this topic was discussed with SA3 delegates.

  - In informal discussions it was clarified that:

    - The e2e key (PCK) for private calls, created in the call initiating MCX UE, can be fetched and decrypted (by MCX server or Recording server) with key material available from KMS. This assumption is valid at least when both MCX UEs (clients) use the same KMS.

    - For group calls, GMK is sent from GMS to the MC UEs in the group. The same GMK could be provided also to MCX server or Recording server. However, the functional architecture should not restrict on how GMKs are delivered between servers within one MC system.

- Action:

  - Re-check the Rel-19 TS(s).

  - If corrections are needed -> CR to Rel-19.

  - If the issue is not a FASMO -> new KI+solution to Rel-20 TR

## Migrated users

- TR23.784:

  - Clause 5.7 / Key Issue 7: Discreet listening and logging for interconnected MC system

  - Clause 5.8 / Key Issue 8: Discreet listening and logging for migrated MC service user

- Analysis:

  - The target for recording is always a user (not MC system) i.e. the KI should be about migrated users (KI8), which includes all the variations described under 5.7 and 5.8 of TR 23.784.

  - The text in 5.7 and 5.8 can be partially utilized, together with the Rel-19 recording architecture to rewrite these KIs.

  - One KI may be enough, but the issues can also be split into multiple KIs, this is TBD.

- Action:

  - New KIs needed, after the relevant scenario(s) is(are) agreed.

## Recording of a user receiving MC service(s) on multiple devices

- TR23.784:

  - Clause 5.9 / Key Issue 9: Discreet listening and logging for user receiving MC service on multiple devices

- Analysis:

  - TR 23.784 has no solution for this KI (even if in the overall evaluation this KI is mapped to several solutions…digging a bit deeper, those solutions do not say anything about this KI).

- Action:

  - This is a valid scenario and KI. Whether any new configurations or procedures are needed is not clear, but a study item is the correct place to study this and document the conclusion/solution.

## Recording of MCData content server and message store services

- TR23.784:

  - Clause 5.12 / Key Issue 12: Discreet listening of MCData content server and message store services

- Analysis:

  - This is a relevant KI.

- Action:

  - Review/revise if needed and add to the Rel-20 TR.

## Recording of an MC service group

- TR23.784:

  - Clause 5.13 / Key Issue 13: Discreet listening towards an MC service group

- Analysis:

  - This is a relevant KI, but already solved for MCPTT and MCVideo in Rel-19.

- Action:

  - If needed, can be added for MCData.

# Solutions (Clause 6 of TR 23.784)

## Functional model, configuration and procedures

- TR23.784:

  - Clause 6.3 / Solution 3: Functionality for on-network logging and replay

  - Clause 6.10 / Solution 10: Functionality for on-network logging and replay

- Analysis:

  - Application plane functional models:

    - The system level on-network func.model TS 23.280: solved in R19 for recording SIP -session based services. For services not using SIP sessions, a study is required to see if something needs to be added.

    - The system level off-network func.model TS 23.280: update needed for recording.

    - The service level on-network func.models (TS 23.379/281/282): update needed

    - The service level off-network func.models (TS 23.379/281/282): update needed

  - Configurations:

    - Solved in R19 for on-network recording

      - NOTE: As a reminder, removal of MCData configurations from Rel-19 (S6-250334 and S6-250335) must be reverted after a Rel-20 WID is approved.

    - Configurations related to off-network recording as well as for interconnection and migration are missing

    - Depending on the decision on 3.1 (see above), additional configuration parameters may be needed.

  - Procedures:

    - on-network recording - done in R19 for recording SIP-session based services. For services not using SIP sessions, a study is required to see if something needs to be added

    - off-network recording – missing

  - The existing text in Solution 3 and Solution 10 is not usable as it is based on ‘wrong’ recording architecture.

- Action:

  - Several actions needed (see above and clause 1 of this DP).

  - Related KI (or KIs) are needed before solutions.

## Interconnection and migration

- TR23.784:

  - Clause 6.4 / Solution 3: On-network logging for interconnection and migration

- Analysis:

  - This is a relevant topic for FS_MCLOG_Ph2.

  - This solution can be used to get ideas what is needed in a solution description, but it cannot be copied to the new TR since the recording architecture is different.

- Action:

  - Related KIs are needed first. Solution(s) for “migrated users” and/or “interconnection and migration” must be added after that.

## MCData message store and MCData content server

- TR23.784:

  - Clause 6.7 / Solution 7: Logging of MCData message store and MCData content server

- Analysis:

  - This is a relevant topic for FS_MCLOG_Ph2.

  - KI / KIs are needed first.

  - This solution can be used as a starting point but it must be adapted to the R19 recording architecture/configurations/procedures. Some new/enhancements may be needed between MCData server and Recording server.

- Action:

  - Solution(s) needed, but KI/KIs must be done first.