3GPP TSG-SA WG6 Meeting#67	S6-252038

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252xxx )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0292 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Onboarding procedure clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | API invoker identity is expected to be known by both API invoker and CAPIF after a successful onboarding. Clause 6.3.2 indicates the API invoker responsibility to provide the API invoker Identity for authentication purposes, but as part of the onboarding procedure there is no API invoker identity exchanged between CAPIF Core Function and API invoker. The API invoker identity is essential in many of the API invoker information flows (e.g. Service API Discovery, API invoker authentication, API invoker authorization, Service Api invocation, …). The API invoker profile built by CCF is expected to contain additional info about the API invoker, but some essential information for this purpose (e.g., API invoker domain name) is not mentioned in the onboarding flow. While the uniqueness of the API invoker identity is implied and appears to be used as an assumption in the information flows involving the API invoker, the uniqueness of the API invoker identity is not mandated either. Lack of uniqueness of the API invoker id can create problems within one CAPIF domain, as well as across interconnected CAPIF domains, if different API invokers could get assigned the same API invoker id. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed solution updates the onboarding information flows to clarify the API invoker identity being provided in the response, as well as its uniqueness. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information flows involving the API invoker do not work if it is not clear how the API invoker ID is created/obtained/exchnaged between the API invoker and CCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1, 8.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.1.2	Information flows

#### 8.1.2.1	Onboard API invoker request

Table 8.1.2.1-1 describes the information flow onboard API invoker request from the API invoker to the CAPIF core function.

Table 8.1.2.1-1: Onboard API invoker request

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding information | M | The information of the API invoker including enrolment details, required for onboarding. This information about the API invoker is used by CCF to uniquely identify the API invoker (e.g., API invoker domain name) and is needed to build the API invoker profile in CCF. |
| > Onboarding criteria | O | Indicates that the API Invoker wishes to on board itself to the CCF only if the CCF supports the features mentioned in criteria information. |
| >> Criteria information | M | This information includes at least one of the criteria listed (supported security methods, supported security methods for given AEFs / AEF types/ service APIs/ Service API categories) for the CCF to onboard the API invoker. |
| APIs for enrollment | O | List of APIs being enrolled for. |
| Proposed expiration time | O | Proposed expiration time for the onboarding. |

#### 8.1.2.2	Onboard API invoker response

Table 8.1.2.2-1 describes the information flow onboard API invoker response from the CAPIF core function to the API invoker.

Table 8.1.2.2-1: Onboard API invoker response

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding status | M | The result of onboarding request i.e., success indication is included if the API invoker is granted permission otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned API invoker profile which includes information to allow the API invoker to be authenticated and to obtain authorization for service APIs, containing the unique API invoker identity associated to the onboarded API invoker profile (e.g., including the OAuth 2.0 client ID provided by the Authorization Function and the CAPIF provider domain). |
| Service API information | O (see NOTE 2) | The service API information as specified in Table 8.7.2.2-1. |
| Reason | O (see NOTE 3) | This element indicates the reason when onboarding status is failure. |
| Expiration time | O | Indicates the expiration time of the onboarding. At expiration, CCF cancels the enrollment of the API invoker from CAPIF. If omitted, it indicates the onboarding does not expire. |
| NOTE 1: Information element shall be present when onboarding status is successful. NOTE 2: Information element may be present when onboarding status is successful. NOTE 3: Information element shall be present when onboarding status is failure. |  |  |

### 8.1.3	Procedure

Figure 8.1.3-1 illustrates the procedure for onboarding the API invoker to the CAPIF. The security aspects of this procedure are specified in subclause 6.1 of 3GPP TS 33.122 [12].

Pre-conditions:

1.	The API invoker is not a recognized user of the CAPIF.

2.	The API invoker has visibility to APIs information (e.g., API catalogue or dashboard - central place for the API provider to manage which APIs are displayed, giving API invokers the ability to enroll for).

Figure 8.1.3-1: Procedure for onboarding the API invoker to the CAPIF

1.	For enrollment of the API invoker to be a recognized user of the CAPIF, the API invoker triggers onboard API invoker request towards the CAPIF core function, providing the information as required for the API management and criteria information as required to onboard the API invoker.

2.	The CAPIF core function begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker, and further initiates a grant process. If the onboard API invoker request message includes onboard criteria information, then the CCF verifies whether the CCF can support the onboard criteria information. The CCF further proceeds with API invoker onboarding procedure, only if the CCF supports the onboard criteria information from the API Invoker. Successful onboarding results in provisioning API invoker profile which includes identity for the API invoker. The authorization information and the list of APIs and the categories of APIs that the API invoker can access subsequent to successful onboarding may also be created. The CAPIF core function may create access control policy (see Table E-1) for the onboarded API invoker considering the network slice information.

NOTE 1:	Completion of onboarding process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

NOTE 2:	The API invoker profile consists of at least the identity information for the API invoker, information required for the authentication and authorization by the CAPIF and the CAPIF identity information.

3.	If the API invoker has triggered the onboard API invoker request and is granted permission, the onboard API invoker response provides success indication including information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

4.	As a result of successful onboarding process, the CAPIF core function is able to authenticate and authorize the API invoker.

* * * END of Change * * * *