3GPP TSG-SA WG6 Meeting #67	S6-252050

Fukuoka, Japan 19th – 23rd May 2025	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0670 | rev |  | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 10.17.4.1.2.1 on using FA as a recipient |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedure on 10.17.4.1.2.1 is not correct when the recipient is a functional alias (FA) in the partner MC system. If a FA, in the partner MC system, is used as the recipient of ACM request, it needs to get resolved by the primary MC system ACMS (with helps from the MC service FA controlling server in the partner MC system) what is the MC service ID of the recipient first and then uses that MC service ID to resend the ACM request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We have agreed to a mechanism for a primary MC system to resolve a functional alias in the partner MC system in clause 10.16.3.3 and is used here in 10.17.4.1.2.1 also. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong guidance for stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.17.4.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

###### 10.17.4.1.2.1	Group membership update by authorized user from a partner MC system

The procedure for an authorized MC service user in a primary MC system to manage group membership in an interconnection group that is defined in a partner MC system is shown in figure 10.17.4.1.2.1-1.

Pre-conditions

-	Primary MC system and partner MC system are interconnected

-	Primary MC system and partner MC system have implemented an ACMS

-	Partner MC system is the MC service group home system of an interconnection group

-	The authorized MC service user of the primary MC system wants to modify the membership of MC users of the primary system in the interconnection group for which the partner MC system is the MC service group home system.

-	When a functional alias is used as the target of the request by an ACM client in the primary MC system, it is required that the ACM server of the primary MC system has subscribed to the MC service functional alias controlling server within the primary MC system.

-	A secure connection has been established between the MC service functional alias controlling servers between the primary and partner MC systems

Figure 10.17.4.1.2.1-1: Group membership update by authorized user from a primary MC system

1.	The ACMC in primary MC system sends a group membership update request to the ACMS in the primary MC system, requesting to modify the membership of users from primary MC system in an interconnection group.

2.	The ACMS of the primary MC system checks whether the MC service user at the ACMC is authorized for the request. If the request targets a functional alias the ACMS of the primary MC system resolves the functional alias to the corresponding MC service ID for which the functional alias is active at the partner MC system by using subsequent steps 3-8; otherwise continue with step 9.

3.	The ACMS sends an MC service functional alias resolution request message to the MC service functional alias controlling server 1 to resolve the functional alias of the ACM request.

4.	The MC service functional alias controlling server 1 determines that the functional alias belongs to the partner MC system and forwards the MC service functional alias resolution request message to MC service functional alias controlling server 2.

5.	The MC service functional alias controlling server 2 resolves the functional alias and determines the corresponding MC service ID to terminate the ACM request and returns it to the MC service functional alias controlling server 1 in the MC service functional alias resolution response message.

NOTE:	Depending on implementation the ACMS can apply additional local policy and decide whether the ACM request is allowed to proceed with the resolved MC service ID(s) (e.g., whether the MC service ID is within the allowed area of the functional alias). If the ACMS of the partner MC system detects that the functional alias used as the target of the ACM request is simultaneously active for multiple MC service users, then the ACMS can proceed by selecting an appropriate MC service ID based on some selection criteria. The selection of an appropriate MC service ID is left to implementation. This selection criteria can include rejection of the ACM request, if no suitable MC service ID is selected.

6.	The MC service functional alias controlling server 1 returns the corresponding MC service ID to ACMS of the primary MC system in the MC service functional alias resolution response message. The ACMS of the primary MC system checks if the MC service user at ACMC of the primary MC system is authorized to initiate the ACM request to the MC service user at the ACMC of partner MC system. If not authorized stop the procedure, otherwise continue with step 7.

7.	The ACMS responds with an MC service functional alias resolution response message that contains the resolved MC service ID back to ACMC of the primary MC system.

8.	The ACMC of the primary MC system sends a new group membership update request towards the resolved MC service ID.

9.	The ACMS of the primary MC system sends the group membership update request to the ACMS of the partner MC system.

10. 	The ACMS of the partner MC system sends a process indication to the primary MC system, and stores, verifies and assesses the incoming request.

Based on local policy,

-	the ACMS of the partner MC system may automatically handle the request as described in clause 10.17.3.2 and continue with step 14,

or

-	the ACMS of the partner MC system requests verification by an authorized ACM user: If the authorized user in the partner MC system is not logged on, the pending request procedure as described in clause 10.17.3.1 is followed.

11.	The ACMS of the partner MC system sends the stored group membership update request to the ACMC of partner MC system.

12.	The authorized MC service user of the partner MC system (manually) screens the content of the group membership update request and decides whether to approve it or not. The authorized user in the partner MC system takes actions needed to apply the group membership update. The actions within the partner MC system to apply these configurations are outside the scope of the 3GPP specifications.

13.	The ACMC of the partner MC system sends a group membership update response to the ACMS of the partner MC system.

14.	The ACMS of the partner MC system sends the group membership update response to the ACMS of the primary MC system.

15.	The ACMS of primary MC system stores the group membership update response. If the ACMC of primary MC system is not logged on to the ACMS of primary MC system it will follow the pending request procedure as described in clause 10.17.3.1.

16.	The ACMS of primary MC system sends the group membership update response to the ACMC of the primary MC system.