3GPP TSG-SA WG6 Meeting #67	S6-252062

Fukuoku, Japan, 17th – 21st May 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	Enhance AIMLE Client Registration and Discovery to Support Diverse AIMLE Deployments

Spec:	3GPP TR 23.700-83 v0.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue#6, the study aspects on support enhancements related to diverse AIMLE deployments scenarios include:

- Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses the information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When the AIMLE server/client are deployed hierarchically, enhancements to the AIMLE client registration and discovery services are needed to support AIMLE service. To support diverse AIMLE deployments scenarios, this pCR proposes to enhance the AIMLE client registration and discovery.

2. Reason for Change

To support diverse AIMLE deployments scenarios, this pCR proposes to enhance the AIMLE client registration and discovery.

3. Conclusions

This paper proposes solution on enhance AIMLE client registration and discovery for diverse AIMLE deployments scenarios.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.1.0.

* * * First Change * * * *

## 7.XX	Solution#XX: Enhance AIMLE Client Registration and Discovery

### 7.XX.1	Solution Description

As introduced in 3GPP TS 23.482, the AIMLE AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses the information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When the AIMLE server/client are deployed hierarchically, enhancements to the AIMLE client registration and discovery services are needed to support AIMLE service provisioning.

To support diverse AIMLE deployments scenarios, this pCR proposes to enhance the AIMLE client registration and discovery services.

The following clauses specify the impact to existing information flows for the enhancements of AIMLE client registration and discovery services.

### 7.XX.1.1	Impact to existing AIMLE client registration

The AIMLE information flows in 3GPP TS 23.482 for AIMLE client registration can be enhanced (highlighted in bold italics) as follows.

#### 8.7.3.2 AIMLE client registration request Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request. Table 8.7.3.2-1: AIMLE client registration request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. List of supported profiles; M; Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided.. > AIMLE client profile; M; Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2.. > List of supported services; M; List of VAL service IDs with corresponding permissions.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > DNN and DNAI(s); O; Identify which EDN or DN the AIMLE client is located.. Table 8.7.3.2-2: AIMLE client profile Information element; Status; Description. Supported AI/ML model types; O; AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks).. Supported AI/ML operations; M; AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split).. AIMLE client time schedule configurations; O; Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations in the given time slot(s) and/or day(s) of the week.. AIMLE client location configurations; O; Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID.. AIMLE client capabilities; M; AIMLE client capability information (e.g. ML application type, allowed resource usage level).. Dataset availability; O; Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers.. Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions.. AIMLE client task capability; O See NOTE 1; Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs). EAS ID; O See NOTE 2; The identifier of the EAS (with AIMLE server deployed which serving the AIMLE client).. AIMLE server ID; O See NOTE 2; The identifier of the AIMLE server which serving the AIMLE client.. Support role in AIML task topology; O; Indicates the supported role by the AIMLE client in AIML task topology (e.g., allow to be connected in a hierarchical or distributed architecture, or not allowed).. NOTE 1: The Green and Energy-efficient task performance may not be applicable to a UE. NOTE 2: At least one of the IEs shall be provided when the AIMLE client is served by Edge-deployed AIMLE server.. #### 8.7.3.3 AIMLE client registration response Table 8.7.3.3-1 shows the AIMLE client registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.3-1: AIMLE client registration response Information element; Status; Description. Successful response; O; Indicates that the registration was successful.. > Registration ID; M; The identifier of the registration.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. ML repository ID; O See NOTE; Identifier of the ML repository where the AIMLE client is registered to.. Failure response; O; Indicates that the registration request failed.. > Cause; O; Indicates the cause of registration request failure. NOTE: This IE shall be provided when ML repository(ies) are deployed hierarchically or distributed..

### 7.XX.1.2	Impact to existing AIMLE client discovery

The AIMLE information flows in 3GPP TS 23.482 for AIMLE client discovery can be enhanced (highlighted in bold italics) as follows.

### 8.8.3 Information flows #### 8.8.3.1 AIMLE client discovery request Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure. Table 8.8.3.1-1: AIMLE client discovery request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. AIMLE client discovery criteria; M; Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. Number of required AIMLE clients; O; Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria.. Table 8.8.3.1-2: AIMLE client discovery criteria Information element; Status; Description. Service requirement; M; Information about the required service. > VAL Service ID; M; VAL Service ID that the client is required to support. This identifies the service associated with the requester.. > Service permission level; O; Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage).. Requested ML model types; O; Requested ML model types (decision trees, linear regression, neural networks).. Requested AIML operations; M; Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split.. Application layer AIMLE client capabilities; M; Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate).. Dataset requirements; O; Information about dataset.. > Dataset availability; O; Dataset availability, including dataset identifiers, dataset size, age, list of dataset features.. > Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions.. AIML client task capability requirements; O; Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs). AIMLE client velocity; O; Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.. Location information; O; Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID, DNN and DNAI(s)) to discover the AIMLE clients.. AIMLE client QoS requirements; O; Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients.. EAS ID; O See NOTE; The identifier of the EAS (with AIMLE server deployed).. AIMLE server ID; O See NOTE; The identifier of the AIMLE server.. Request role in AIML task topology; O; Requested role to the AIMLE client in AIML task topology (e.g., to be connected in a hierarchical or distributed architecture).. NOTE: At least one of the IEs shall be provided when Edge-deployed AIMLE server.. #### 8.8.3.2 AIMLE client discovery response Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client discovery procedure. Table 8.8.3.2-1: AIMLE client discovery response Information element; Status; Description. Status; M; The status for the request: success or fail.. List of AIMLE client IDs; M; A list of AIMLE client IDs that matches the AIML discovery criteria.. > AIML supported tasks; O; Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks.. > EAS ID; O See NOTE; The identifier of the EAS (with AIMLE server deployed which serving the AIMLE client).. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server which serving the AIMLE client.. > Support role in AIML task topology; O; Indicates the supported role by the AIMLE client in AIML task topology (e.g., allow to be connected in a hierarchical or distributed architecture, or not allowed).. NOTE: At least one of the IEs shall be provided when the AIMLE client is served by Edge-deployed AIMLE server..

### 7.XX.2	Architecture Impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 7.XX.3	Corresponding APIs

Editor’s Note:	The corresponding APIs for supporting the solution is FFS.

### 7.XX.4	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***