3GPP TSG-SA6 Meeting #67	S6-252071

Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.392 | CR | 0008 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to authentication aspect indicated in various procedures preconditions. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2025-5-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the DC application and profile configuration procedures, its indicated that parameters required for the authentication is configured in the pre-condition. But in the procedure, there is no indication on how the mutual authentication is done. Also, its indicated that Requester identity is used for authentication but more appropriately this could be used for authorization of the request in alignment with other procedures. Hence, to align the procedures better, its indicated that mutual authentication is successful as precondition and Requester Identity is used for authorization purpose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedures defined in the clauses 8.2.2.2, 8.2.3.2, 8.2.4.2, 8.5.2 are updated to indicate that authentication is completed successfully as part of pre-condition and Requester identity is used for authorization purpose and corresponding update is done. Additional editorial corrections are done in 8.5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This may lead to inconsistent stage-2 requirements leading to gap in stage-3 implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2, 8.2.3.2, 8.2.4.2, 8.2.5.2, 8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.2.2	Procedure

Figure 8.2.2.2-1 illustrates the DC application and profile configuration procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and mutual authentication is successful.

3.	Both DC application and profile have been uploaded to Controlling Application Server.

Figure 8.2.2.2-1: DC application and profile configuration

1.	The Controlling Application Server sends a DC application and profile configuration request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request based on the Requested identity. If the authorization is successful, the MMTel Enabler Server allocates an APPID and creates the DC application profile for each DC Application included in the DC application and profile configuration request, and maintain the mapping of APPID and DC Application Profile.

3.	The MMTel Enabler Server configures the DC application and profile on DCAR.

4.	The MMTel Enabler Server sends a DC application and profile configuration response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.2.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

* * * Next Change * * * *

#### 8.2.3.2	Procedure

Figure 8.2.3.2-1 illustrates the DC application and profile update procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and mutual authentication is successful.

Figure 8.2.3.2-1: DC application and profile update

1.	The Controlling Application Server sends a DC application and profile update request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request based on the Requester Identity. If the authorization is successful, the MMTel Enabler Server updates the DC Application or DC application profile for each DC Application based on information provided in the request.

3.	The MMTel Enabler Server updates the DC application and profile to DCAR.

4.	The MMTel Enabler Server sends a DC application and profile update response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.3.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

* * * Next Change * * * *

#### 8.2.4.2	Procedure

Figure 8.2.4.2-1 illustrates the DC application deletion procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and mutual authentication is successful.

Figure 8.2.4.2-1: DC application deletion

1.	The Controlling Application Server sends a DC Application delete request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.4.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request based on the Requester Identity. If the authorization is successful, the MMTel Enabler Server requests the DCAR to delete the DC application and profile (DC applications and profile need to be deleted at the same time) and delete the mapping of APPID and DC application profile.

3.	The MMTel Enabler Server deletes the DC application and profile on DCAR.

4.	The MMTel Enabler Server sends a DC Application delete response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.4.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

* * * Next Change * * * *

#### 8.2.5.2	Procedure

Figure 8.2.5.2-1 illustrates the procedure of getting DC application profile information.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and mutual authentication is successful.

Figure 8.2.5.2-1: Getting DC application profile information

1.	The Controlling Application Server sends a DC application and profile information retrieval request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.5.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request based on the Requester Identity. If the authorization is successful, the MMTel Enabler Server gets the DC application Profile information based on the APPID included in the Getting DC application profile information request.

3.	The MMTel Enabler Server sends a Getting DC application profile information response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.5.3.2.

* * * Next Change * * * *

### 8.5.2	Procedure

Figure 8.5.2-1 illustrates the procedure of multiple call control handling coordination among different Application Servers.

Pre-conditions:

1.	The Application Servers and MMTel Enabler Server have mutually authenticated each other.

2.	The call control handling policy (e.g. the priority of each Application Server) is pre-configured in MMTel Enabler Server.

3.	Application Servers have completed call event subscription to MMTel Enabler Server and the MMTel Enabler Server subscribes the call event from DCSF/IMS core.

4.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

Figure 8.5.2-1: Multiple call control handling coordination among different Application Servers

1.	DCSF/IMS Core node sends call event notification, i.e, Nimsas_SessionEventControl_Notify as specifed in 3GPP TS 23.228 [3], to MMTel Enabler Server (e.g. SessionEstablishmentRequestEvent, MediaChangeRequestEvent, etc.).

2.	The MMTel Enabler Server checks which Application Server has subscribed this call event notification.

3.	The MMTel Enabler Server sends Call event notification to Application Server(s) which has subscribed for this call event notification. The Call event notification request includes information elements as specified in Annex AA.2.4.2 of 3GPP TS 23.228 [3].

4-5.	Application Servers receive call event notification and sends Call control request (e.g. create new call session, terminate call session or call control, etc.) based on its own service logic. The Call control request includes information elements as specified in Annex AA.2.4.3 or Annex AA.2.4.4 of 3GPP TS 23.228 [3].

6.	The MMTel Enabler Server receives call control requests from different Application Servers.It checks whether these call control requests are contradictory or not and makes call control handling decision based on service policy and service priority (e.g. if no conflict happens, to decide call control request execution sequence; if conflict happens, to deny call control request from certain AS with lower priority, etc.).

7-8.	The MMTel Enabler Server sends call control responses to each Application Server based on the call control handling decision including, whether the call control request is accepted or the failure reason if the call control request is not accepted. The call control response includes information elements as specified in Annex AA.2.4.3 or Annex AA.2.4.4 of 3GPP TS 23.228 [3].

9.	The MMTel Enabler Server sends call control requests decided in step 6 from Application Servers (or permitted Application Servers in conflict scenario) to DCSF/IMS core network by using Nimsas_MediaControl Service or Nimsas_ImsSessionManagement Service as specifed in 3GPP TS 23.228 [3].The DCSF/IMS core network sends a corresponding response (e.g, whether the call is established successfully or not) as specifed in 3GPP TS 23.228 [3] to the MMTel Enabler Server.

10. 	The MMTel Enabler Server sends the call control result notification request to the subscribed Application Servers in conflict scenario decided in step 6. The call control result notification includes information elements as specified in clause 8.5.3.1. The Application Server sends the corresponding control result notification response to the MMTel Enabler Server.

* * * End of Changes * * * *