3GPP TSG-SA6 Meeting #67	S6-252079

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0390 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Essential corrections to various eLSAPP procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-5-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Various essential and editorial corrections are done for the eLSAPP procedures to ensure requirements are not mis-interpreted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All the clauses indicated are updated with corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 requirements may be mis-interpreted and also impacts the quality of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.54, 9.3.2.65, 9.3.2.66, 9.3.23, 9.3.23.1, 9.3.23.2, 9.3.23.3, 9.3.24.1, 9.3.26.2, 9.3.26.3, 9.4.10.2, 9.4.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.54	Location tracing configuration cancel response

Table 9.3.2.54-1 describes the information flow from the location management server to the VAL server for the location tracing configuration cancel response.

Table 9.3.2.54-1: Location tracing configuration cancel response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | It indicates the location tracing configuration cancel request result. |

* * * Next Change * * * *

#### 9.3.2.65	Short-Range based positioning information request

Table 9.3.2.65-1 describes the information flow from the VAL server to the SEAL LMS for the Short-Range based positioning information request.

Table 9.3.2.65-1: Short-Range based positioning information request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | Identity of the requestor (e.g. VAL server or SEAL LMS) |
| VAL service ID | O | Identity of the VAL service (ranging/SL positioning service) for which the ranging/SL positioning information is requested. |
| Requested Short-Range based positioning information | M | Indicates the requested Short-Range based positioning information (e.g. range, direction, relative position, relative velocity, etc.). |
| UE list | M | A list of reference UE identities that have been configured for ranging/SL positioning operation, including client, target, and reference UEs. |
| QoS requirement | O | The QoS requirement for the ranging/SL positioning operation. |
| Reporting events | O | Reporting event triggers as specified in table 9.3.2.4-1. |
| Expiry Time | O | The expiry time for the Short-Range based positioning information request. |

* * * Next Change * * * *

#### 9.3.2.66	Short-Range based positioning information response

Table 9.3.2.66-1 describes the information flow from the SEAL LMS to the VAL server for the Short-Range based positioning information response.

Table 9.3.2.66-1: Short-Range based positioning information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | A status for the request: success or failure. If the status is failure, a failure code may be provided. |
| Cause | O | The cause of the failure. |
| Short-Range based positioning information | O (NOTE) | The Short-Range based positioning information associated with locations of UEs within the field of view of the target UE. |
| NOTE: This information element is present when the status for the request is success. |  |  |

* * * Next Change * * * *

### 9.3.23	Optimization of location service for multiple UEs sharing the same location

#### 9.3.23.1	General

If multiple VAL UEs share the same location area with each other, and when the VAL server requests the location information for each UE among them, the location management server will select one or several of them to obtain the location data instead of triggering all of UE's positioning procedure, which could reduce the signaling messages, save energy and power consumption, etc.

The following are the procedures to describe the related functions for such scenario.

* * * Next Change * * * *

#### 9.3.23.2	Procedure of LM Server identifying the UEs sharing the same location

Figure 9.3.23.2-1 describes the procedure on how the location management server identifies the UEs that are sharing the same location.

Pre-condition:

-	The VAL UE1 and VAL UE2 are associated and may belong to the same user or different users.

-	The location management client 1(in VAL UE1) and location management client 2(in VAL UE2) have reported the location capabilities (e.g. associated ID) for VAL UE1 and VAL UE2 to location management server respectively as defined in clause 9.3.15.

NOTE:	The associated ID for VAL UE1 and VAL UE2 is same.

Figure 9.3.23.2-1: Procedure of LM Server identifying the UEs that sharing the same location

1.	The location management server identifies the registered location management client1 and location management server client 2 are associated and may share the same location with each other based on the received location service registration information (e.g. same associated ID as defined in clause 9.3.15). Then it further verifies if they are sharing the same location at present.

2.	The location management server initiates the verify location sharing request to the location management server client 1 with the user ID of location management client 2 to verify if they are sharing the same location.

NOTE:	The step 2 may occur after the location management server receives the location request to the location management client 2 from the VAL server.

3.	The location management client 1 checks if the location management client 2 is close enough and shares the same location via the following ways.

-	The location management client 1 may trigger the location reporting to the location management client 2 via some positioning capabilities (e.g., ProSe) and receive off-network location report from the location management client 2 and determines that the VAL UE2 is within allowed proximity range of the VAL UE1 as specified in step 1 of clause 9.3.23.3.

-	The location management client 1 may ask the VAL user to check if VAL UE1 and VAL UE2 are in the same location. If the answer is yes, the VAL user may inform the location management client 1 that the VAL UE1 and VAL UE2 are in the same location area.

4.	The location management server client 1 sends the verify location sharing response to the location management server with the confirmation of sharing same location with location management client 2, and the validity timer is also included to indicate the valid duration for the confirmation.

Before the validity timer expired, if the location management client 1 discovers the location management client 2 is far away from itself and they are not able to share the same location, the location management client 1 will inform the location management server they no longer sharing the same location as specified in step 2(disable) of clause 9.3.23.3.

If the response is negative (e.g. the location management client 1 confirms the location management client 2 is not close with location management client 1 or the location management client 1 can't confirm whether both of UEs are closing enough), the location management server will not reuse the stored UE1's location for UE2.

* * * Next Change * * * *

#### 9.3.23.3	Location reuse request

Figure 9.3.23.3-1 illustrates the procedure of client-triggered enable location reuse request.

Pre-condition:

1.	All UEs in the association are registered to share the location.

2.	UE has configured other UEs in the association to receive offline location reports as specified in clause 9.5.3.

3.	The VAL UE1 and VAL UE2 are associated and may belong to the same user or different users.

4.	The LM Client 1(in VAL UE1) and LM Client 2(in VAL UE2) have reported the location capabilities (e.g. associated ID) for VAL UE1 and VAL UE2 to LM server respectively as defined in clause 9.3.15.

Figure 9.3.23.3-1: Location reuse procedure

1.	The SEAL LMC-1 (in UE-1) receives off-network location report from the SEAL LMC-2 (in UE-2). Based on location report, if UE-2 is within certain range of UE-1, the SEAL LMC-1 determines that the UE-2 is within allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are close enough) and so UE-1's location can be used by SEAL LMS instead of UE-2's location.

Based on location report, if UE-2 is out of certain range of UE-1, the SEAL LMC-1 determines that the UE-2 is outside allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are not close enough) and so UE-1's location can not be used by SEAL LMS instead of UE-2's location.

2.	The SEAL LMC-1 (in UE-1) sends location reuse request message to the SEAL LMS to enable reuse of location of UE-1 for UE-2. If both UE1 and UE2 are close enough (i.e. within allowed proximity range), the request includes indication to enable reuse of location of UE-1 for UE-2. If both UE1 and UE2 are not close enough (i.e. outside allowed proximity range), the request includes indication to disable reuse of location of UE-1 for UE-2. The request message also includes VAL user’s identity, identity of VAL UE-1, identity of UE-2 and latest location report of UE-1.

3.	The SEAL LMS authorizes the user. If authorized, the SEAL LMS identifies that if SEAL LMC-1 and SEAL LMC-2 are associated (e.g. same associated ID) or not based on the received registration information of SEAL LMC-1 and SEAL LMC-2, and if reusing of location is allowed of not.

4.	If both UE-1 and UE-2 are assocaited and reusing of location is allowed (as determined in step-3), then the SEAL LMS stores the location of UE-1. If the request includes indication to enable reuse of location of UE-1, the SEAL LMS enables reuse of location of UE-1 for UE-2. If the request includes indication to disable reuse of location of UE-1, the SEAL LMS disables reuse of location of UE-1 for UE-2. The SEAL LMS sends response back to the SEAL LMC-1.

When location reuse is enabled, if location request for UE-2 is received from VAL server, the SEAL LMS reuses and provides the location of UE-1 (instead of determining the location of UE-2) to the VAL server. When location reuse is disable, if location request for UE-2 is received from VAL server, the SEAL LMS provides the location of UE-2 only (by determining the location of UE-2) to the VAL server.

* * * Next Change * * * *

#### 9.3.24.1	Procedure of SL positioning /ranging management service

The procedure provides a mechanism for configuring and provisioning information on the entities to be used and their responsibilities for a SL positioning / ranging service. In this procedure, an SL positioning enabler capability at LMS is authorized / delegated by SL positioning server to support the provisioning/delivery of ranging services in an area.

Figure 9.3.24.1-1 shows a high-level procedure with the details.

Figure 9.3.24.1-1: Support for SL positioning procedure

1.	The VAL server (SL Positioning / Ranging Server) sends an SL positioning management subscription request to SEAL LMS to provision and manage SL positioning / ranging operation and configurations.

2.	The SEAL LMS authorizes the VAL subscription request.

3.	The SEAL LMS sends an SL positioning management subscription response to the requestor VAL Server.

4.	The SEAL LMS discovers all the SEAL LMCs in the target service area, and categorizes them by their capabilities as target UE, Reference UEs, or even Located UEs (if Reference UE location is known or for out of coverage scenarios). The identification of UEs in a target area happens via existing SEAL LMS procedures (as in clauses 9.3.10 and 9.3.12).

5.	The SEAL LMS after getting all UE information at the target area, calculates the relative distances among them, and may request/receive HDMaps (e.g., from other VAL servers) for the target area.

Then, the SEAL LMS evaluates the need of reference UE (based on the information received in the step 4), and if reference UE is needed (e.g. in NLoS situation), the SEAL LMS selects the appropriate VAL UE to serve as reference and configures the VAL UEs (target, reference, client) with configuration information (thresholds, reporting configurations).

The LMS may also leverage analytics services (e.g. NWDAF) to obtain VAL UE location related predictions (e.g. UE location prediction, relative distance prediction) to discover reference UEs.

NOTE:	The selection of UE as reference UEs can be based on their capabilities and the selection criteria specified by the VAL server.

6.	The SEAL LMS sends a SL positioning management notification message to the VAL server to indicate the roles of the VAL UEs and acknowledging that the SL positioning provisioning is performed.

7.	The SEAL LMS configures the SEAL LMCs with the configuration information applicable to the corresponding VAL UEs based on step 5, and in particular the expected role (acting as reference UE, target UE) in relation ranging/SL positioning service. Based on this configuration, the LM clients interacts each other using off-network location management procedures as described in clause 9.5.

* * * Next Change * * * *

#### 9.3.26.2	Confirm location service subscription

The application is running on a VAL server. The application would like to use confirm location service and subscribes to LMS for it using the following procedure:

Figure 9.3.26.2-1: Procedure of confirm location service subscription

1.	The VAL server subscribes to LMS by sending Confirm location service subscription request to receive notification every time UE using specific application is requesting confirmation of location.

2.	The LMS shall check if the VAL server is authorized to subscribe for the confirm location service notifications for a specific application identifier.

3.	If the VAL server and specific application are authorized to subscribe for confirm location service, the LMS creates subscription based on provided parameters in the request.

4.	The LMS sends to VAL server confirm location service subscription response.

* * * Next Change * * * *

#### 9.3.26.3	Confirm location verification

The location management client (LMC)/application in the UE gets from non-3GPP positioning (e.g. GNSS) in the UE its location. The application in the UE wants to check if this is really the correct location.

Pre-conditions:

1.	The LMC is authorized to discover and to use Confirm Location API provided by the LMS;

2.	LMS is authorized to use Nnef Event Exposure API, based on SLA with NEF/5GS of MNO;

3.	It is assumed there is user consent given in the UE for exposing location information from UE to LMS;

2.	VAL server has subscribed to LMS for notification, whenever LMC requests for confirm location report from the LMS.

Figure 9.3.26.3-1: Procedure of confirm location verification

1.	The UE sends Confirm Location request from the LMC towards LMS. It contains the location measured from the UE (non-3GPP location information, e.g. GNSS). It contains latitude and longitude, accuracy, timestamp, carrier name, cell id (known to the UE from the network signalling), application identifier.

2.	The LMS shall check if there is subscription (based on VAL subscription in clause 6.12.2.1) for confirm location service with the specific application identifier received in step1. If subscription is present, step 3 is performed. If subscription is not present the LMS replies back to LMC with confirm location status unknown (step 4).

3.	LMS requests UE location from 5GC/LMF via NEF (LMS send determine Location operation via NEF (N33 interface) and using Nlmf_Location API defined in 3GPP TS 29.572 [8] – parameters available from the location reporting are described in the TS). It also requests accuracy to be a parameter present in the response. LMS can also perform a location information request to GMLC directly or via NEF (as defined in 6.1. of 3GPP TS 23.273 [5]), acting as AF. LMS can also perform a location information request to 3rd party location servers via LM-3P interface. LMS can also use the T8 interface to obtain UE location. Or the LMS may already have the UE location based on the methods described in 3GPP TS 23.434 [9]. The received response contains the UE location from the network. LMS compares the location provided from the network with the location provided from the LMC. LMS can use event exposure using NEF according to table 4.15.3.1-1 of TS 23.502 to obtain user location information from AMF.

4.	LMS sends confirm location report to LMC.

5.	The LMS sends Confirm location service usage notification towards the VAL server with the UE identifier and application using the service.

* * * Next Change * * * *

#### 9.4.10.2	Subscribe_ SLPositioningManagement

API operation name: Subscribe_ SL_Positioning_Management

Description: Subscription to SL positioning management.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.61

Outputs: See subclause 9.3.2.62.

See subclause 9.3.24.1 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.12.3	Confirm Location Service Subscription

API operation name: ConfirmLocationServiceSubscription

Description: Subscription for the Confirm Location Service.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.73

Outputs: See subclause 9.3.2.74

See subclause 9.3.26.2 for the details of usage of this API operation.

* * * End of Changes * * * *