3GPP TSG-SA6 Meeting #67	S6-252081

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

Source:	Nokia

Title:	Pseudo-CR on moving technical solutions addressing adoption gaps to adoption solutions clause

Spec:	3GPP TR 23.700-35 v0.4.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rajesh Babu Natarajan rajesh_babu.natarajan@nokia.com

- Introduction

The Technical solution#2 and Technical solution#3 are actually referring to Adoption gaps and hence they need to be documented under the Adoption solution clause.

- Reason for Change

Solutions indicated as Technical solution #2 and Technical Solution #3 are actually referring to Adoption gaps. The solutions addressing Adoption gaps are documented in the clause 8.2.x and the solutions addressing Technical gaps are doumented in the clause 8.1.x.

Hence, to document the solutions in appropriate clauses, it is proposed to move solutions indicated as Technical solution #2 and Technical solution #3 to Adoptions solutions clause (i.e. 8.2.x).

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.4.0.

* * * First Change * * * *

### 8.1.2	Void

* * * Next Change * * * *

### 8.1.3	Void

* * * Next Change * * * *

### 8.2.X	Adoption solution #X: API design guidelines

#### 8.2.X.1	Solution description

This solution maps to Gap# 5.2.2-1in clause 5.2. This solution is about how to establish common evaluation criteria to support easy-to-use API.

Application developer is one of the important customers of SEAL services APIs. The APIs defined according to the following API guidelines which benefits the application implementation should be met and added to clause 9 of 3GPP TS 23.222 [9]:

-	Each of the SEAL services offered by a SEAL Function shall be self-contained (e.g., contain required parameters to support the service, extensible and evolvable), reusable and use management schemes independently of other SEAL services offered by the same SEAL Function.

-	The expected impact to implementation of Application shall be minimized.

-	Complies with (or adapts to) the API paradigm used by application.

-	SEAL services API provides value added services based on 5G/5GA network capabilities (including 5G core and OAM. The SEAL services can be designed in the following way:

-	Application layer requirement oriented

-	The application layer requirements on 5G system include data transmission performance optimization related, or information/resource related. An application layer requirement-oriented API service can be designed to provide certain optimization on the performance of the transmission of application data (e.g., Reducing latency, or improving reliability), or providing the information (e.g., network statues) which is required by application(s).

Editor's Note:	Whether the “Application layer requirement oriented” is accepted as SEAL service design guidelines needs to be further evaluated.

NOTE: Application layer requirement-oriented API(s) should be proposed with the justification of the use case(s) for the requirements.

#### 8.2.X.2	Solution evaluation

The guidelines defined in this solution are aligned with the principle of defining easy-to-use service(s).

NOTE:	The API service design guidelines will only impact the newly added services and will not impact the existing architecture or API services.

* * * Next Change * * * *

### 8.2.Y	Adoption solution #Y: SEAL service design guidance framework

#### 8.2.Y.1	Description

##### 8.2.Y.1.1	Aspects related to Guidance framework

###### 8.2.Y.1.1.1	General

This guidance framework addresses Gap#5-4-2-1 by providing a systematic approach to help application developers understand and utilize SEAL services effectively. The guidance explains service applicability through detailed scenario analysis, presents comparative advantages over existing solutions, and offers practical implementation recommendations.

###### 8.2.Y.1.1.2	Service Applicability Analysis

The first aspect of this guidance is to enumerate potential application scenarios and industries for each SEAL service, enabling developers from relevant domains to quickly understand the applicability of these services.

For example, when describing the location management service, we identify its applicability in scenarios such as indoor positioning, fleet management, and emergency response. By providing such comprehensive scenario mapping, developers can easily identify which SEAL services align with their specific use cases and industry requirements.

###### 8.2.Y.1.1.3	Comparative Advantages

The second aspect of this guidance is to describe the advantages of SEAL services by comparing them with similar solutions (such as 3GPP core network services and OTT services).

When compared with 3GPP core network services, SEAL services demonstrate advantages in API simplification, enhanced reliability, and unified authentication. In contrast to OTT services, SEAL services provide superior QoS guarantees and deeper network integration. These advantages are illustrated through concrete examples and performance metrics in different application contexts.

Through explicit comparative analysis, developers can more quickly identify and adopt SEAL services that best suit their needs.

###### 8.2.Y.1.1.4	Development Guidance

The third aspect of this guidance is to provide best practice examples for some specific scenarios.

For each SEAL service, we select specific scenarios and provide best practice examples to ensure efficient integration and optimal usage. These guidelines include detailed workflows that outline API interactions step by step, ensuring consistency and reliability. Additionally, we offer architecture-level deployment recommendations, covering scalability, security, and performance considerations tailored to different environments.

By following these best practices, developers can quickly understand SEAL services, accelerate implementation, and seamlessly integrate them into existing systems.

###### 8.2.Y.1.2	Implementation of guidance framework

Editor's Note: This clause is FFS

#### 8.2.Y.2	Solution evaluation

This solution addresses Gap #4 by proposing a guidance framework for SEAL service design, which helps application developers understand and utilize SEAL services effectively.

* * * End of Change * * * *