3GPP TSG-SA WG6 Meeting #67	S6-252089

Fukuoku, Japan, 17th – 21st May 2025	(revision of S6-251447)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0729 | rev | 3 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Eees_EASInformationProvisioning and Eees_EASDiscovery APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Misalignments exist in the specification in the service description clauses and the procedures: - In clause 8.15.2.2, only EEC is as consumer of the Eees_EASInformationProvisioning API, the same also in clauses 8.15.4.1, 8.15.4.2, 8.15.3.2, 8.15.3.3, and 8.15.2.2. EES as consumer of this service API is introduced in clause 8.18.2.4.5, but missing in clause 8.15. - In clauses 8.8.1.1, 8.8.2.4, 8.8.3.2, and 8.8.4.2, it described that EAS sends requests to EES for EAS discovery, reuse the mechanism introduced in clause 8.5. Thus, EAS is also consumer of the Eees_EASDiscovery API. - In clauses 8.8.2A.1 and 8.8.2A.6, CAS may send EAS discovery requests tot EES. Thus, CAS is also consumer of the Eees_EASDiscovery API. This paper proposes to align the consumer of the Eees_EASInformationProvisioning API and the Eees_EASDiscovery API, in the service description clauses and the procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Add description about EES as consumer of Eees_EASInformationProvisioning API to clause 8.15. - Added EAS and CAS to the consumer list of Eees_EASDiscovery API. In rev 3, the main changes include: - Add EES as consumer of the Eees_EASDiscovery service, to align with the description in clauses 8.8.3.2 and 8.8.4.3. - Remove Eees_TargetEASDiscovery from the table 6.7.2-2. - Remove "Target" before the "EASDiscovery" in table 6.7.2-5. - Add NOTE to the EAS discovery response table in clause 8.5.3.3. - Remove Eees_TargetEASDiscovery API from the ACR API table. - Remove API clauses for Eees_TargetEASDiscovery. - Add IEs to Table 8.15.3.2-1 EAS information provisioning request, to align the description in clause 8.18.2.4.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Descripiton about EES as consumer of Eees_EASInformationProvisioning API is missing. Misalignments exist in the specification in the service description calsues and the procedures for the Eees_EASDiscovery API. Duplications exist in the EES service on EAS discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.5.1, 8.5.3.3, 8.8.4.3, 8.8.5.1, 8.8.5.2, 8.15.1, 8.15.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The main changes in rev 3 include: - Add EES as consumer of the Eees_EASDiscovery service, to align with the description in clauses 8.8.3.2 and 8.8.4.3. - Remove Eees_TargetEASDiscovery from the table 6.7.2-2. - Remove "Target" before the "EASDiscovery" in table 6.7.2-5. - Add NOTE to the EAS discovery response table in clause 8.5.3.3. - Remove Eees_TargetEASDiscovery API from the ACR API table. - Remove API clauses for Eees_TargetEASDiscovery. - Add IEs to Table 8.15.3.2-1 EAS information provisioning request, to align the description in clause 8.18.2.4.5. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES, CES | 8.8.3.3 |
| Eecs_ECSRegistration | ECS | 8.17.2.2 |
| Eecs_ECSDiscovery | ECS | 8.17.2.3 |
| Eecs_ECSServiceProvisioning | ECS | 8.17.2.4 |
| Eecs_EASInfoManagement | EES | 8.20.2 |
| Eecs_ACREvents | EES | 8.8.3.11 |

Table 6.7.2-2 summarizes the APIs exposed by the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC, EAS, CAS, EES | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS, EES, CAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | EAS | 8.6.7 |
| Eees_AppContextRelocation | EEC, EAS, EES | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EES, EAS | 8.8.3.6 |
| Eees_SelectedTargetEAS | EAS, CAS, EES | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS, CAS, EES | 8.8.3.8 |
| Eees_ACRParameterInformation | EES, CES | 8.8.3.9 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES, CES | 8.9.4.3 |
| Eees_EASInformationProvisioning | EEC, EES | 8.15 |
| Eees_CommonEasAnnouncement | EES | 8.19 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

Table 6.7.2-3 summarizes the APIs exposed by the CAS.

Table 6.7.2-3: APIs provided by the CAS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Ecas_SelectedEES | EES | 8.8.3.10 |

Table 6.7.2-4 summarizes the APIs exposed the EEC.

Table 6.7.2-4: APIs provided by the EEC

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eeec_ACRegistration | AC | 8.14.4.2 |
| Eeec_EASDiscovery | AC | 8.14.4.3 |
| Eeec_ACRTrigger | AC | 8.14.4.4 |
| Eeec_Services | AC | 8.14.4.5 |
| Eeec_UEId | AC | 8.14.4.6 |

Table 6.7.2-5 summarizes the EES APIs re-used by the CES.

Table 6.7.2-5: APIs re-used by the CES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EASRegistration | CAS | 8.4.3 |
| Eees_UELocation | CAS | 8.6.2 |
| Eees_ACRManagementEvent | CAS | 8.6.3 |
| Eees_AppClientInformation | CAS | 8.6.4 |
| Eees_UEIdentifier | CAS | 8.6.5 |
| Eees_SessionWithQoS | CAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | CAS | 8.6.7 |
| Eees_EASDiscovery | EES | 8.8.3.2 |
| Eees_AppContextRelocation | CAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_SelectedTargetEAS | CAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | CAS | 8.8.3.8 |
| Eees_ACRParameterInformation | EES | 8.8.3.9 |
| Eees_EECContextPush | EES | 8.9.4.3 |

* * * Next Change * * * *

### 8.5.1	General

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest.

EAS discovery enables the EEC, EAS, EES, and CAS to obtain information about available EASs of interest (e.g. instantiated EASs registered with the EES and instantiable EASs that may be created when needed). The discovery of the EASs is based on matching EAS discovery filters provided in the request.

When multiple EASs are discovered for a specific AC, the EEC may select one or more EASs to enable AC communication with one of the selected EASs. The selection algorithm is outside the scope of this specification. Once the EAS is selected, the EEC may subscribe for the ACR event notifications at the EES of the selected EAS, as described in clause 8.8.3.5.2. The EDN configuration information received from ECS may be used for establishing a connection to EAS(s).

EAS discovery may be initiated by the EEC when a certain trigger condition at the UE is met. Some examples are as follows:

-	AC related updates available at the EEC (e.g. due to AC installation/re-installation/activation), AC requesting application server access;

-	Lifetime received via EAS discovery response specified in clause 8.5.3 is expired; or

-	EEC detects the need of application context relocation as in clause 8.8.

NOTE:	When the EEC decides to perform EAS discovery is up to EEC implementation.

EAS discovery may also be initiated by the EAS, EES, or CAS when a certain trigger condition is met, as described in clauses 8.8.3.2 and 8.8.2A.1.

If the EASs consisting of the EAS bundle register to different EESs and if the Bundle Type of the  EAS bundle is set to Direct bundle, then the EEC sends the EAS discovery request message to the associated EES for the bundle EAS information.

* * * Next Change * * * *

#### 8.5.3.3	EAS discovery response

Table 8.5.3.3-1 describes information elements for the EAS discovery response from the EES to the EEC.

Table 8.5.3.3-1: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list (NOTE 1, NOTE 3) | O | List of discovered EAS(s). Each element includes the information described below. |
| >> EAS profile (NOTE 6) | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >> Traffic influence performed indication (NOTE 5) | O | Indicates whether the EES performed traffic influence for the EAS or not. |
| >> Application Group ID list (NOTE 4) | O | List of Application Group IDs associated with EAS |
| >> EES Endpoint | O | The endpoint address (e.g. URI, IP address) of the EES. |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| > Analytics information | O | Analytics data for each discovered application server. |
| >> Statistical data | O | Indicates the statistical analytics data (e.g. number of times the client received expected performance from the EAS). |
| >> Predictive data | O | Indicates predictive analytics data for EAS service status (e.g. EAS schedule, EAS status) change. |
| > Instantiable EAS Information (NOTE 1) | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not instantiated yet, instantiation in progress). |
| > EAS instantiation completion time | O | Indicates the predicted time when the EAS instantiation is expected to complete. |
| >> Instantiation criteria (NOTE 2) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |
| NOTE 1: At least one of the IEs must be included in the Successful response. NOTE 2: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". NOTE 3: If EAS discovery is used for ENS scenario, discovered EAS list contains only those EASs which are allowed to be used by the subscribers of the serving MNO. NOTE 4: "Application Group ID list" IE shall be present when "Application Group profile" is included for "AC profile" in EAS discovery request as specified in clause 8.5.3.2. NOTE 5: “Traffic influence performed indication” is present only when the “EAS selection request indicator” is provided in the EAS discovery request message. NOTE 6: "EAS Service KPIs" IE shall be present in the case T-EAS discovery. |  |  |

* * * Next Change * * * *

#### 8.8.4.3	EAS discovery response

The information elements specified in the Table 8.5.3.3-1 is used for the EAS discovery response sent from the EES to the EAS

.

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_EASDiscovery | Request | Request/Response | EAS, EES, CAS |
| Eees_SelectedTargetEAS | Declare | Request/Response | EAS |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES, CES |
| Eees_AppContextRelocation | Request | Request/Response | EEC, EAS, EES |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_EELManagedACR | Request | Request/Response | EAS |
|  | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
| Eees_ACRStatusUpdate | Request | Request/Response | EAS |
| Eees_ACRParameterInformation | Request | Request/Response | EES, CES |
| Ecas_SelectedEES | Declare | Request/Response | EES |
| Eecs_ACREvents | Subscribe | Subscribe/Notify | EES |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

* * * Next Change * * * *

#### 8.8.5.2	Eees_EASDiscovery API

See clause 8.5.4.2 for details of the API operations for T-EAS discovery.

#### 8.8.5.2.1	Void

#### 8.8.5.2.2	Void

#### See clause 8.8.3.2 for details of usage of the T-EAS discovery operation.

* * * Next Change * * * *

### 8.15.1	General

EAS information provisioning procedure allows the EEC to exchange information with the EES about selected EAS and/or ACR scenario selection, and allows the EES exchange information with another EES about selection of the EAS for the service consumption as described in clause 8.18.2.4.5.

When service continuity is required, ACR scenarios may be combined to perform ACR detection in one or more of the EEC, the EES and the EAS; the related procedures are specified in clauses 8.15.2, 8.6.3 and referred to in clause 8.8.2. The selection of ACR scenario(s) may be performed by the EEC or the EES for a given AC and the selected EAS from the common supported ACR scenarios of AC, EEC, selected EES and selected EAS. The selection of ACR scenario(s) for each EAS in EAS bundles may be performed separately by the EEC or the EES (or the main EES in case of bundle EAS served by multiple EESs, which is the EES registering the main EAS) for a given AC and the selected EAS(s) in a bundle, based on the service continuity support by AC, EEC, EES(s) and EAS(s) when the coordinated ACR need to be performed for the bundle EAS. In addition, the EEC or EES can further determine the one or more ACR scenario(s) for the selected EAS based on the AC service KPI.

NOTE:	How to select ACR scenario(s) at the EEC or EES considering AC service KPI is implementation specific.

Instantiable EAS Information may be provided to the EEC in EAS discovery response and EAS discovery notification, as specified in clause 8.5.3.3 and 8.5.3.6. Triggering the instantiation of an EAS by the EEC may be announced to the EES by including the selected EASID in the EAS information provisioning request without including the selected EAS endpoint.

The EAS information provisioning request types supported are:

-	"ACR scenario selection announcement". Inform the EES about the EAS that has been selected by the EEC and may provide the selected ACR scenario list to the EES. For the EAS bundles scenario, the selected bundled EAS(s) and the selected ACR scenario list for EAS bundles by EEC are included and notified to the EES.

-	"ACR scenario selection request". Inform the EES to perform ACR scenario selection. For the EAS bundles scenario, the request may inform the EES to determine the ACR scenario list for EAS bundles.

-	"EAS selection request". Inform the EES of EAS selection, e.g. for Application Group.

* * * Next Change * * * *

#### 8.15.3.2	EAS information provisioning request

Table 8.15.3.2-1 describes the information elements for EAS information provisioning request from the EEC to the EES.

Table 8.15.3.2-1: EAS information provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | The identifier of the EEC. |
| ACID | M | The identifier of the AC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Selected EAS ID(s) | O | The identifier(s) of the selected EAS or the selected EAS(s) for EAS bundles, which is either instantiated or instantiable. |
| Application Group ID (NOTE 4) | O | Application group identifier as defined in 7.2.11. |
| List of EES(s) (NOTE 4) | O | List of EES information (e.g. address information) corresponding to the Application group ID which is used for the common EAS announcement between these EESs as provided by ECS during service provisioning. |
| Selected EAS Endpoint(s) | O | The endpoint(s) of the selected EAS or the selected EAS(s) for EAS bundles |
| DNAIs and service area of the selected EAS(s) | O | For each selected EAS ID, it includes the DNAIs and/or service area for EAS bundles, as described in "EAS Geographical Service Area" IE, "EAS Topological Service Area" IE and "List of EAS DNAI(s)" IE of Table 8.2.4-1. |
| Request type | O | Request types: - ACR scenario selection announcement - ACR scenario selection request - EAS selection |
| Selected ACR scenario list (NOTE 1) | O | The list of ACR scenarios (or the list of ACR scenarios for EAS bundles) selected by the EEC |
| AC Profile (NOTE 2, NOTE 3) | O | AC Profile as described in Table 8.2.2-1 |
| EEC Service Continuity Support (NOTE 2, NOTE 5) | O | Indicates if the EEC supports service continuity or not. The IE indicates which ACR scenarios are supported by the EEC, also indicates the EEC ability (e.g. EAS bundle information) of handling bundled EAS ACR. |
| Associated EES(s) endpoint | O | EES information of the other EES(s) which support the direct bundled EAS, within the same EDN, and associated with the EASID list. |
| CAS information | O | Target CAS information received from AC. |
| Leading EES ID (NOTE 6) | O | The identifier of the leading EES. |
| Leading EES Endpoint (NOTE 6) | O | The endpoint of the leading EES. |
| NOTE 1: The IE may be present only if Selected EASID(s) and Selected EAS Endpoint(s) are present and Request type is "ACR scenario selection announcement" NOTE 2: The IEs are present only if request type is “ACR scenario selection request” NOTE 3: The IE is present if AC Profile is not shared to EES previously NOTE 4: This IE may be present only if the request type is "EAS selection". NOTE 5: The EAS bundle information is not applicable for proxy type of EAS bundle. NOTE 6: The IEs shall be present only if the request is from leading EES. |  |  |

* * * END of Changes * * * *