3GPP TSG-SA WG6 Meeting #67	S6-252104

Fukuoka, Japan 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0065 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for PIN service switch configure procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PIN Service Switch Configure procedure has misalignments between the procedure description, figure and information flows. Additionally, a reference to the PIN Service Switch Configure is incorrect in the PIN Service Switch procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct a reference to PIN Service Switch Configure procedure. - Align PIN Service Switch Configure procedure, figure and information flow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PIN Service Switch Configure procedure will be misinterpreted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.1.2, 8.7.2.1.3, 8.7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.7.2.1.2	PIN Service Switch procedure

Pre-conditions:

1.	The PIN Client established an application session with an Application Server;

2.	The PIN Client joined a PIN.

Figure 8.7.2.1.2-1: PIN Service Switch procedure

1.	The PIN Client sends a PIN Service Switch request to the PIN Server. The PIN Service Switch request includes the requestor identifier [PIN Client ID], security credential, a PIN identifier [PIN ID], an Application Client identifier [ACID], an Application Server identifier, an application session identifier, can include an IP 4 tuple that describes the traffic of the application session and can include a target PIN Client [PIN Client ID] if a target is known.

2.	Upon receiving the request from the PIN Client, the PIN Server checks if the PIN Client is authorized to request service switching for the given Application Server and validates the request. If the request is authorized and valid, the PIN server can determine a target PIN Client if the target PIN Client was not included in the request. The PIN Server performs a PIN Service Switch Configure procedure as in clause 8.7.2.1.3. The PIN Server can use the PIN ID to identify the PIN Management Client instance and the Application Server identifier, or IP 4 tuple, to identify the Application Server instance.

3.	If the processing of the request was successful, the PIN Server sends a PIN Service Switch response to the PIN Client that indicates the Service Swich request was successfully processed and can include the target PIN Client information if it was decided by the PIN Server. Otherwise, the PIN Server sends a PIN Service Switch response to the PIN Client indicating that the request processing failed and can include appropriate reasons.

Upon reciving the PIN Service Switch response, the PIN Client validates if the request was succesful and can indicate the result to the Application Client; the PIN Client can transfer the application context to the target PIN Client. If the PIN Service Switch response indicated a failure, the PIN Client can attempt perform service switch again for the same or a different target PIN Client considering the failure reason.

* * * Next Change * * * *

##### 8.7.2.1.3	PIN Service Switch Configure procedure

Pre-conditions:

1.	The PIN Client successfully requested a service switch with the PIN Server

2.	The AS is successfully subscribed for PIN service switch notifications with the PIN server.

Figure 8.7.2.1.3-1: PIN Service Switch Configure procedure

1.	The PIN Server sends the PIN Configuration Service Switch Configure request to the PIN Management Client. The PIN Configuration Service Switch Configure request includes the requestor identifier, security credential, PIN identifier, Application Client identifier, Application Server identifier, target PIN Client identifier, application session identifier and can include IP 4 tuple that descibes the traffic of the application session.

NOTE 1:	The PIN Server first performs PIN Configuration Service Switch Configure procedure with the PIN Management Client, then if successful, notifies the Application Server.

2.	Upon receiving the request from the PIN Server, the PIN Management Client checks if the PIN Server is authorized to request service switch configuration and validates the request.

If the request is authorized and valid, the PIN Management Client prepares for sending PIN Management Service Switch Configure request to the PIN Gateway Client or the target PIN Client. The PIN Management Client can use the PIN ID to identify the PIN Gateway Client instance and the target PIN Client identifier to identify the target PIN Client.

3.	The PIN Management Client sends the PIN Management Service Switch Configure request to the PIN Gateway Client or the target PIN Client including the information defined in step 1.

NOTE 2:	The PIN Management Client first performs PIN Management Service Switch Configure procedure with the PIN Gateway Client, then if successful, with the target PIN Client.

4.	Upon receiving the request from the PIN Management Client, the PIN Gateway Client or the target PIN Client checks if the PIN Managment Client is authorized to request service switch configuration and validates the request. If the PIN Management Client is authorized and the request is valid, the PIN Gateway Client or the target PIN Client use the information provided in the request to switch the application session to the target PIN Client.

NOTE 3:	The target Application Client needs to receive the application context prior to re-establishing a switched application session with the Application Server.

NOTE 4:	How the target Application Client recovers the switched application session is out of scope of this specification.

5.	If the processing of the request was successful, the PIN Gateway Client or the target PIN Client send a PIN management Service Switch Configure response to the PIN Management Client indicating the processing was successful. Otherwise, the PIN Gateway Client or target PIN Client send a PIN Management Service Switch Configure response to the PIN Management Client indicating that processing the request failed and can include appropriate reasons.

6.	If the processing of the request was successful, the PIN Management Client sends a PIN Configuration Service Switch Configure response to the PIN Server that indicates the request was successfully processed. Otherwise, the PIN Management Client sends a PIN Configuration Service Switch Configure response to the PIN Server indicating the processing the request failed and can include appropriate reasons.

7.	The PIN Server sends the PIN service switch notification to the Application Server if the procedure with the PIN Management Client was successful in step 6. The PIN service switch notification includes the PIN server identifier, PIN identifier, Application Client identifier, target PIN Client identifier, application session identifier and can include IP 4 tuple that descibes the traffic of the application session.

8.	Upon receiving the PIN service switch notification, the Application Server uses the information provided in the notification to switch the application session to the target PIN client.

* * * Next Change * * * *

#### 8.7.3.4	PIN configuration service switch configure request

Table 8.7.3.4-1 describes information elements for the PIN configuration service switch configure request that is sent from the PIN server to the PIN management client.

Table 8.7.3.4-1: PIN configuration service switch configure request

| Information element | Status | Description |
| --- | --- | --- |
| PIN server identifier | M | Requestor identifier. |
| Security credentials | M | Security credentials of the PIN server. |
| PIN identifier | M | Identifier of the PIN. |
| Application client identifier | M | Identifier of the application client. |
| Application server identifier | M | Identifier of the application server. |
| Application session identifier | M | Identifier of the application session. |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) |
| Target PIN client identifier | M | Target PIN client identifier. |

* * * End of Changes * * * *