3GPP TSG-SA WG6 Meeting #67	S6-252108

Fukuoka, Japan, 19th – 23rd May 2025

Source:	InterDigital

Title:	Solution on hierarchical AIMLE server registration

Spec:	3GPP TR 23.700-83 v0.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a solution for Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

2. Reason for Change

This contribution enables AIMLE server registration in hierarchical AIMLE deployment considering a hierarchical AIMLE deployment in edge data networks as described in Annex A.4 of 3GPP TS 23.482.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.1.0.

* * * First Change * * * *

## 7.x	Solution #x: Hierarchical AIMLE server registration

### 7.x.1	Solution description

#### 7.x.1.1 Hierarchical AIMLE server registration

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes AIMLE server registration procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482.

A central AIMLE server needs information about AIMLE clients available in different EDN service areas when, for example when performing operations requiring the selection of AIMLE clients, and AIMLE clients in an EDN service area may register to the edge deployed AIMLE server. Therefore, interactions between the edge deployed AIMLE servers and the central AIMLE server are needed to support hierarchical AIMLE deployment scenarios.

Figure 7.x.1.1-1 describes a procedure for an AIMLE server to register with another AIMLE server. In such a scenario, an AIMLE server deployed in an EDN registers to a central AIMLE server.

Pre-conditions:

1.	AIMLE clients have registered with an edge deployed AIMLE server as described in clause 8.7.2.2 of 3GPP TS 23.482. The edge deployed AIMLE server receives AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE server has been pre-configured with endpoint information of a central AIMLE server.

Figure 7.x.1.1-1. Hierarchical AIMLE server registration procedure

1.	The edge deployed AIMLE server sends a hierarchical AIMLE server registration request to a central AIMLE server. The registration request includes the requestor identifier, security credentials, endpoint information and a list of AIMLE clients registered with the AIMLE server. The request may include EDN information  and AIMLE server capabilities.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server stores the registration information and creates a registration identifier. The central AIMLE server sends a hierarchical AIMLE server registration response to the requestor. If the central AIMLE server has registered the requestor, the response includes an indication of success, a registration identifier and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

#### 7.x.1.2 Hierarchical AIMLE server registration update

Pre-conditions:

- The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.x.1.2-1. Hierarchical AIMLE server registration update procedure

1.	The edge deployed AIMLE server sends a hierarchical AIMLE server registration update request to the central AIMLE server. The request includes the registration identifier, security credentials and may include updated AIMLE server information as described in Table 7.x.3.3-1. If the request is to renew the registration, only the registration identifier is included in the request.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server updates the information associated with the registration identifier and may update the registration expiration time if the request only includes the registration identifier. The central AIMLE server sends a hierarchical AIMLE server registration update response to the requestor. If the central AIMLE server has updates the registration, the response includes an indication of success, and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

#### 7.x.1.3 Hierarchical AIMLE server de-registration

Pre-conditions:

- The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.x.1.3-1. Hierarchical AIMLE server de-registration procedure

1.	The edge deployed AIMLE server sends a hierarchical AIMLE server de-registration request to the central AIMLE server. The de-registration request includes the registration identifier and security credentials.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server terminates the registration indicated in the request. The central AIMLE server sends a hierarchical AIMLE server de-registration response to the requestor. If the central AIMLE server has de-registered the requestor, the response includes an indication of success. If the requestor was not de-registered, the response includes an indication of failure and may include a reason for failure.

### 7.x.2	Architecture impacts

No architectural impact has been identified.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	Hierarchical AIMLE server registration request

Table 7.x.3.1-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for registering AIMLE server and clients in the EDN service area.

Table 7.x.3.1-1: Hierarchical AIMLE server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the registering AIMLE server. |
| Security credentials | M | The security credentials of the requestor. |
| AIMLE server endpoint | M | The endpoint information (e.g., URL, URI, IP address, port) of the registering AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registering AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O (NOTE) | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O (NOTE) | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registering AIMLE server. |
| List of registered AIMLE clients | M | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registering AIMLE server. |
| NOTE: At least one of the information element is present. |  |  |

#### 7.x.3.2	Hierarchical AIMLE server registration response

Table 7.x.3.2-1 shows the response sent by the central AIMLE server to the edge deployed AIMLE server in response to the registration request.

Table 7.x.3.2-1: Hierarchical AIMLE server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration was successful. |
| > Registration ID | M | The identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the registration. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration failed. |
| > Cause | O | Indicates the cause of the failure |
| NOTE: One of the information element must be present. |  |  |

#### 7.x.3.3	Hierarchical AIMLE server registration update request

Table 7.x.3.3-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the hierarchical AIMLE server registration update request.

Table 7.x.3.3-1: Hierarchical AIMLE server registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of an existing registration. |
| Security credentials | M | The security credentials of the requestor. |
| AIMLE server endpoint | O | The endpoint information (e.g., URL, URI, IP address, port) of the registered AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registered AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O | The identifier of the EDN service area associated with the registered AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registered AIMLE server. |
| List of registered AIMLE clients | O | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registered AIMLE server. |

#### 7.x.3.4	Hierarchical AIMLE server registration update response

Table 7.x.3.4-1 shows the hierarchical AIMLE server registration update response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.x.3.4-1: Hierarchical AIMLE server registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration update was successful. |
| > Expiration time | O | Indicates the expiration time of the updated registration. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration update failed. |
| > Cause | O | Indicates the cause the failure. |
| NOTE: One of the information element must be present. |  |  |

#### 7.x.3.5	Hierarchical AIMLE server de-registration request

Table 7.x.3.5-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the hierarchical AIMLE server de-registration request.

Table 7.x.3.5-1: Hierarchical AIMLE server de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the existing registration. |
| Security credentials | M | The security credentials of the requestor. |

#### 7.x.3.6	Hierarchical AIMLE server de-registration response

Table 7.x.3.6-1 shows the hierarchical AIMLE server de-registration response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.x.3.6-1: hierarchical AIMLE server de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the de-registration was successful. |
| Failure response | O (NOTE) | Indicates that the de-registration failed. |
| > Cause | O | Indicates the cause of the failure. |
| NOTE: One of the information element must be present. |  |  |

#### 7.x.4 Solution evaluation

Editor's Note:		The solution evaluation is FFS.

* * * End of Changes * * * *