3GPP TSG-SA WG6 Meeting #67	S6-252110

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0383 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating cl.7.17.3.1.3 and cl. 7.17.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Similar to initiating an ad hoc group emergency alert (which is done based on criteria), in the case of the ad hoc group call setup based on criteria the MCData server keeps checking whether other MCData clients meet the criteria, or whether the participating MCData clients no longer meet the criteria. This aspect is clearly described in the ad hoc group emergency alert initiation in clause 10.10.3.3.1 of 3GPP TS 23.280 but it is missing in the ad hoc group call setup via criteria as in clauses 7.17.3.1.3 and 7.17.3.2.3. Therefore, this CR clarifies this aspect. Similar updates are agreed in S6-251256 and S6-251257. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a sentence in clauses 7.17.3.1.3 and 7.17.3.2.3 similar to the one in clause 10.10.3.3.1 of 3GPP TS 23.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear that the MCData server continuously checks adding or removing MCData users into an ongoing ad hoc group call which is setup via criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.3.1.3 and 7.17.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.17.3.1.3	Ad hoc group data communication setup with MCData server determining the participants lists

Figure 7.17.3.1.3-1 below illustrates the ad hoc group data communication setup procedure initiated by the MCData user and MCData client 1 wherein the list of participants is determined by the MCData server based on the citeria received from the MCData client.

Pre-conditions:

1.	The MCData user at MCData client 1 is authorized to initate ad hoc group data communication.

2.	The MCData user at MCData client 1 wants to invite MCData users who are satisying certain criteria for the ad hoc group data communication.

Figure 7.17.3.1.3-1: Ad hoc group data communication participants determined by MCData server

1.	User at MCData client 1 would like to initiate an ad hoc group data communication in-order to invite the participants satisfying specific criteria. The MCData client 1 initiates the ad hoc group data communication by sending the ad hoc group data session request containing the details of the criteria to be applied by the MCData server for determining the participants list. If end-to-end encryption is supported, the Encryption supported information element shall be set to true and pre-configured MCData group whose configuration is to be applied is included. An SDP offer containing the MCData client media parameters is included. If the MCData user of MCData client 1 has selected a functional alias, then the ad hoc group data session request contains that functional alias.

If the MCData user at MCData client 1 initiates an MCData emergency ad hoc group data communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i.	the MCData ad hoc group data session request shall contain an emergency indicator;

ii.	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state of MCData client 1 is retained until explicitly cancelled by the user of MCData client 1.

2.	If the ad hoc group data communication is supported, the MCData server verifies whether the user at MCData client 1 is authorized to initiate an ad hoc group data communication. If not authorized, the MCData server rejects the ad hoc group data session request as specified in the step 3. The MCData server accepts the ad hoc group data session request if the ad hoc group data communication is supported and authorized.

If functional alias is present, the MCData server checks whether the provided functional alias is allowed to be used and has been activated for the user.

If location information was included in the ad hoc group data session request, the MCData server checks the privacy policy of the MCData user to decide if the location information of MCData client 1 can be provided to other users on the data communication (refer to Annex A.3 "Authorisation to provide location information to other MCData users on a data communication when talking").

If an emergency indicator is present in the received MCData ad hoc group data session request, the MCData ad hoc group is considered to be in the in-progress emergency state until this ad hoc group data communication is terminated;

If an imminent peril indicator is present in the received MCData ad hoc group data session request, the MCData ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group data communication is terminated. and

If the information received in the request in step 1 does not contain an ad hoc group ID from an ad hoc group emergency alert, the MCData server forms the ad hoc group by using received information, and determines the preconfigured group to be used for the configuration of the ad hoc group. The MCData server assigns a MCData group ID for the newly formed ad hoc group. Further, the ad hoc group participants are included to ad hoc group once determined as specified in the step 4.

3.	The MCData server shall send the ad hoc group data session request return message to MCData client 1 containing the below:

i.	The MCData ad hoc group ID, either generated by the MCData server, if not included in the ad hoc group call request of step 1, or if the provided MCData ad hoc group ID is not accepted by the MCData server, or provided by the MCData client 1 if the ad hoc group ID is from an ad hoc group emergency alert;

ii.	The group ID of the pre-configured group to be used for the ad hoc group communication (only included when the ad hoc group data session is authorized); and

iii.	Result of whether the ad hoc group data session request is authorized or not

If the ad hoc group data session request is not authorized, the MCData server and MCData client 1 shall not proceed with the rest of the steps.

4.	The MCData server determines the list of participants to be invited for the ad hoc group data communication based on the information present in the information element Criteria for determining the participants. This information element could carry either criteria or indicator identifying pre-defined criteria or a combination of both. Depending on the criteria provided and based on local policy, the MCData server may modify the content of the criteria received in step 1 to determine the list of participants.

NOTE 1:	The content of the Criteria information element, the details of the pre-defined criteria, and the way how their MCData server determines the list of participants are left to implementation.

5.	The MCData server sends the ad hoc group data session requests towards the MCData clients 2 and 3. While sending the ad hoc group data session requests, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group data communication if end-to-end encryption is requested. The MCData server considers the ad hoc group data communication participants as implicitly affiliated to the ad hoc group.

6.	The receiving MCData clients notify their corresponding MCData user about the incoming ad hoc group data communication.

7.	The receiving MCData clients may accept or reject the ad hoc group data session requests and send ad hoc group data session responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

8.	The MCData server sends the ad hoc group data session response to MCData client 1 through the signalling path to inform about successful data communication establishment. The ad hoc group data session response contains the call resulting criteria used by the MCData server to determine the list of participants to be invited.

9.	The MCData server may notify the initiating MCData user of all MCData users who acknowledged the ad hoc group data session request and joined the ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the MCData ad hoc group data communication. If the authorized users (not shown in figure) are configured to receive the participants information of ad hoc group data communication, the MCData server provides ad hoc group data session notify about all MCData users who acknowledged the ad hoc group data session request and joined the ad hoc group data communication, and when MCData users joins or leaves the MCData ad hoc group data communication. All ad hoc group data session notify messages contain the call resulting criteria used by the MCData server to determine the list of participants to be invited.

NOTE 2:	The authorized user can learn who is currently affiliated to the current adhoc group data communication.

10.	MCData client 1, MCData client 2 and MCData client 3 establish media plane resources.

The MCData server continuously checks whether other MCData clients meet the criteria in order to add them to the ongoing ad hoc group communication, or whether any of the participating MCData clients no longer meet the criteria in order to remove them from the ongoing ad hoc group communication.

# * * * * Second change * * * *

##### 7.17.3.2.3	Ad hoc group data communication setup – Participants list determined by the MCData server

Figure 7.17.3.2.3-1 below illustrates the ad hoc group data communication setup procedure initiated by an authorized user wherein the list of participants is determined by the MCData server based on the citeria received from the MCData client and determined MCData users are from multiple MCData systems.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system share their users' information to the primary MC system.

2.	The authorized MCData user/dispatcher belongs to the primary MC system.

3.	The MCData server 1 of the primary MC system is where the authorized MCData user/dispatcher creates the ad hoc group.

4.	Some users of the ad hoc group may belong to MCData server 2 of the partner MC systems.

5.	The pre-configured group identity and pre-configured group configuration to be used for an ad hoc group have been preconfigured in MCData client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

Figure 7.17.3.2.3-1: Ad hoc group data communication setup involving multiple MCData systems

1-3.	Same as described in subclause 7.17.3.1.1.

4.	The MCData server 1 determines the list of participants from the primary MC system and determines the partner MC system to be involved in the ad hoc group data communication based on the information present in the information element Criteria for determining the participants. This information element carries the criteria, indicator identifying pre-defined criteria, or a combination of both. Depending on the criteria provided and based on local policy, the MCData server may modify the content of the criteria received in step 1 to determine the list of participants.

NOTE 1:	The content of the Criteria information element, the details of the pre-defined criteria, and the way how the MCData server determines the list of participants are left to implementation.

5.	The MCData server 1 needs to involve the partner MC system based on the agreement and based on the criteria for determining the participants list, it sends the ad hoc group data session get userlist request to the MCData server 2. This request carries the call resulting criteria to be used by the partner MC system as determined in step 4.

6.	MCData server 2 evaluates the criteria and determines the participants satisfying the criteria (i.e. MCData client 3 and MCData client 4) and sends the response containing the list of MCData users satisfying the criteria. The MCData server 2 may apply local policies if any while determining the participants satisfying the criteria.

7.	The MCData server 1 compiles the list of participants to be invited for the ad hoc group data communication including the participants from both primary and partner MC system.

8a-8b.	The MCData server 1 sends the ad hoc group data session request towards the MCData client 3 and MCData client 4. While sending the ad hoc group data session request, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group data communication if end-to-end encryption is requested. The MCData server 1 considers the ad hoc group communication participants as implicitly affiliated to the ad hoc group.

9.	The MCData server 1 sends the ad hoc group data session request towards the MCData client 2. While sending the ad hoc group data session request, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group data communication if end-to-end encryption is requested.

10a-10c.	The MCData clients receive incoming ad hoc group data communication and further notify their corresponding MCData users.

11.	The MCData client 2 accepts the ad hoc group data session request, and sends ad hoc group data session response to the MCData server 1.

12.	The MCData client 3 accepts the ad hoc group data session request, and sends ad hoc group data session response to the MCData server 1.

13.	The MCData client 4 accepts the ad hoc group data session request, and sends ad hoc group data session response to the MCData server 1.

14.	The MCData server 1 sends the ad hoc group data session response to MCData client 1 through the signalling path to inform about result of the participants responses. The ad hoc group data session response contains the call resulting criteria used by the MCData server to determine the list of participants to be invited.

15.	The MCData server 1 may notify the initiating MCData user of all MCData users who acknowledged the ad hoc group data session request and joined the ad hoc group data communication. The MCData server 1 more than once during the data communication may send this notification to the initiating MCData user whenever an MCData user joins or leaves the MCData ad hoc group data communication. If the authorized users (not shown in figure) are configured to receive the participants information of ad hoc group data communication, the MCData server provides ad hoc group data session notify about all MCData users who acknowledged the ad hoc group data session request and joined the ad hoc group data communication, and when MCData users joins or leaves the MCData ad hoc group data communication. All ad hoc group data session notify messages contain the call resulting criteria used by the MCData server to determine the list of participants to be invited.

16.	The MCData client 1, MCData client 2, MCData client 3 and MCData client 4 establish media plane resources for data communication.

NOTE 2:	The ad hoc group data session request and response exchanged between MCData server 1 of primary MC system and MCData client 3/MCData client 4 will always traversal through the MCData server 2.

The MCData server at the primary MC system continuously checks whether other MCData clients meet the criteria in order to add them to the ongoing ad hoc group communication, or whether any of the participating MCData clients no longer meet the criteria in order to remove them from the ongoing ad hoc group communication.

# * * * * End of changes * * * *