3GPP TSG-SA WG6 Meeting #67	S6-252115

Fukuoka, Japan, 19 - 23 May 2025

Source:	China Mobile

Title:	Sol for KI#3 support for QoS information setting and adjustment based on available bitrate

Spec:	3GPP TR 23.700-51

Agenda item:	9.5

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#3,

-	Based on the 5GS information (e.g., congestion information, data rate information, round trip delay, available bitrate and data rate limitation), whether and how application enabler layer can be enhanced to support to traffic adaptation (e.g., enhanced rate control based on exposed available data rate/ rate limiting information)?

2. Reason for Change

In Release 18 and Release 19, SA2 has defined that 5GS and XR/media services cooperate to provide a better user experience using External Network Exposure. The 5GS can expose available bitrate described in clause 5.37.4 of TS 23.501[3]. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation[3]. During this duration, NG-RAN can not provide a better bitrate, so SEALDD transmission bitrate is suggested to keep align with NG-RAN. If the available bitrate is changed, SEALDD can adjust QoS to keep align with new value of NG-RAN.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[7]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[x]	3GPP TS 23.433: "Data Delivery enabler for vertical applications".

* * * Next Change * * * *

5	Solutions

5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x |  |  | X |  |
| Sol #2 |  |  |  |  |

5.x	Solution #x: QoS information setting and adjustment based on available bitrate

5.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [x].

5.x.2	Solution description

This solution addresses on application enabler layer enhancement to support for QoS information setting and adjustment based on available bitrate exposed from PCF.

In Release 18 and Release 19, SA2 has defined that 5GS and XR/media services cooperate to provide a better user experience using External Network Exposure. The 5GS can expose available bitrate described in clause 5.37.4 of TS 23.501[3]. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation[3]. During this duration, NG-RAN can not provide a better bitrate, so SEALDD transmission bitrate is suggested to keep align with NG-RAN. If the available bitrate is changed, SEALDD can adjust QoS to keep align with new value of NG-RAN.

5.x.2.1	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [x] can be enhanced (highlighted in bold italics) as follows.

9.12.2.1.2 SEALDD enabled XR data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing XR data transmission connection, and the SEALDD facilitates the XR application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled XR data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for XR traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_XRTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. If VAL server does not provides QoS requirements, SEALDD derives QoS information from 5GC via N5. If UL and/or DL available bitrate is provided in the QoS monitoring report, SEALDD determines QoS information based on available bitrate and subscribes QoS Monitoring. When available rate exceed a pre-defined threshhold (e.g. available bitrate + N bit), SEALDD server receive notification and adjust QoS information into new value. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes XR transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the XR transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates. 7-8. Same as step 8-9 of clause 9.2.2.2. The XR application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

5.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End Change * * * *

<Proposed change in revision marks>