3GPP TSG-SA WG6 Meeting #67	S6-252144

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252xxx)

Source:	Ericsson

Title:	Pseudo-CR on SEAL service for power saving in IoT devices

Spec:	3GPP TR 23.700-35 v. 0.4.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR addresses the gap analysis discussed in clause 4.4.

2. Reason for Change

A solution is introduced in this pCR to address the technical gap in clause 4.4 related to power saving in IoT devices.

3. Conclusions

A solution on SEAL service for power saving in IoT devices.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v. 0.4.0

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS)" 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[12]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[13]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[14]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[x]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[y]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

* * * Second Change * * * *

### 8.2.x	Solution #x: New SEAL service for power saving in IoT devices

This solution corresponds to the technical gap described in clause 4.4, where the use of SS_NetworkResourceAdaptation and MonitoringEvent APIs to meet IoT application requirement, e.g., power saving, is not clear.

#### 8.2.x.1	Solution description

The solution presents two procedures which correspond to two cases:

-	Clause 8.2.x.3 describes the procedure related to the case when VAL server triggers power saving assist service for power saving in IoT devices.

-	Clause 8.2.x.4 describes the procedure related to the case when VAL client triggers power saving assist service for power saving in IoT devices.

The related information element is described in clause 8.2.x.2.

#### 8.2.x.2	Information elements

Table 8.2.x.2-1: Power saving assist request

| Information element | Status | Description |
| --- | --- | --- |
| VAL User or VAL UE(s) ID(s) (NOTE) | M | The identity of the VAL user whose UE(s) are target for power savings configuration. Alternatively, is the UE ID on which the NRM client resided |
| VAL Service ID | M | The identity of the VAL service |
| Power saving requirements | M | The desired power saving requirements for the VAL UE(s), e.g., power saving mode (power-saving, balanced, performance) to be enforced for the target user(s) |
| NOTE: If the consumer is the NRM client, this IE shall be the UE ID. If the consumer is VAL server, the IE may be a list of UE IDs or identity of VAL user. |  |  |

Table 8.2.x.2-2: Power saving configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE which is a target for power savings configuration |
| Requested power saving configuration | M | The desired power saving configuration to be updated for the target user, e.g., DRX configurations in clause 6.2.3 and clause 6.7.3 of 3GPP TS 36.331 [x] or DRX configurations in clause 6.2.3 of 3GPP TS 38.331 [y]. |

Table 8.2.x.2-3: Power saving configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE which is a target for power savings configuration |
| Result | M | Indicates whether the result of the power saving configuration request is successful or not |
| Accepted power saving configuration | O (NOTE) | Indicates the accepted power saving configuration |
| NOTE: This information element is present if the accepted power saving configurations are different from the desired one. |  |  |

Table 8.2.x.2-4: Power saving assist response

| Information element | Status | Description |
| --- | --- | --- |
| VAL User or VAL UE(s) ID(s) (NOTE) | M | The identity of the VAL user whose UE(s) are target for power savings configuration. Alternatively, is the UE ID on which the NRM client resided. |
| >Result | M | Indicates whether the result of the power saving configuration request is successful or not |
| >Accepted power saving configuration | O | Indicates the accepted power saving configuration |
| NOTE: If the consumer is the NRM client, this IE shall be the UE ID. If the consumer is VAL server, the IE may be a list of UE IDs or identity of VAL user. |  |  |

#### 8.2.x.3	Procedure case 1

Figure 8.2.x.3-1 describes the procedure when the VAL server triggers power saving assist service for power saving in IoT devices.

Figure 8.2.x.3-1: Procedure for VAL server triggers power saving assist service

1.	VAL server requests the NRM Server to assist power saving of the VAL user(s) that are part of VAL Service. The request contains the list of VAL UEs and VAL Users that are targets for the power saving, VAL service ID, power saving requirements, e.g., mode (power-saving, balanced, performance) (see Table 8.2.x.2-1).

2.	NRM server determines, based on the VAL service ID, UE(s) access type and power saving mode, the power saving configuration, network parameters and communication pattern parameters for each requested VAL UE(s). Furthermore, the SEAL Server may determine the VAL UE ID(s) associated with the VAL user(s) included in step 1.

3.	SEAL Server interacts with 3GPP CN (e.g., CpProvisioning and/or NpConfiguration APIs) to provision the VAL UE(s) network parameters and communication pattern parameters determined in step 2.

4.	NRM Server interacts with NRM Client with recommended power saving configuration for the UE (e.g., the determined power saving configuration in step 2) (see Table 8.2.x.2.-2).

5.	The target UE interacts with 3GPP RAN as per current procedures, to indicate the desired power saving configuration (e.g., DRX configuration). The UE receives from 3GPP RAN the accepted power saving configuration.

6.	NRM Client responds to the NRM Server with power saving configuration response and indicates the accepted power saving configuration (see Table 8.2.x.2-3).

7.	NRM Server responds to the VAL Server with power saving assist response (see Table 8.2.x.2-4).

#### 8.2.x.4		Procedure case 2

Figure 8.2.x.4-1 describes the procedure when the VAL client triggers power saving assist service for power saving in IoT devices.

Figure 8.2.x.4-1: Procedure for VAL client triggers power saving assist service

0.	VAL Server may interact with VAL UE(s) that are part of VAL Service for power saving request.

1.	NRM Client requests the NRM Server to assist power saving of the VAL UE(s). The NRM client indicates the desired power saving configuration to the NRM Server (see Table 8.2.x.2-1).

2.	Steps 2-6 as in Figure 8.2.x.3-1.

3.	The NRM Server responds to the NRM Client with power saving assist response (see Table 8.2.x.2-4).

4.	 The VAL UE may interact with VAL Server for power saving response.

#### 8.2.x.5	Solution evaluation

The solution addresses the existing technical gap in clause 4.4. related to SS_NetworkResourceAdaptation and MonitoringEvent APIs e.g., for power saving scenarios. The solution describes two cases based on the entity that triggers power saving assist service for power saving in IoT devices.

* * * End of Changes * * * *