3GPP TSG-SA WG6 Meeting #67	S6-252151

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0392 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the procedure and IEs for SL positioning management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 send a LS to SA6 to ask the details about the IEs and procedures of SL positioning management service. The following are the reasons for the changes: - As there is no description about "SL positioning parameters"in the related procedure and there is no "SL positioning parameters" reported in SL positioning management notification, we have no idea how to utilize such parameters in the SL positioning management service. Suggest to remove such IE in Table 9.3.2.61-1. - The purposes of the SL positioning management susbcription request is to identify which UEs are served as reference or target UEs. And the LMS can categorize them by their capabilities and slection criteria specificed by VAL server. Then LMS will report the results to the VAL server. The notification report is not event-triggered. So there is no need to add any triggering events in the request and notification information flow. - The “Reason” IE means failure reason for the notification report. But the example “the reference UE cannot be found” is not the reason but the result which can be showed “NULL” in “Reference and target UE ID (s) and address” IE to indicate that. So the example should be corrected to be a reason. - “client UE” is the UE that triggered the service request via UE itself. The SEAL LMS cannot select and determine which UEs are the client UE. So just remove the client UE in the procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove SL positioning parameters IE in Table 9.3.2.61-1. - Remove triggering events in the request and notification information flow. - Correct the Reason example in notification information flow. - Remove the client UE in the procedure. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SL positioning management service can not be performed in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.61, 9.3.2.63, 9.3.2.64, 9.3.24.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.61	SL positioning management subscription request

Table 9.3.2.61-1 describes the information flow from the VAL server to the location management server for SL positioning management subscription request.

Table 9.3.2.61-1: SL positioning management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server. |
| VAL service ID | M | Identity of the VAL service (SL positioning service) for which the location information is subscribed. |
| VAL service area | O | The geographical area /zone or topological area (e.g. cells) for which the subscription applies. |
| VAL UE/user Identities list | O | List of VAL users or VAL UEs participating at the SL positioning service (will be served as reference and target VAL UEs). |
| VAL UE selection criteria | O | Selection criteria of UEs of different roles (i.e., reference UEs and target UEs). |
| Reporting configuration | M | It indicates the threshold and interval time between consecutive reports. |

* * * Next Change * * * *

9.3.2.63	SL positioning management notification

Table 9.3.2.63-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.63-1: SL positioning management notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription. |
| List of reference and target UE ID (s) and address | M | Identifies the list of UEs to serve as reference and target UEs. |
| List of Client UE IDs and addresses | O | The identities of the client UEs which are allowed to get ranging information for other UEs. |
| Reason | O | It indicates the failure reason for the notification report (e.g., “the UE is not allowed to be a reference UE "). |

* * * Next Change * * * *

9.3.2.64	SL positioning configuration command

Table 9.3.2.64-1 describes the information flow from the location management server to the location management clients or from LMC to VAL client.

Table 9.3.2.64-1: SL positioning configuration command

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requestor SEAL server or the subscriber VAL server |
| VAL service ID | M | Identity of the VAL service (SL positioning service) |
| VAL service area | M | The geographical area /zone or topological area (e.g. cells) for which the subscription applies. |
| VAL UE/user Identities list | M | List of VAL users or VAL UEs participating at the SL positioning service (will be served as reference and target VAL UEs). |
| SL positioning / Ranging configuration parameters | M | The configuration information for the SL positioning service. This includes the roles of the VAL UEs (reference or target) in the SL positioning service. |
| Reporting configuration | M | Indicates the threshold and interval time between consecutive reports expected from the target VAL UEs. |

* * * Next Change * * * *

9.3.24	Support for sidelink positioning / ranging management

9.3.24.1	Procedure of SL positioning /ranging management service

The procedure provides a mechanism for configuring and provisioning information on the entities to be used and their responsibilities for a SL positioning / ranging service. In this procedure, an SL positioning enabler capability at LMS is authorized / delegated by SL positioning server to support the provisioning/delivery of ranging services in an area.

Figure 9.3.24.1-1 shows a high-level procedure with the details.

Figure 9.3.24.1-1: Support for SL positioning procedure

1.	The VAL server (SL Positioning / Ranging Server) sends an SL positioning management subscription request as defined in clause 9.3.2.61 to SEAL LMS to provision and manage SL positioning / ranging operation and configurations.

2.	The SEAL LMS authorizes the VAL subscription request.

3.	The SEAL LMS sends an SL positioning management subscription response to the requestor VAL Server.

4.	The SEAL LMS discovers all the SEAL LMCs in the target service area, and categorizes them by their capabilities and slection criteria specificed by VAL server in step 1as target UE, Reference UEs, or even Located UEs (if Reference UE location is known or for out of coverage scenarios). The identification of UEs in a target area happens via existing SEAL LMS procedures (as in clauses 9.3.10 and 9.3.12).

5.	The SEAL LMS after getting all UE information at the target area, calculates the relative distances among them, and may request/receive HDMaps (e.g., from other VAL servers) for the target area.

Then, SEAL LMS evaluates the need of reference UE (based the information in step 4), and if reference UE is needed (e.g. in NLoS situation), the SEAL LMS selects the appropriate VAL UE to serve as reference and configures the VAL UEs (target, reference) with configuration information (thresholds, reporting configurations).

The LMS may also leverage analytics services (e.g. NWDAF) to obtain VAL UE location related predictions (e.g. UE location prediction, relative distance prediction) to discover reference UEs.

NOTE:	The selection of UE as reference UEs can be based on their capabilities and the selection criteria specified by the VAL server.

6.	The SEAL LMS sends a SL positioning management notify message to the VAL server to indicate the roles of the VAL UEs and acknowledging that the SL positioning provisioning is performed.

7.	The SEAL LMS configures the SEAL LMCs with the configuration information applicable to the corresponding VAL UEs based on step 5, and in particular the expected role (acting as reference UE, target UE) in relation ranging/SL positioning service. Based on this configuration, the LM clients interacts each other using off-network location management procedures as described in clause 9.5.

* * * End of Changes * * * *