3GPP TSG-SA WG6 Meeting #67	S6-252160

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252xxx)

Source:	Lenovo

Title:	New gap and solution on SB SEAL enhancements

Spec:	3GPP TR 23.700-35 v0.4.0

Agenda item:	8.9

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides a new gap analysis and solution on SEAL supporting MSA.

2. Reason for Change

This paper provides a new gap analysis and solution on SEAL supporting MSA.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.4.0.

* * * First Change * * * *

## 5.x	Generic gap #X: SEAL supporting MSA

### 5.x.1	Analysis

3GPP system uses service-based representations of the architecture. However, different architectures have different types of SBA:

- 3GPP SA2 which is about the 3gpp system architecture uses conventional SBA, where the NF is the service producer and the SBI bus is used for the interaction with service consumers (NFs/AFs).

- 3GPP SA5, which is about OAM aspects of the network, uses service-based management architecture, closer to microservice paradigm. 3GPP SA5 specifies the notion of MnS as an API that can be integrated in any management function. Allows an MnS Producer to interact with an MnS Consumer (a service producer with a service consumer) via a standardized MnS interface.

- 3GPP SA6 uses SA2 paradigm for SEAL, however in other enablers (like EDGEAPP) the service-based support has different variants (SA2-based for interaction with 5GC, and microservice-based for interaction among EDGEAPP entities incl. EEC)

SEAL is offering a common services platform to ease the vertical integration with 5GS, to provide support capabilities to assist verticals (automotive, smart factory, drones, mIOT) when interacting with one or more 4G/5G networks (PLMNs, NPNs); and to allow the dynamic translation of the vertical application requirements to network requirements and vice versa.

In microservices architecture (MSA), applications and services often require related functionality, such as monitoring, logging, configuration, and networking services. These peripheral tasks can be implemented as separate components or services. If the application is decomposed into services, then each service can be built using different languages and technologies. While this gives more flexibility, it means that each component has its own dependencies and requires language-specific libraries to access the underlying platform, and any resources shared with the parent application. In addition, deploying these features as separate services can add latency to the application.

In MSA, the role of the sidecar is to augment and improve the application container, often without the application container’s knowledge. In its simplest form, a sidecar container can be used to add functionality to a container that might otherwise be difficult to improve. A key benefit of sidecars is that it allows primary application to use a heterogeneous set of languages and frameworks. A component located in a sidecar service can be consumed by applications written in different languages using different frameworks.

A microservice in the context of SEAL can be defined as a constituent of a SEAL or VAL capability service corresponding to a functional service within the edge/cloud platform or VAL UE (app or SEAL client). Such microservice granularity depends on the MNO/SEAL service provider and examples of such services are:

- vertical applications (gaming application e.g. login server, chat server, XR applications, v2x applications)

- SEAL services provided by SEAL servers (NRMS, LMS,..)

- UE app services (SEAL client services, AI/ML user side services etc)

- Network/OAM services (as provided by Core and OAM APIs)

Microservices can be grouped into clusters given the dependencies among capabilies. Example of microservice cluster for a vertical service can be the multiplayer gaming scenario, which requires the gaming app services maybe in different edge / clouds (e.g. login server, chat server,..) as well as the user applications, as well as the SEAL services (QoS monitoring and control, ADAES analytics services). All these microservices can form a cluster for the given gaming scenario, and we want to enable the service-based interactions using a service mesh. in this service mesh there can be two options, one for single domain (e.g. all services within SEAL) and one for cross-domain (e.g. service mesh spans across MNO/SEAL provider and vertical domain (as shown in Figure 5.x.1-1.

Figure 5.x.1-1 different types of service mesh deployments

In 3GPP there can be different interactions among different planes/domains (OAM, 5GS, SEAL,..), since the enablement layer is on top of 5G core and OAM, and there are interactions via APIs which can be service based. Such interactions may involve different MNOs, ECSPs, CSPs, verticals, ASPs/enterprises. To support inter-plane SBA for some cases including multiple service providers, there needs to be a secure, flexible, and consumer-friendly architecture. For this reason, the use of service mesh may be beneficial for such cross-domain interactions.

### 5.x.2	Identified gaps

Gap#5-x-2-1: It is essential to study how to support MSA in SEAL assuming also cross-domain/plane interactions.

Gap#5-x-2-2: It is essential to study how to configure and control/manage a microservice environment within enablement layer consisting of multiple domains for service mesh deployments (e.g. using sidecars).

## 8.2	Solutions for SEAL services adoption gaps

### 8.2.x	Adoption Solution#X: SEAL supporting MSA

#### 8.2.x.1	Solution description

This solution is to solve GAP# 5-x-2-1 and 5-x-2-2.

This solution describes the configuration and setup of the microservice environment and in particular the dimensioning of sidecars (or gateways/proxies) and the capabilities included per sidecar. Such configuration may also include the translation of the application requirements to CRUD operations (create, read, update, and delete) at the sidecar/proxy, e.g., for a sidecar on an OAM Service how to map the service requirements to standard HTTP method requests.

The solution introduces an inter-plane μService / proxy configuration (IPμSC) service which aims to establish a logical microservice domain based on sidecars/proxies and configure service mesh topology and signalling means (sidecars, gateways) to be implemented with applications (per cluster or per individual μService).

This solution consists of the following steps:

Figure 5.x.2-1 procedure for MSA support

1. The consumer (who can be the 3rd party customer or the edge/cloud provider) subscribes or requests from the IPμSC to setup a microservice environment for a particular use case (per application profile/service type, slice, or event e.g., stadium event,…)

2. After authorizing the subscription / request (IPμSC may provide a response/ack to the consumer), IPμSC configures the microservice environment for the particular use case (e.g., for service profile X). Such configuration takes as input the SLA etc., and the requirements for telco services in different domains, and determines to migrate the required microservice to the microservice environment. Also, the configuration of APIs can be part of this configuration.

3. Based on the configuration, the IPμSC determines the sidecars and/or proxies and/or gateways required for the customized setup. Sidecars / GWs can be deployed for the interactions among different domains.

4. IPμSC registers the microservices and the sidecars/GWs/proxies at the respective registry (e.g. CCF). There can be registration to more than one registry, e.g for enablement/ NEF services to CCF, for OAM services to MnS discovery service. Such registration will indicate the service parameters (API info), the service type/profile as well as whether sidecar is used. if sidecar is used, the registration includes the sidecar ID/address and info / capabilities.

5. The respective service registry approves/authorizes the requests and stores the registrations. Such approval can be provided by OAM / service operator’s domain.

6. The respective service registry sends a response/ack to IPμSC.

7. IPμSC sends the configuration parameters to the respective applications, and in particular the configuration of triggers and mapping to CRUD operations. This step may also provide information of the sidecar (identity, capabilities etc)

8. IPμSC notifies optionally the consumer about the sidecar/GW/proxy information to allow the consumer to directly interact/invoke API towards the sidecar (otherwise this can be done via requesting the service registry).

9. The application may interact with the sidecar/GW for activating/initiating it and when it is needed for the operation phase the sidecar translates the capabilities of the application to issue request towards MnS discovery Service producer or CCF.

* * * Next Change * * * *