3GPP TSG-SA WG6 Meeting#67	S6-252173

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252xyz )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.946 | CR | 0010 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAPIF functional models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.222 v18.7.0 specifies the support of the following functional models: - 6.2.0 Functional model description for CAPIF - 6.2.1 Functional model description to support 3rd party API providers - 6.2.2 Functional model description to support CAPIF interconnection - 6.2.3 Functional model description to support RNAA These functional models are currently covered in TR 23.946, in clause 4.2 and 4.4, but the functional model for CAPIF interconnection is missing, and the references to the clauses in TS 23.222 where the functional models are described are also missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Include in clause 4.2 the link with the CAPIF functional models specified in TS 23.222 and with clause 4.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definition of CAPIF functional models in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 4.2	Functional Architecture

The Common API framework (CAPIF) functional architecture is specified in clause 6 of 3GPP TS 23.222 [2] and consists of CAPIF core function, API Provider Domain Functions and API invoker. CAPIF architecture is based on the well-known Service Oriented Architecture (SOA) design paradigm, where a Service producer (i.e. API provider) is able to publish (1) the offered service APIs which can be discovered (2) by the Service consumers (e.g. API invokers) and further can invoke (3) the discovered service APIs as per the authorization.

Figure 4.2-1: CAPIF based on SOA

Figure 4.2-1 provides an illustration of CAPIF based on SOA with the following relationship:

1.	The functionalities related to Service Access Controller are supported by CAPIF core function.

2.	The Service producer is the API Provider Domain Functions.

3.	The Service consumer is the API Invoker.

Most CAPIF APIs are provided by a CAPIF core function entity and can be consumed by applications (API invokers) and application providers (CAPIF defines an API provider as three functions: API exposure, API publishing, and API management). CAPIF core function APIs enable onboarding, publishing, discovery, authentication, registration, authorization, logging, charging, monitoring, configuration, topology hiding, and other procedures. One CAPIF API (AEF security) is provided by the API exposure function.

The CAPIF functional architecture supports different functional models:

a)	The CAPIF functional architecture when the CCF and the API exposure, API publishing and API magement function of the API provider belong to the PLMN trust domain is described in clause 6.2.0 of 3GPP TS23.222 [2].

b)	The CAPIF functional architecture to support 3rd party API providers is described 6.2.1 of 3GPP TS23.222 [2]. Figure 4.2-2 below depicts this functional model:

Figure 4.2-2: Functional model for the CAPIF to support 3rd party API providers

(adapted from 3GPP TS 23.222 [2] Figure 6.2.1-1: Functional model for the CAPIF to support 3rd party API providers).

c)	The CAPIF functional architecture to support CAPIF interconnection (to support publishing and discovering of service APIs in different CAPIF providers) is described in clause 6.2.2 of 3GPP TS23.222 [2].

d)	The CAPIF functional architecture to support RNAA is described in clause 6.2.3 of 3GPP TS23.222 [2]. Clause 4.4 and clause 4.5 provide an overview of RNAA in this release of the report.

The CAPIF architecture includes multiple deployment models e.g. centralized vs. distributed, single vs. multiple API providers (see 3GPP TS23.222 [2] clause 7).

* * * END of Change * * * *