3GPP TSG-SA WG6 Meeting #67	S6-252193

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0047 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to spatial anchor information and handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some of the application specific information specified in the create procedure are not really application specific and some of them need be part of the spatial anchor common information which needs to be handled by SAn server (expiry, state of the anchor). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Expiry and state of the anchor is removed from the application specification information mentioned as part of NOTE in clause 8.3.1.2 Expiry and state of the anchor is added as part of the Spatial anchor profile. Procedures are updated to handle these information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information captured in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 7.3.2.1, 8.3.2.3.1, 8.3.4.2, 8.3.5.2.1, 8.3.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.1.2	Procedure

Figure 8.3.1.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.1.2-1: Spatial anchor creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor create request to the SAn server. The request includes a requestor identifier, security credentials, and list of spatial anchors to be created as described in clause 8.3.1.3.1. The request includes a position, allowed entities list defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). If the request includes spatial anchor specific information, that information remains transparent to the SAn server and can be stored with the spatial anchor. The request can indicate to create multiple spatial anchors.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a name, a URL, a media data, a type, a manufacturing company, a date, a ratings, and customer premise information (e.g. a residence, office or shop) etc. Such information may be stored in key-value pair and is transparent to the SAn server.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the spatial anchor(s), assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The SAn server sends a spatial anchor create response to the requestor. If the SAn server created spatial anchors, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and can include a reason for failure.

* * * Next Change * * * *

#### 7.3.2.1	Spatial anchor profile

Table 7.3.2.1-1 describes information elements for the spatial anchor profile stored in the SAn server.

Table 7.3.2.1-1: Spatial anchor profile

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| Spatial map position (NOTE) | O | Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information. |
| Spatial anchor specific information | O | A container to contain application specific information. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > VAL Service ID | M | The identifier of the application service. |
| Service area | O | The service area where the spatial anchor is accessible. |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| Expiry timestamp | O | Indicates the timestamp after which the spatial anchor has to be deleted or state change to in-active by the SAn server. |
| Spatial anchor state | O | Indicates the state of the spatial anchor (active or inactive). Spatial anchors in inactive state is not discoverable.Absence of this IE shall be considered as spatial anchor is in active state. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

##### 8.3.2.3.1	Spatial anchor update request

Table 8.3.2.3.1-1 describes information elements for the spatial anchor update request from the SAn client or VAL server to the SAn server.

Table 8.3.2.3.1-1: Spatial anchor update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| > VAL service information | O | Information about the application service associated with the spatial anchor. |
| >> Service ID | O | The identifier of the application service. |
| >> Service endpoint | O | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be updated. |
| > Spatial anchor ID | M | Identifier of the spatial anchor to be updated. |
| > Position information | O | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format). |
| Time period or duration | O | Specifies the time period or duration of how long the spatial anchor to be in active or in-active state. |
| Expiry handling indication | O | Indicates whether the spatial anchor has to be deleted or its state to be updated as inactive on expiry by the SAn server. |
| Spatial anchor state | O | Indicates the state of the spatial anchor (active or inactive). Spatial anchors in inactive state is not discoverable.Absence of this IE shall be considered as spatial anchor is in active state. |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| NOTE: If Position information is included, at least one of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.3.4.2	Procedure

Figure 8.3.4.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.4.2-1: Spatial anchor discovery

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor discovery request message to the SAn server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and can include spatial anchor discovery filters as described in clause 8.3.4.3.1. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request can additionally include a VAL client identifier.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies spatial anchor(s) that satisfies the spatial anchor discovery filters.

NOTE 1:	User preferences in the spatial anchor discovery filters can be compared with spatial anchor specific information.

For each matching spatial anchor, the SAn server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the allowed entities list. To determine the location of interest of the requestor, the SAn server may use location information provided in the request. If location information is not provided in the request, the SAn server may obtain location information by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [6] and 3GPP TS 23.502 [5] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4].

3.	The SAn server sends a spatial anchor discovery response to the requestor. If the SAn server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors and may includes the recommended list of spatial anchors. The SAn server may choose not to incude the spatial anchors which are in in-active state as part of the discovery response. Otherwise, the response includes an indication of failure and may include a reason for failure.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the SAn client, the SAn client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

* * * Next Change * * * *

##### 8.3.5.2.1	Subscribe

Figure 8.3.5.2.1-1 illustrates the spatial anchor subscription procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.5.2.1-1: Spatial anchor subscription

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor subscribe request to the SAn server. The request includes a requestor identifier, security credentials, a notification endpoint, and event information for which the requestor is subscribing. The request may include an expiration time.

a.	The requestor may include the "discovery" event type to indicate the SAn server to notify the requestor when the SAn server detects events related to the management of spatial anchors (e.g., creation, update, deletion, spatial anchor state changes from active to in-active or vice versa) according to discovery event filters and spatial anchor filters provided in the request. If the requestor is a SAn client, the request includes a VAL client identifier.

b.	The VAL server may include the "UE with spatial anchor interest" event type to indicate the SAn server to notify the VAL server when the SAn server detects UE(s) with interest in specific spatial anchors according to the spatial anchor of interest event filters and spatial anchor filters provided in the request.

c.	The requestor may include the "spatial anchor in range" event type to indicate the SAn server to notify the requestor when the SAn server detects that spatial anchors become in range or out of range of the subscriber according to range event filters provided in the request. Range event filters can include a range for detecting spatial anchors, an indication to enable persistent search, the pose of the subscriber. If the requestor is a SAn client, the spatial anchor subscribe request includes a VAL client identifier.

2.	Upon receiving the request from the requestor, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "discovery", the SAn server starts monitoring spatial anchors management events according to the discovery event filters and spatial anchor filters.

b.	If the subscribed event is for "UE with spatial anchor interest", the SAn server starts monitoring spatial anchors of interest according to the UE with spatial anchor interest event filters and spatial anchor filters. If the spatial anchors of interest have been discovered by a UE, the SAn server can obtain the UE location information from the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

c.	If the subscribed event is for "spatial anchor in range" and the requestor is a SAn client, the SAn server starts monitoring the corresponding UE location and direction corresponding to the pose information according to the range event filters and spatial anchor filters. The UE location information can be obtained by using capabilities of the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server monitors the distance between UEs and the spatial anchor of interest to determine whether the spatial anchors becomes in-range or out-of-range of the UE. If persistent search is enabled, the SAn server monitors location of the user (or UE) and pose of the user, along with a direction of movement, and discovers a list of spatial anchors which becomes in-range or out of range of the user device based on the location of the user device, the pose of the user and in the user's field of view.

3.	The SAn server sends a spatial anchor subscribe response to the requestor. If the SAn server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the SAn server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

##### 8.3.5.2.2	Notify

Figure 8.3.5.2.2-1 illustrates the spatial anchor notify operation between the SAn server and a SAn client or VAL server.

Pre-conditions:

1.	The SAn client or VAL server has subscribed for spatial anchor notifications with the SAn server;

Figure 8.3.5.2.2-1: Spatial anchor notification

1.	An event occurs at the SAn server that satisfies trigger conditions for notifying a subscriber (e.g., SAn client or VAL server) according to subscribed events and discovery filters provided for the events.

2.	The SAn server sends a spatial anchor notification to the requestor indicating the event. The notification includes a subscription identity and includes event-specific information. The SAn server may choose to include the spatial anchors in the notification only if their state is active.

a.	If the subscribed event is for "discovery", the notification includes the list of determined spatial anchors

b.	If the subscribed event is for "UE with spatial anchor interest", the notification includes information about UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

c.	If the subscribed event is for "spatial anchor in range", the notification includes the list of determined spatial anchors that become in range, out of range of the subscriber.