3GPP TSG-SA WG6 Meeting #67	S6-252194

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-251450)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0144 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SEALDD enabled S&F transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,[Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in TS 22.865, Annex A, the Store and Forward satellite operation mode “is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. (…) (…) under “S&F Satellite operation” mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (Step A and B in Figure A-1). In Step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In Step B, connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place. So, the satellite moves from being connected to the UE in step A to being connected to the ground network in step B”. SA6 has specified in TS 23.433 the support in SEALDD for data transmission for S&F Satellite operation. The intention is to adapt the DL data delivery to the S&F events received from the 3GPP Core Network. However, it was overlooked that the SEALDD connection management service(s) is based on the establishment of a connection between the SEALDD client (served by the service link) and the SEALDD server (served by feeder link). Note that establishing a SEALDD connection is incompatible with the Store & Forward operation mode: the UE becomes in idle mode in the CN when the UE hosting the SEALDD client loses connectivity with the Satellite (the service link becomes unavailable), and hence all the connectivity related information is cleaned up. In addition, TS 23.682, clauses 5.6.1.11 and 5.6.3.10, specify the network report of whether a UE is in S&F mode and provides guiding timing information about when to try to contact the UE. - 5.6.3.10, step 1a “the MME detects that the UE is registered in S&F mode, moving from S&F mode to not registered in S&F mode (see clause 4.13.9 of TS 23.401 [7]) or there is change in previously reported values of Estimated S&F DL Delivery Time and/or the feeder link availability period” - 5.6.3.10, step 3 “Step 3 of clause 5.6.3.1 is executed. Depending on operator configuration, the MME sends to the SCS/AS that a UE is registered in S&F Mode or moving from S&F mode to not registered in S&F mode, an Estimated S&F DL Delivery Time and the feeder link availability period.” In these descriptions the GW is not mentioned, so the corresponding text in step 4 needs to be removed. New Reason for the changes: As the satellite S&F events depends on exposure from SA2, to align with the SA2’s specification, the S&F events notification from 3GPP CN should be corrected to keep align with the definition/ terminology in SA2 TS 23.682. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since it has not been studied during Rel-19 how the establishment of a connection between the SEALDD client and the SEALDD server is affected when the satellite mode of operation is S&F, this CR removes the use of the SEALDD regular connection establishment procedure and introduces a NOTE to clarify how the SEALDD client and server connect in S&F mode is out the scope of the current release. New changes(all of new changes are marked in blue): - Add reference of TS 23.682; - Add Feeder link availability period in step 4; - Correct number of the step from 6 to 4 in NOTE4; - Correct DL estimated delivery time to estimated S&FDL delivery time in step 6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect SEALDD procedure when the UEs is in S&F Satellite operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.2.1, 10.2.2.1, 10.2.3.1, 10.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-251450 |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************/

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[13]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[14]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[15]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[16]	3GPP TS 23.303: "Proximity-based Services (ProSe); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[18]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[19]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[20]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[21]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[22]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[23]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[xx]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[yy]	3GPP TS 23.682: " Technical Specification Group Services and System Aspects; Architecture enhancements to facilitate communications with packet data networks and applications".

# /******************************* Second Change *****************************/

10.2.1	General

This clause provides the services to support the transmission for delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) based on SEALDD capabilities including adapt the DL data delivery according to the S&F events from the 3GPP core network.

NOTE:	In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the VAL UE and of the intermittent availability of the feeder (ground) link when the satellite can exchange data with the ground network (VAL server). To support Store and Forward Satellite operation, network functionality needs to be deployed on the satellite, as specified in 3GPP TS 23.401 [xx].

# /******************************* Third Change *****************************/

10.2.2.1	SEALDD enabled data transmission for delay-tolerant satellite services

Figure 10.2.2.1-1 illustrate the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit its data between the VAL client and VAL server.

Pre-condition:

-	The VAL server discovers and selects the SEALDD server by CAPIF functions.

Figure 10.2.2.1-1: SEALDD enabled data transmission for delay-tolerant satellite services

1.	The VAL server decides to use SEALDD service for delay-tolerant traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_DTwithS&FTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates the SEALDD-S data transmission connection information (e.g., address/port) of SEALDD server side to receive the application data packets from the VAL server to be delivered to the VAL UE. The SEALDD server responds with a Sdd_DTwithS&FTransmission response.

NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional, if the SEALDD server uses the downlink pull mode to fetch the application data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the application data to the address provided by VAL server.

3.	The SEALDD server and the SEALDD client enable S&F operation.

NOTE 2:	The procedure to enable the S&F mode for the SEALDD server and SEALDD client is out of the scope of this release.

4.	The SEALDD server subscribes the UE S&F status events from the 3GPP CN and is notified about the UE S&F status events information when the feeder link is available e.g., whether the UE  is registered in S&F Mode,  the Estimated S&F DL delivery time, and the Feeder link availability period as specified in 3GPP TS 23.682[yy]. The SEALDD server stores the received UE S&F status events information for further control the downlink application data transmission.

5.	The VAL server sends downlink application data to the SEALDD server which further delivers it to the VAL UE via the S&F satellite when the feeder link is available.

NOTE 3:	Based on implementation, the SEALDD server can monitor the availability of terrestrial network connectivity and hence enable connectivity before the feeder link is available.

NOTE 4:	The SEALDD server may receive the DLapplication data from the VAL server in step 5 before receiving the UE S&F status events notification from the 3GPP CN in step 4.

6.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the UE S&F status events information, when the UE is registered in S&F mode:

-	pending the DL data transmission when the Feeder Link is not available;

-	(re-)transmitting the pending DL data when the Feeder Link is available.

-	DL data volume to be transmitted is smaller than maximum S&F data storage quota per application per UE.

-	If estimated S&F DL delivery time is received from 3GPP CN, it may be used by the SEALDD server to estimate whether the transmitted DL application data is successfully delivered to the UE and its corresponding storage is available for new DL data. For each Feeder Link available period, the SEALDD server starts an estimated delivery timer based on the estimated S&F DL delivery time which is corresponding to the current feeder link. The estimated delivery timer may be greater than or equal to the estimated S&F DL delivery time.  In next Feeder Link available period, the SEALDD server checks the previous estimated delivery timer is expired or not before transmitting the DL data. Also, the maximum S&F data storage quota per application per UE is also considered.

a.	If timer expires, the SEAL server calculates the new DL data volume (e.g., increasing the volume by certain data size (e.g., size of data previously delivered to the SEALDD client)).

b.	If timer not expires, the SEALDD server also checks the available data volume within the maximum S&F data storage quota. The SEALDD server may send DL data limited within the available data volume. If no available data volume, the SEALDD server pends the DL data transmission until the timer expires.

NOTE 5:	The data forwarding based on priority policy is out of the scope of the current release.

7.	The SEALDD server sends the DL data to the VAL UE via the 3GPP network and the S&F network functionality deployed in the S&F satellite.

8.	Optionally, the SEALDD server may send Sdd_DTwithS&FTransmission notification to the VAL server to influence the DL application data transmission from VAL server if the VAL UE connection status changes or the DL data transmission from the VAL server needs to be adjusted.

# /******************************* Fourth Change *****************************/

10.2.3.1	SEALDD enabled DTwithS&F transmission request

Table 10.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the delay-tolerant with S&F application transmission service.

Table 10.2.3.1-1: SEALDD enabled DTwithS&F transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | M | Identifier of specific UE or VAL user |
| SEALDD-S Data transmission connection information | M | Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. |
| DL data delivery status subscription indication | O | Indicates the VAL server expected to receive the DL delivery status notification |

# /******************************* Fifth Change *****************************/

10.2.3.3	SEALDD DTwithS&F transmission notification

Table 10.2.3.3-1 describes the information flow from the SEALDD server to the VAL server to notify S&F delivery related events.

Table 10.2.3.3-1: SEALDD DL delivery notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event SEALDD client connection status for DL data delivery e.g., reachable, unreachable, sleeping. |
| DL data delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL data delivery |
| > Adjust DL data volume | O | Indicates adjusting DL data volume with suggested traffic volume |
| > Pending DL data delivery | O | Indicates pending DL data delivery due to e.g., feeder link is not available |
| > Resume DL data delivery | O | Indicates pending DL data delivery due to e.g., feeder link is available |
| Identity | M | Identifier of VAL UE or VAL user. |
| VAL service ID | O | Identity of the VAL service. |
| NOTE: Either Event ID IE or the DL data delivery instruction ID is present. |  |  |

# /******************************* End of Changes *****************************/