3GPP TSG-SA WG6 Meeting #67	S6-252199

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252xxx)

Source:	China Mobile

Title:	Pseudo-CR on solution of DC Application list search

Spec:	3GPP TR 23.700-57 v0.1.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This contribution is proposed to contribute the solution of KI#2 regarding DC Application list search.

This pCR proposes new Key Issue for TS23_700_57.

2. Reason for Change

Contribution with new Key Issue.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.1.0.

* * * First Change * * * *

6.X	Solution #X: DC Application list search

6.X.1	Description

This solution resolves Key Issue #2 "DC Application list search".

This solution uses the architecture of MMTel Enabler as specified 3GPP TS 23.392 [2].

6.X.2	Procedures

Figure 6.x.2-1 below illustrates the procedure of Getting call information from the eMMTel Enabler Server.

Pre-conditions:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [4].

Figure 6.x.2-1: MMTel Enabler Client in the UE search the DC application list

1.	The  MMTel Enabler Client sends a DC Application list search request to the eMMTel Enabler Server. The request includes information elements as specified in Table 6.x,2-1.

Editor’s note:	Whether the URL for downloading the DC Application list, as specified in 3GPP TS 26.114 [5], can be modified (e.g., by adding some parameters) is FFS, and coordination with SA4 is required.

Table 6.x.2-1: Information elements in DC Application list search request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| MMTel Enabler Client version (see NOTE) | M | he version of MMTel Enabler Client |
| Home operator | O | It indicates the home operator of the device and this shall be represented as <MCCMNC> combination. Whereby MCC and MNC shall be replaced by the respective values of the home network in decimal format and with a 2-digit MNC padded out to 3 digits by inserting a 0 at the beginning. |
| search criteria | O | The criteria included by the user in this request and used by the MMTel Enabler server to filter the DC Application list |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different DC Applications based on this IE. |  |  |

2.	The MMTel Enabler server verifies that the sender is authorized to search the DC Application list and filter the DC Application list based on search criteria and other information included in the DC Application list search request. The MMTel Enabler Server may interact with DCSF for searching the DC Application list.

NOTE:	In a implementation, the DCSF may support the DC application profile functionality of MMTel Enabler Server function, the DCSF may also interact with the MMTel Enabler Server to support this procedure, i.e. The MMTel Enabler Client sends a request to the DCSF, which then sends the request to the MMTel Enabler Server. The response in the opposite direction is also sent to the MMTel Enabler Client through the DCSF.

3.	The MMTel Enabler Server sends a DC Application list search response to the MMTel Enabler client in the UE. The response message includes information elements as specified in clause 8.3.2.3.4 in 3GPP TS 23.392 [2].

6.X.3	Impacts on Existing Nodes and Functionality

In this solution, new parameters are proposed to be included in the DC Application list search request, which may impact the URL for downloading the DC Application list. Whether the URL for downloading the DC Application list, as specified in 3GPP TS 26.114 [5], can be modified (e.g., by adding some parameters) is FFS, and coordination with SA4 is required.