3GPP TSG-SA WG6 Meeting #67	S6-25xxxx

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-251144)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0150 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support 5G MBS in MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon,China BroadNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA6#66, the CR0624 was agreed to support MBMS in MBMS gateway UE. However, 5MBS is used to deliver the one-to-many group communication. Some concepts and terms are changed. So a new clause of supporting 5MBS for MC gateway UE should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses to support the 5MBS for MC gateway UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5MBS is not supported by MC gateway UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {7.x} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

7.x	5MBS support for MC clients hosted on non-3GPP devices

The 5MBS support for MC clients hosted on non-3GPP devices are supported by the procedures and information flows as defined in clause 11.5.3 of 3GPP TS 23.280 [3] with following clarifications and changes:

NOTE:	In 5G MBS and MC over 5MBS, some terms are changed. The bullets below mainly capture changes in terminology.

-	the "MBMS" is the "5MBS", the "MBMS bearer" is the "MBS session".

-	the "TMGI(s)" IE should be "MBS session ID", and its corresponding description should be "identifier(s) of the MBS session(s)";

-	the message "MC GW MBMS bearer announcement" is renamed as "MC GW MBS session announcement” and shall take the IEs from table 7.3.2.1 without the "eMBMS bearer information" IE.

-	the message "MC GW MBMS listening status report" is renamed as "MC GW MBS listening status report” and shall take the IEs from table 7.3.2.10 - "MBS listening status report ".

-	the message "MC GW MapGroupToBearer request" is renamed as "MC GW MapGroupToSessionStream request" shall take the IEs from table 7.3.2.3 – MapGroupToSessionStream.

-	the message "MC GW MapGroupToBearer response" is renamed as "MC GW MapGroupToSessionStream response" shall reuse the IEs from table 11.5.3.2.4 in 3GPP TS 23.280 [3].

-	the message "MC GW UnMapGroupToBearer request" shall take the IEs from table 7.3.2.7 -UnMapGroupToSessionStream.

-	the message "MC GW UnMapGroupToBearer response" is renamed as "MC GW UnMapGroupToSessionStream response" shall reuse the IEs from table 11.5.3.2.8 in 3GPP TS 23.280 [3].

-	the message "MC GW MBMS bearer suspension indication" is renamed as "MC GW MBMS bearer suspension indication" shall reuse the IEs from table 11.5.3.2.6 in 3GPP TS 23.280 [3].

In 5G MBS, the multicast is newly introduced and it requires the MC service UE to join the multicast MBS session. When such multicast MBS session is used for MC gateway UE scenario, the MC gateway UE needs to join the multicast MBS session. Accordingly, MC gateway UE should provide a new message "MC GW UE session join report" message which shall take the IEs from table 7.3.2.2 - " UE session join notification" except the MC service ID IE.

The figure 7.x.5-1 illustrates the high-level procedure that a multicast MBS session is used in MC gateway UE scenario and the MBMS bearer announcement by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters to enable the use of the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 7.x.5-1-1: Handling of MBMS bearer announcement

1.	The MC service server establishes the multicast MBS session according to the procedures defined in 3GPP TS 23.247 [15].

2.	The MC service server provides service description information associated with the multicast MBS session to the MC client residing on non-3GPP devices via MC gateway UE. The multicast MBS session type is included.

3.	The MC client sends the MC GW MBS session announcement to the MC gateway UE containing the service description information received from the MC service server.

4.	The MC gateway UE stores the information. The MC gateway UE joins the multicast MBS session towards the 3GPP network.

5.	The MC gateway UE sends the MC GW UE session join notification to the MC client after the success of multicast MBS session join.

6.	The MC gateway UE monitors the MBS reception quality and sends the MC GW MBS listening status report.

7.	The MC client establishes the transport resources with the MC gateway UE to receive the MC service data from the MC gateway UE. The MC Gateway UE forwards the MC service data it received over the multicast MBS session from the MC service server to the MC client over this transport resources.

8.	The MC client sends the MBS listening status report to the MC service server.

9.	The MC client sends the MBS UE session join notification to the MC service server indicating it has joined the multicast MBS session.