3GPP TSG-SA WG6 Meeting #67	S6-252220

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0472 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ambient listening call pause and resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA1 introduce new requirements about ambient listening call being pause and resume as described in [R6.16.2.2.1-005]. Ambient listening now is a “sendonly/recvonly” call between the listened to user and the listening user. This CR is proposed to use the SDP negotiation mechanism to change the media stream to ‘inactive’ and the media will be paused. When the new call is terminated, the media stream will be changed back to “sendonly/recvonly” and the ambient call is resumed. Such mechaism is similar to the IMS call HOLD. Using this mechansim can make the switch of “pause” and “resume” operations quickly without releasing and restoring the signalling plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new pause and resume procedure of ambient listening call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient listening call is terminated by other incoming call and the ambient listening is stopped. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {10.14.3.x} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

#### 10.14.3.x	Ambient listening call "pause" and "resume"

Figure 10.14.3.x-1 illustrates the high-level procedure for ambient listening call being "paused" and "resumed" via the SDP re-negotiation. When a new call is detected at the "listened to" MCPTT client, the "listened to" MCPTT client sends the ambient listening call update message to change the SDP attribute to inactive, so the media is paused. Later when the new call is detected as terminated, the "listened to" MCPTT clients sends the ambient listening call update message to change the SDP attribute back to sendonly automatically without any interactions with the "listened to"user, so the media to the "listening" user is resumed.

Pre-conditions:

-	There is an ongoing ambient listening call between MCPTT client 1 and MCPTT client 2. The call is either a remotely initiated ambient listening call or a locally initiated ambient listening call.

-	MCPTT user 1 is the "listening" user at MCPTT client 1, and MCPTT user 2 is the "listened to" user at MCPTT client 2.

Figure 10.14.3.x-1: Ambient listening call pause and resume

1.	The MCPTT client 1 used by the "listened to" user detects that a new call is initiated or coming in. The new call may be controlled by the same MCPTT server as the ongoing ambient listening call or a different MCPTT server.

1a.	The MCPTT client 1 receives a new group call request or a private call request.

1b.	The MCPTT client 1 sends out a new group call request or a private call request.

2.	The MCPTT client sends the ambient listening call update request to the MCPTT server to change the SDP attribute to inactive.

3.	The MCPTT server sends the ambient listening call update request to the MCPTT client 2 with the inactive attribute.

Optionally, it may include a reason information indicating that a new call is occurred at the "listened to" user in step 2 and step 3.

4.	MCPTT client 2 sends ambient listening call update response to the MCPTT server with the answered inactive attribute.

5.	MCPTT server returns ambient listening call update response to the MCPTT client 1with the answered inactive attribute. Then the ambient listening call media is paused.

6.	The new group call or private call is established and the media plane is also established. The MCPTT client 1 starts to change media data in the new call.

7.	The MCPTT client 1 used by the "listened to" user detects the new call is terminated via the following ways:

7a.	The MCPTT client 1 receives a new group call release request or a private call release request.

7b.	The MCPTT client 1 sends out the group call release request or a private call release request.

8.	Based on the detected call termination event, the MCPTT client 1 sends the ambient listening call update request to the MCPTT server automatically without any interactions with the "listened to"user. The sendonly attribute is included. An implicit floor request may be included.

Optionally, it may include a reason information indicating that the "listened to" user is back and resumes the media transmission.

9.	The MCPTT server sends the ambient listening call update request to the MCPTT client 2 with the sendonly attribute.

10.	The MCPTT client 2 sends the ambient listening call update response with the recvonly attribute.

11.	The MCPTT server sends the ambient listening call update response with the recvonly attribute.

12.	The MCPTT server sends the floor granted to the MCPTT client 1.

13.	The MCPTT server sends the floor taken to the MCPTT client 2.

14.	The MCPTT server sends a floor taken to the MCPTT client 2. Optionally, it may include a reason information indicating that the new call is terminated at the "listened to" user and the "listened to" user resumes the media transmission.