3GPP TSG-SA WG6 Meeting #67	S6-252227

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0147 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and alignments of clause 10.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) In clause 10.2.2, some description about S&F exposed event and information are not aligned with latest SA2 output in TS 23.682. It may cause disconnection and confusion between SA6 TS and SA2 TS. 2) The “S&F data storage quota” may not always be available at the SEALDD server. But currently all the DL data transmission is controlled by this parameter. So it is proposed to update the description to clarify it. 3) Now the “S&F data storage quota” is restricted to per application per UE. Such granularity is too fine to control at the satellite. So it is propose to change it to cover coarse granularities for better control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction the descriptions about SA2 exposed event and information. Update the description about S&F data storage quota |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment event and information between the SA2 TS and SA6 TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2.1, 10.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

10.2.2.1	SEALDD enabled data transmission for delay-tolerant satellite services

Figure 10.2.2.1-1 illustrates the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit its data between the VAL client and VAL server.

Pre-condition:

-	The VAL server discovers and select the SEALDD server by CAPIF functions.

Figure 10.2.2.1-1: SEALDD enabled data transmission for delay-tolerant satellite services

1.	The VAL server decides to use SEALDD service for delay-tolerant traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_DTwithS&FTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates the SEALDD-S data transmission connection information (e.g., address/port) of SEALDD server side to receive the application data packets from the VAL server to be delivered to the VAL UE. The SEALDD server responds with a Sdd_DTwithS&FTransmission response.

NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional, if the SEALDD server uses the downlink pull mode to fetch the application data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the application data to the address provided by VAL server.

3.	Same as step 3-9 of clause 9.2.2.2.

4.	The SEALDD server subscribes and is notified about the store and forward satellite operation information from the 3GPP CN as specified in clause 5.6.3.10 of 3GPP TS 23.682 [24] e.g., whether UE is registered in S&F Mode, the estimated S&F DL delivery time indicating the estimated/expected time required to deliver the data to UE from the time data has been received in the 3GPP network (i.e., SCEF or PGW), feeder link availability period. The SEALDD server stores or the received S&F satellite operation information for further control the downlink application data transmission.

5.	The VAL server sends downlink application data to the SEALDD server which further delivers it to the VAL UE.

NOTE 2:	The SEALDD server may receive the DL application data from the VAL server in step 10 before S&F satellite operation information from the 3GPP CN in step 9.

6.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the S&F satellite operation information, when the UE is in S&F mode:

-	pending the DL data transmission when the Feeder Link is not available;

-	(re-)transmitting the pending DL data when the Feeder Link is available.

-	DL data volume to be transmitted is smaller than maximum S&F data storage quota.

-	If estimated S&F DL delivery time is received from 3GPP CN, it may be used by the SEALDD server to estimate whether the transmitted DL application data is successfully delivered to the UE and its corresponding storage is available for new DL data. For each Feeder Link available period, the SEALDD server starts an estimated delivery timer based on the estimated S&F DL delivery time which is corresponding to the current feeder link. The estimated delivery timer may be greater than or equal to the  estimated S&F DL delivery time. When in next Feeder Link available period, the SEALDD server checks the previous estimated delivery timer is expired or not before transmitting the DL data. Also, the maximum S&F data storage quota is also considered.

a.	If the timer expires, the SEAL server sends DL data. If the maximum S&F data storage quota is configured, the SEALDD server may further calculate the volume (e.g., increasing the volume by certain data size (e.g., size of data previously delivered to the SEALDD client)) of new DL data to be sent.

b.	If the timer not expire, the SEALDD server may not send the DL data and wait until the timer expires. If the maximum S&F data storage quota is configured, the SEALDD server may further also checks the available data volume within the maximum S&F data storage quota. The SEALDD server may send DL data limited within the available data volume. If no available data volume, the SEALDD server pends the DL data transmission until the timer expires.

NOTE 3:	The SEALDD server can use the estimated S&F DL delivery time as the estimated/expected time required to deliver the data to UE from the time data has been received at the SEALDD server, as the data delivery latency from the SEALDD server to the 3GPP network (i.e., the SCEF or PGW) is quite small and can be ignored compared with the estimated S&F DL delivery time from network to the UE.

NOTE 4:	The maximum S&F data storage quota can be per UE, per application or per SEALDD server or any combination of them.

Editor's note:	whether the SEALDD server will consider the forwarding with higher priority policy for the DL data delivery is FFS.

7.	The SEALDD server sends the DL data to the VAL UE via the 3GPP network as indicated in step 11.

8.	Optionally, the SEALDD server may send Sdd_DTwithS&FTransmission notification to the VAL server to influence the DL application data transmission from VAL server if the VAL UE connection status changes or the DL data transmission from the VAL server needs to be adjusted.

10.2.2.2	Client initiated data transmission for delay-tolerant satellite services

Figure 10.2.2.2-1 illustrates the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit data initiated by the VAL client to VAL server.

Pre-condition:

-	The VAL client is aware of the availability of delay-tolerant satellite services to transmit data.

Figure 10.2.2.2-1: Client initiated data transmission for delay-tolerant satellite services

1.	The VAL client decides to use SEALDD service for delay-tolerant traffic transfer.

2.	Same as steps 2-4 of clause 9.2.2.7.

3.	The SEALDD client provides the acknowledgment for the request made in step 1.

4.	The SEALDD client sends the UL data to the SEALDD server via the 3GPP network.

5.	The SEALDD server subscribes the recipient &F satellite operation information from the 3GPP CN as specified in clause 5.6.3.10 of 3GPP TS 23.682 [24] and is notified about the recipient UE &F satellite operation information e.g., whether UE is registered in S&F Mode, the estimated S&F DL delivery time. The SEALDD server stores the received recipient UE S&F status events information for further control the downlink application data transmission.

Editor's note:	How SEALDD server knows the recipient UE information is FFS.

6.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the UE S&F status events information, when the recipient UE is in S&F mode and receives the estimated S&F DL delivery time is received from 3GPP CN. If the recipient UE is not available or not registered to S&F mode, then the SEALDD server handles the DL data as per the service policy.

7.	The SEALDD server sends the Sdd_S&FTransmission_notification to the SEALDD client and VAL client including the estimated S&F DL delivery time of the recipient UE.

NOTE 1:	VAL client can use the estimated S&F DL delivery time of the recipient UE to control the UL application data delivery.

8.	The SEALDD server sends the DL data to the VAL server.

NOTE 2:	Details of step 1, step 3 and step 7 are out of scope of the current specification.