3GPP TSG-SA WG6 Meeting #66	S6-252267

Goteborg, Sweden, 7th – 11th April 2025	(revision of 251186)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0143 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to server initiated SEALD data transmission connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During SA6#63, a new service operation was introduced in CR 0102 to differentiate the regular transmission connection request sent from the server from the regualr transmission connection request sent from the client. However, that may cause may cause potential backward compatibility issue in stage-3 level as from day one, i.e., Rel-18 (as stated by requirement in TS 23.433) the client and the server can use the very same message (“SEALDD regular transmission connection establishment request” and “SEALDD regular transmission connection establishment response”). So this CR is proposed to void the new claues 9.2.3.17 and 9.2.3.18 and add new NOTEs to the clause 9.2.3.3 and 9.2.3.4 to clarify the applicable direction of those IEs. To aligned, the clause 9.3.3.3 and 9.3.3.4 are also modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Void 9.2.3.17, 9.2.3.18 - Add NOTEs to clause 9.2.3.3 and 9.2.3.4 - Add NOTEs to clause 9.3.3.3 and 9.3.3.4 - The procedure and figure are updated accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leading to implementation mistakes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3, 9.2.3.4, 9.2.3.17, 9.2.3.18, 9.2.4.3, 9.3.3.3, 9.3.3.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************/

9.2.2.3	SEALDD enabled regular data transmission connection establishment based on policy

The SEALDD servers has Data Delivery (DD) policy being provisioned. Before the application communication between VAL client and VAL server starts, the DD policy is enforced by the SEALDD server to establish the SEALDD connection.

Pre-conditions:

1.	The SEALDD server has DD policies available.

Figure 9.2.2.3-1: Policy enforced by SEALDD server for connectivity

1.	The VAL server subscribes to SEALDD event exposure for connection status using the procedure defined in clause 9.2.2.6.

NOTE 1:	The VAL server can update/delete an existing subscription at the SEALDD Server when required.

2.	When the time for data transmission is about to start, the SEALDD server enforces the policy to trigger regular data transmission connection establishment. If spatial condition for UE is provided, the SEALDD server also ensures the UE’s location requirement is satisfied when establishing regular data transmission connection (e.g. by using NEF service for monitoring UE location or SEAL location service for UE entering area of interest).

3.	If there is a special routing requirement for SEALDD user plane traffic (e.g. running on a specific slice and DNN), the SEALDD server interacts with 3GPP CN to provision service specific parameters with NEF as described in 3GPP TS 23.502 [6], clause 4.15.6.10 and clause 4.15.6.7.

If there are QoS requirements in the DD policy, the SEALDD server also applies QoS to ensure the quality for SEALDD traffic by utilizing NEF/PCF/NRM/EES service for QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4].

If the DD policy specifies failure detection report, the SEALDD server may subscribe to CN analytics (e.g. DN performance analytics) from NEF/NWDAF and further notify data delivery status of application traffic to VAL client (via SEALDD client) and VAL server based on analytics result.

4.	The SEALDD server allocates an IP address and port for sending and receiving packet over SEALDD-S reference point, then SEALDD server sends SEALDD connection establishment notification (i.e. SEALDD connection status notification with establishment event, as described in Table 9.2.3.9-1) to the VAL server with VAL service ID, the address.

5-6.	The SEALDD server allocates a SEALDD-S connection information for sending and receiving packet over SEALDD-UU reference point, then SEALDD server sends SEALDD regular data transmission connection establishment request to the SEALDD client with SEALDD-UU flow ID, VAL service ID, the address. The request is responded by the SEALDD client. UE IP address may be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane.

NOTE 2:	Step 4 and step 5 can be done in parallel.

NOTE 3:	SEALDD server can know the UE address (e.g. via SEALDD connection establishment request initiated by SEALDD client, but rejected by SEALDD server due to policy condition not met) before step 5, if not, step 5 can be sent via application triggering.

7.	The SEALDD client further notifies the VAL client about the SEALDD connection being established.

Upon receiving application traffic from VAL client (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies application traffic based on the VAL service ID and further sends the application traffic to VAL server. The downlink application traffic sent from VAL server to VAL client is processed similarly.

# /******************************* Next Change *****************************/

9.2.3.3	SEALDD regular transmission connection establishment request

Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the regular SEALDD connection establishment.

Table 9.2.3.3-1: SEALDD regular transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of either: - the SEALDD client side (for client side initiated request) or, - the SEALDD server side (for server side initiated request) used to establish SEALDD connection. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O (NOTE 6) | Identifies the DD communication lifetime, applicable for SEALDD server side initiated request. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2, NOTE 7) | Indicates BAT and periodicity adaptation capability of the SEALDD client side (for client side initiated request) per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 2, NOTE 4, NOTE 7) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for client side initiated request). It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| L4S feedback capability | O (See NOTE 3, NOTE 7) | Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request |
| VAL UE client access capability | O (See NOTE 7) | Indicates the access technologies supported by VAL UE client. E.g.,3GPP, Non-3GPP(WLAN) |
| XR Application device capability information | O (See NOTE 5, NOTE 7) | Indicates XR Application device capability information (e.g., encoding capabilities) |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 5: This IE is used for the SEALDD enabled XR traffic data delivery. NOTE 6: This IE is only present in the message from SEALDD server to SEALDD client direction. NOTE 7: This IE is only present in the message from SEALDD client to SEALDD server direction. |  |  |

# /*******************************Next Change *****************************/

9.2.3.4	SEALDD regular transmission connection establishment response

Table 9.2.3.4-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding to the regular SEALDD connection establishment.

Table 9.2.3.4-1: SEALDD regular transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of either: - the SEALDD server side (for client side initiated request) or, - the SEALDD client side (for server side initiated request) used to establish SEALDD connection. |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD client or SEALDD server to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| Capability for BAT and periodicity adaptation | O (See NOTE 3, NOTE 5) | Indicates BAT and periodicity adaptation capability of the SEALDD client side (for server side initiated request) per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 3, NOTE 4, NOTE 5) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for server side initiated request). It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure NOTE 3: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 5: These IEs are only present in the message from SEALDD client to SEALDD server direction. |  |  |

# /******************************* Next Change *****************************/

9.2.3.17	void

9.2.3.18	void

# /******************************* Next Change *****************************/

9.2.4.3	Sdd_RegularTransmissionConnection_Establish operation

API operation name: Sdd_RegularTransmissionConnection_Establish

Description: The consumer requests for one time for SEALDD enabled regular data connection establishment.

Inputs: See clause 9.2.3.3.

Outputs: See clause 9.2.3.4.

See clause 9.2.2.1, 9.2.2.2 and 9.2.2.3 for details of usage of this operation.

# /******************************* Next Change *****************************/

9.3.3.3	SEALDD URLLC transmission connection establishment request

Table 9.3.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the URLLC transmission connection establishment.

Table 9.3.3.3-1: SEALDD URLLC transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | O | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flow IDs | M (See NOTE 1) | Identity of the SEALDD-UU flows. |
| VAL server ID | O (See NOTE 1) | Identity of the VAL server. |
| VAL service ID | O (See NOTE 1) | Identity of the VAL service. |
| SEALDD traffic descriptors | O | A pair of SEALDD traffic descriptors (e.g. address, port, transport layer protocol) of either: - the SEALDD client side (for client side initiated request) or, - the SEALDD server side (for server side initiated request) used to establish redundant SEALDD connection. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2, NOTE 4) | Indicates BAT and periodicity adaptation capability of the SEALDD client (for client side initiated request). For URLLC transmission, it includes capability for BAT and periodicity adaptation per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 2, NOTE 3, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for client side initiated request). It includes BAT, BAT window, periodicity, and periodicity range. For URLLC transmission, it includes assistance information per SEALDD traffic descriptor. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped from the VAL service ID. NOTE 2: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 3: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 4: This IE is only present in the message from SEALDD client to SEALDD server direction. |  |  |

9.3.3.4	SEALDD URLLC transmission connection establishment response

Table 9.3.3.4-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding to the URLLC transmission connection establishment.

Table 9.3.3.4-1: SEALDD URLLC transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptors | O | A pair of SEALDD traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD server side (for client side initiated request) or the SEALDD client side (for server side initiated request) used to establish redundant SEALDD connection. |
| Cause | O (See NOTE 1) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2, NOTE 4) | Indicates BAT and periodicity adaptation capability of SEALDD client side (for server initiated request). For URLLC transmission, it includes capability for BAT and periodicity adaptation per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 2, NOTE 3, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for server side initiated request). It includes BAT, BAT window, periodicity, and periodicity range. For URLLC transmission, it includes assistance information per SEALDD traffic descriptor. |
| NOTE 1: The IE is only present if the Result is failure. NOTE 2: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 3: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 4: This IE is only present in the message from SEALDD client to SEALDD server direction. |  |  |

# /******************************* End of Changes *****************************/