3GPP TSG-SA WG6 Meeting #67	S6-252285

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252099)

Source:	ZTE Corporation

Title:	Pseudo-CR on Enhanced technical solution#1

Spec:	3GPP TR 23.700-35 v0.4.0

Agenda item:	9.3

Document for:	Approval

Contact:	Yang Li (li.yang1226@zte.com.cn)

1. Introduction

This paper provides some enhancements for technical solution#1.

2. Reason for Change

This solution proposes some enhancements for technical solution#1.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35.

* * * First Change * * * *

8.1.1.1.2	Procedure

8.1.1.1.2.1	SEALDD enabled best effort signalling transmission connection establishment procedure

Figure 8.3.1.1.2.1-1 illustrates the procedure for best effort signalling transmission connection establishment. In this clause, considering VAL server’s requirement on signalling transmission is not strict, then the VAL server can invoke best effort signalling transmission connection establishment API to enable the connection establishment.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 8.3.1.1.2.1-1: SEALDD best effort signalling transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling packets from SEALDD server. The VAL server sends SEALDD enabled best effort signalling transmission connection establishment request to the SEALDD server. The SEALDD enabled best effort signalling transmission connection establishment request includes the VAL server ID, VAL service ID to identify the VAL application traffic, the SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD enabled best effort signalling transmission connection establishment response.

3-4.	Same as step 3-4 of clause 9.2.2.1 in 3GPP TS 23.433 [2].

5.	The SEALDD client allocates SEALDD-UU flow ID(s) mapping to application signalling traffic for application signalling transmission. The SEALDD client sends SEALDD signalling transmission connection establishment initiated by SEALDD client request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), VAL server ID, VAL service ID and the SEALDD traffic descriptor of the SEALDD client side (the SEALDD-S connection information of the SEALDD client for receiving the downlink SEALDD-UU signalling traffic). The request message also contains the selected VAL server endpoint information. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434 clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the signalling connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the signalling connection establishment, otherwise SEALDD server returns failed result e.g. performs connection reject.

6-8.	Same as step 6-8 of clause 9.2.2.1 in 3GPP TS 23.433 [2].

8.1.1.1.3	Signalling flow

8.1.1.1.3.1	SEALDD enabled best effort signalling transmission connection establishment request

Table 8.3.1.1.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the best effort application signalling traffic transmission service.

Table 8.3.1.1.3.1-1: SEALDD enabled best effort signalling transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL) of the VAL server to receive the signalling traffic from the SEALDD server |
| Requested 5GS Delay | O | The Requested 5GS Delay provided by the VAL server |

8.1.1.1.3.2	SEALDD enabled best effort signalling transmission connection establishment response

Table 8.3.1.1.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the best effort application signalling traffic transmission.

Table 8.1.1.1.3.2-1: SEALDD enabled best effort signalling transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O (See NOTE1) | Address information (e.g., IP address and/or port, URL) of the SEALDD server to receive the signalling traffic from the VAL server |
| Cause | O (See NOTE2) | Indicates the reason for the failure, e.g., SEALDD policy mismatch. |
| NOTE1: Information element shall be present when the Result is successful. NOTE2: Information element shall be present when the Result is failure. |  |  |

8.1.1.1.3.3	SEALDD signalling transmission connection establishment initiated by SEALDD client request

Table 8.3.1.1.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the SEALDD signalling transmission connection establishment.

Table 8.1.1.1.3.3-1: SEALDD signalling transmission connection establishment initiated by SEALDD client request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client-side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g., address, port, URL, transport layer protocol) of the SEALDD client side (for client-side initiated request) |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server-side initiated request. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

8.1.1.1.3.4	SEALDD signalling transmission connection establishment initiated by SEALDD client response

Table 8.3.1.1.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the SEALDD signalling transmission connection establishment initiated by SEALDD client.

Table 8.1.1.1.3.4-1: SEALDD signalling transmission connection establishment initiated by SEALDD client response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O (See NOTE1) | SEALDD traffic descriptor (e.g., address, port, URL, transport layer protocol) of the SEALDD server side |
| Pending timer | O (See NOTE 2) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Cause | O (See NOTE 3) | Indicates the reason for the failure |
| NOTE 1 Information element shall be present when the Result is successful. NOTE 2: The Pending Timer is used for the SEALDD enabled bandwidth control for different VAL users, applicable for client-side initiated request. NOTE 3: Information element shall be present when the Result is failure. |  |  |

* * * End of Changes * * * *