3GPP TSG-SA WG6 Meeting #67	S6-252309

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-251145, 252221)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0666 | rev | 3 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change MC GW MBMS bearer quality report message name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon,China BroadNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The procedure 11.5.3.3.4 is designed to report the MBMS listening status per TMGI from the MC gateway UE to the MC clients in the non-3GPP device who further report the MBMS listening status to the MC service server. Currently, the table 11.5.3.2.5-1 title “MC GW MapGroupToBearer response” is not aligned with its clause title “MC GW MBMS bearer quality report”. The “MC GW MBMS bearer quality report” is derived from the MBMS listening status report message (from the MC service client to the MC service server) by removing the MC service ID. So it is better to use “MC GW MBMS listening status” as explained as above rather than inventing a totally new message name. - In table 11.5.3.2.5-1 and the table 10.7.2.2, the IE description is not fully aligned with the correspondig IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the clause 11.5.3.2.5 - Update the procedure accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment table title cause issues to the downstream working group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.3.2.5, 11.5.3.3.4, 10.7.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In Rev 1, the message title is changed, and the procedure is updated accordingly. |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

11.5.3.2.5	MC GW MBMS listening status report

Table 11.5.3.2.5-1 describes the information flow from the MC gateway UE to the MC client which resides on a non-3GPP device for the MC GW MBMS listening status report.

Table 11.5.3.2.5-1: MC GW MBMS listening status report

| Information element | Status | Description |
| --- | --- | --- |
| TMGI(s) | M | TMGI(s) information. |
| MBMS listening status(s) | M | The MBMS listening status per TMGI. |
| MBMS reception quality level(s) | O | The reception quality level per TMGI |

# /**************************** 2nd changes **********************/

11.5.3.3.4	Procedure for reporting MBMS listening status

The MC gateway UE listening on the MBMS bearer has to report the MBMS listening status and the optional MBMS bearer quality to the MC clients so that MC clients can report the same to the MC service server. MC gateway UE monitors an MBMS bearer to receive MC service media. Based on the MBMS listening status and the received quality (e.g. radio level quality) the MC gateway UE needs to inform the MC clients which requested the MC gateway UE to listen on MBMS bearer, whether it is able to receive the MC service media on the MBMS bearer with sufficient quality or not the MC Clients can inform the MC service server accordingly.

Figure 11.5.3.3.4-1 illustrates the procedure for MC clients residing on non-3GPP devices reporting the MC service server about the MBMS bearer listening status.

Pre-conditions:

1.	There is an MBMS bearer activated and the MBMS bearer information is announced to the MC gateway UE.

2.	The MC gateway UE is located in the MBMS broadcasting area

3.	The MC gateway UE monitors SIB-13 (or SIB-20) and (SC-)MCCH to receive the modulation and coding scheme.

4.	The MC gateway UE monitors the cell specific reference signal and when MBSFN transmission is used, the MBSFN specific reference signals.

Figure 11.5.3.3.4-1: Reporting MBMS listening status report

1.	The MC gateway UE follows the Step 1 of the procedure as described in clause 10.7.3.6.2 for the MC service UE. The corresponding MBMS bearer listening status and quality information are forwarded to all the MC clients which have asked the MC gateway UE to listen on the particular MBMS bearer.

2.	If the MBMS bearer quality reaches a certain threshold, the MC gateway UE sends an MC GW MBMS listening status report to the MC client. The threshold is used to define the MBMS listening status, which indicates if the MBMS bearer quality has been acceptable or not to receive a specific MC service media. If the MBMS bearer quality is mapped to a different MBMS reception quality level, the MC gateway UE may send an MBMS listening status report including the MBMS reception quality level to the MC Client.

NOTE:	The threshold used to indicate MBMS bearer quality depends on service type (i.e. MCPTT, MCVideo or MCData) and the metrics used. The metrics used and the associated thresholds are out of scope of this specification.

3.	The MC Client sends the MBMS listening status report to the MC Service server via MC gateway UE containing the information received in the MC GW MBMS listening status report.

4.	The MC service server may send an additional proposal for measurements e.g. information about neighbouring MBMS bearers. This message may be an MBMS bearer announcement message.

# /**************************** Next changes **********************/

10.7.2.2	MBMS listening status report

Table 10.7.2.2-1 describes the information flow for the MBMS listening status report from MC service client to MC service server. The information is used for the decision on the switching from MBMS bearer to unicast bearer or vice versa.

Table 10.7.2.2-1: MBMS listening status report

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user who wants to report the MBMS listening status. |
| TMGI(s) | M | TMGI(s) information. |
| MBMS listening status(s) | M | The MBMS listening status per TMGI. |
| MBMS reception quality level(s) | O | The reception quality level per TMGI (see NOTE) |
| NOTE: The set of quality levels helps service continuity in MBMS scenarios. A reception quality level may help to make an efficient switching decision to another bearer. How these levels are used is implementation specific. |  |  |

# /**************************** End of changes **********************/