3GPP TSG-SA6 Meeting #67	S6-252322

# Fukuoka, Japan, 19th May 2025 - 23th May 2025 	                   (revision of S6-252025)

Source:	Airbus

Title:	Solution: Recording MCVideo one to one pull and push

Spec:	3GPP TR 23.700-39 V0.1.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This contribution proposes a solution for recording of MCVideo one to one pull and push services. It refers to key issue x.

2. Reason for Change

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.1.0.

* * * First Change * * * *

### 6.x	Solution x – Recording of MCVideo one to one pull and push

As described in Key issue x and Scenario 4.1.x, MC Video one to one pull and push procedures are MC Video private call with specific parameter values. I.e. the existing functionality in the Rel-19 TSs can be utilized for this service.

The required changes/amendments for the Rel-19 TSs are clarifications. This solution proposes the following changes to TS 23.280 clause 7.5.2.34.

---------------

#### 7.5.2.34	Reference point REC-4 (between recording server and MC service servers)

The REC-4 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the communication sessions of target users and target groups.

NOTE:	In this version of this specification the communication sessions that may be recorded are MCPTT and MCVideo group and private calls.

The REC-4 reference point shall use the SIP-2 reference point for transport and routing of signalling and communication sessions related metadata. If an MC service server and a recording server are served by different SIP cores, then the REC-4 reference point shall also use the SIP-3 reference point for transport and routing of signalling and communication sessions related metadata.

MC service server and recording server shall use Session Recording Protocol as specified in IETF RFC-7866 [40] for establishing and controlling recording sessions between MC service servers and recording server for MCPTT and MCVideo private and group communications. IETF RFC-7866 [40] specifies the use of SIP, Session Description Protocol (SDP), and RTP to establish a recording session.

------------------

* * * Next Change * * * *

# Annex B: Mapping of stage 1 requirements to solutions

| Requirement | Scenarios | Key Issues | Solutions | NOTE |
| --- | --- | --- | --- | --- |
| [R-6.15.4-001] [R-6.15.4-002] [R-6.15.4-003] [R-6.15.4-004] | 4.1.x | x | x |  |

* * * End of changes * * * *