3GPP TSG-SA WG6 Meeting#67	S6-252365

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252163 )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0308 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In CAPIF interconnection scenarios, when CCF-B and CCF-A belong to trust domains of different CAPIF providers, the two CAPIF providers have business agreement for service API sharing. As described in clause 8.25.3.7, if the CCF-B determines that the service API access authorization cannot be done by itself alone, then the CCF-B sends an interconnection obtain API authorization from the CCF-A. For the obtain service API authorization procedure, it is needed that the API invoker Id is unique across all the API invokers of a CAPIF domain. In addition, in CAPIF interconnection scenarios, API invoker Id uniqueness across CAPIF provider domains is required to ensure the correctness of the procedures where API invoker Id is involved in both, CCF-A and CCF-B and interconnection procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define CAPIF requirements that require API invoker id uniqueness across all API invokers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements about API invoker identification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.13.2	Requirements

[AR-4.13.2-a] The CAPIF shall support uniform addressing (e.g. identity) for communication within the same trust domain or from the 3rd party trust domain.

[AR-4.13.2-b] The CAPIF shall support identities for uniquely identifying each service API.

[AR-4.13.2-c] The CAPIF shall support identifiers for uniquely identifying each API invoker.

* * * END of Change * * * *