3GPP TSG-SA WG6 Meeting #67	S6-252405

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252222)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on new solution of broadcast data delivery service to KI#1

Spec:	3GPP TR 23.700-52 0.1.0

Agenda item:	9.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed a new solution to KI#1.

2. Reason for Change

In this solution, the SEALDD server will handle the broadcast delivery service request and interacts with the 5GS and eMBMS accordingly. And the SEALDD server act as the media ingress node towards the 3GPP network broadcast user plane functions (e.g., UPF(s), BM-SC).

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.1.0.

* * * First Change * * * *

6	Solutions

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x | X |  |  |  |
| Sol #2 |  |  |  |  |

6.x	Solution #x: SEALDD enabled broadcast data delivery

6.x.1	Solution description

Editor's Note:	This clause describes the solution. Each solution should clearly describe which of the key issues it covers and how.

6.x.1.1	General

In this solution, the SEALDD server provides new data delivery services to deliver the same content (e.g., application signalling or application user plane data) from a single source entity to all the recipients in a certain area. The SEALDD server provides:

- SEALDD broadcast data delivery service. It is used for the application that provides the same content to all the recipients in a certain area. It requires the support of the broadcast 5MBS session and/or eMBMS at the network and UE.

In general, the broadcast delivery services require the VAL server to provide its service requirements e.g., service area, start and end time of the delivery, QoS requirements. The SEALDD server provides its own address as the ingress address for the VAL server to send the DL data. During the data delivery service update with service area update, more MBS session may be created and additional MB-UPF addresses are returned to the SEALDD server. However, SEALDD server hides the multiple network user plane addresses towards the VAL server and the ingress address keep the same. For broadcast data delivery service, the SEALDD server determines to create both eMBMS bearer and MBS session based on the service area.

In this solution, the SEALDD server acts as the NRM server to interact with the 3GPP network to manage the eMBMS bearer and 5MBS session. Also the SEALDD client acts as the NRM client to interact with SEALDD server acting as the role of NRM server to handle the session/service announcement, listening status report. The SEALDD-UU utilizes the multicast/broadcast functions of NRM-UU to support this broadcast data delivery service.

Figure 6.x.1.1 -1 shows the high-level functional model for this service.

Figure 6.x.1.1-1: Broadcast delivery service functional model

6.x.1.2	Broadcast data delivery service

6.x.1.2.1	Broadcast data delivery service create

Figure 6.x.1.2.1-1 illustrates the high-level procedure of creating a broadcast data delivery service for the VAL server DL data delivery.

Figure 6.x.1.2.1-1: Broadcast delivery service create

1.	The VAL server invokes the Sdd_BroadcastDelivery create service. The service requirements e.g., service area, start and end time of the delivery, QoS requirements are included.

2.	The SEALDD server determines that the eMBMS bearer and/or the MBS session are needed due to the service area, and UE capabilities.

3.	The SEALDD server acts as the NRM server to interacts with the BM-SC to activate the eMBMS bearer as described in clause 14.3.4.2, and 14.3.4.2 3GPP TS 23.434 [23.434], and/or interacts with the 5GS to starts the broadcast MBS session as described in clause 14.3.4A.2 of 3GPP TS 23.434 [23.434].

The SEALDD server acts as the NRM server and the SEALDD client acts as the NRM client to handle the handle the session/service announcement, listening status report as specified in clause 14.3.2.3 and 14.3.2.4 of 3GPP TS 23.434 [23.434]。

4.	The SEALDD server allocates an ingress address for the VAL server to send its DL data. The SEALDD server returns the Sdd_BroadcastDelivery create response with the allocated ingress address to the VAL server.

5.	The VAL server starts to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the established eMBMS bearer and/or the broadcast MBS session.

6.	Later, the SEALDD server may receive the listening status report from the SEALDD client 1 that it unable to receive the DL broadcast data.

7.	The SEALDD server sends the DL data to the SEALDD client 1 via unicast as well.

6.x.1.2.2	Broadcast data delivery service update

Figure 6.x.1.2.2-1 illustrates the high-level procedure of updating a broadcast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast delivery service as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.2-1: Broadcast delivery service update

1.	The VAL server invokes the Sdd_broadcast data delivery update service. The TMGI and updated parameters, e.g., service area or QoS requirements are included.

2.	The SEALDD server acts as the NRM server to initiate the eMBMS bearer update and/or the broadcast MBS session update accordingly and interact with the SEALDD client acting as NRM client as described in clause 14.3.4A.3 of 3GPP TS 23.434 [23.434]. If the service areas are updated and new broadcast MBS session(s) is created, addition MB-UPF ingress address(es) may be also returned. The SEALDD server associates it with the allocated ingress address towards the VAL server.

3.	The SEALDD server returns the Sdd_BroadcastDelivery update response indicating the success of update.

4.	The VAL server continues to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the updated eMBMS bearer and/or the broadcast MBS session.

6.x.1.2.3	Broadcast data delivery service delete

Figure 6.x.1.2.3-1 illustrates the high-level procedure of deleting a broadcast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast delivery service as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.3-1: Broadcast delivery service delete

1.	The VAL server no longer requires the broadcast delivery service. It invokes the Sdd_broadcast data delivery delete service. The TMGI is included.

2.	The SEALDD server acts as the NRM server and performs the eMBMS bearer delete and/or the broadcast MBS session release accordingly and interact with the SEALDD client which acting as the NRM client as described in clause 14.3.4A.4 of 3GPP TS 23.434 [23.434].

4.	The SEALDD server returns the Sdd_BroadcastDelivery delete response indicating the success of update.

6.x.2	Architecture Impacts

There are no architecture impacts.

6.x.3	Corresponding services

This solution introduces a new SEALDD data delivery service.

6.x.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.