3GPP TSG-SA WG6 Meeting #67	S6-252406

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252223)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on new solution of multicast data delivery service to KI#1

Spec:	3GPP TR 23.700-52 0.1.0

Agenda item:	9.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed a new solution to KI#1.

2. Reason for Change

In this solution, the SEALDD server will handle the multicast data delivery request and interacts with the 5GS accordingly. And the SEALDD server act as the media ingress node towards the 3GPP network multicast user plane functions e.g., UPF(s).

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.1.0.

* * * First Change * * * *

6	Solutions

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x | X |  |  |  |
| Sol #2 |  |  |  |  |

6.x	Solution #x: SEALDD enabled multicast data delivery

6.x.1	Solution description

Editor's Note:	This clause describes the solution. Each solution should clearly describe which of the key issues it covers and how.

6.x.1.1	General

In this solution, the SEALDD server provides new data delivery services to deliver the same content (e.g., application signalling or application user plane data) from a single source entity to a group of pre-defined recipients. The SEALDD server provides multicast delivery services:

- SEALDD multicast data delivery service. It is used for the application that provides the same content to a group of pre-defined recipient. It requires the support of the multicast 5MBS session feature at the network and UE. Compared with broadcast data delivery service, it may provide user level QoS guarantee.

In general, the multicast delivery services require the VAL server to provide its service requirements e.g., service area, QoS requirements. The SEALDD server provides its own address as the ingress address for the VAL server to send the DL data. During the data delivery service update with service area update, more MBS session may be created and additional MB-UPF addresses are returned to the SEALDD server. However, SEALDD server hides the multiple network user plane addresses towards the VAL server and the ingress address keep the same.

In this solution, the SEALDD server acts as the NRM server to interact with the 3GPP network to manage the 5G multicast MBS session. Also the SEALDD client acts as the NRM client to interact with SEALDD server acting as the role of NRM server to handle the session/service announcement, listening status report and UE session join notification. The SEALDD-UU utilizes the multicast functions of NRM-UU to support this multicast data delivery service.

Figure 6.x.1.1 -1 shows the high-level functional model for this service.

Figure 6.x.1.1-1:Multicast delivery service functional model

Editor’s note:	Whether to have a single data delivery service to cover both the broadcast data delivery and multicast data delivery is FFS.

6.x.1.2	Multicast data delivery service

6.x.1.2.1	Multicast data delivery service create

Figure 6.x.1.2.1-1 illustrates the high-level procedure of creating a multicast data delivery service for the VAL server DL data delivery.

Figure 6.x.1.2.1-1: Multicast delivery service create

1.	The VAL server invokes the Sdd_MulticastDelivery create service. The service requirements e.g., service area, activation or deactivation, QoS requirements are included.

2.	The SEALDD server determines to create a multicast flow. The SEALDD server allocates the broadcast flow ID, the source multicast address and port used for DL data delivery to the recipients.

3.	The SEALDD server acts as the NRM server to interacts with the 5GS to create the multicast MBS session as described in clause 14.3.4A.2 of 3GPP TS 23.434 [23.434].

4.	The SEALDD server allocates an ingress address for the VAL server to send its DL data. The SEALDD also allocates a data delivery service ID which is used to update and delete the created data delivery service.The SEALDD server returns the Sdd_MulticastDelivery create response with the allocated ingress address to the VAL server.

5.	The VAL server provision the service information including the SEALDD server address, data delivery service ID to the VAL client.

6. The VAL client sends the service request to the SEALDD client. The SEALDD server address, data delivery service ID are included.

7.	The SEALDD client sends the connection request (data delivery service ID) to the SEALDD server.

8.	The SEALDD server determines and returns the media related information including TMGI, broadcast flow ID, the source multicast address and port, protocols used for the DL data delivery based on the data delivery service ID.

9.	The UE starts to lisening the broadcast channel as inticated by the TMGI to receive the DL data.

10.		The VAL server starts to send DL data to the allocated ingress address. The SEALDD server encapsulates the received DL data within the multicast flow packet with the source multicast address and further delivers the encapsulated packet(s). via the established multicast MBS session. The 3GPP network may use PTM or PTP to deliver the DL to the UE.

NOTE:	The VAL server can start to send data to the SEALDD server after step 5. And the SEALDD server can start to send DL data to the network even before step 7. Then the UE can immediately receive the data after step 9.

6.x.1.2.2	Multicast data delivery service update

Figure 6.x.1.2.2-1 illustrates the high-level procedure of updating a multicast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a multicast delivery service as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.2-1: Multicast delivery service update

1.	The VAL server invokes the Sdd_Multicast data delivery update service. The data delivery service ID and updated parameters, e.g., service area, session status (activation or deactivation) or QoS requirements are included. For example, the VAL server temporarily has no DL data to transmit, it may deactivate the multicast MBS session.

2.	The SEALDD server acts as the NRM server and performs the multicast MBS session update accordingly as described in clause 14.3.4A.3 of 3GPP TS 23.434 [23.434]. If the service areas are updated and new multicast MBS session(s) is created, addition MB-UPF ingress address(es) may be also returned. The SEALDD server associates it with the allocated ingress address towards the VAL server.

3.	The SEALDD server returns the Sdd_MulticastDelivery update response indicating the success of update.

4.	The VAL server continues to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the updated multicast MBS session(s).

6.x.1.2.3	Multicast data delivery service delete

Figure 6.x.1.2.3-1 illustrates the high-level procedure of deleting a multicast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a multicast delivery service as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.3-1: Multicast delivery service delete

1.	The VAL server no longer requires the multicast delivery service. It invokes the Sdd_Multicast data delivery delete service. Thedata delivery service ID is included.

2.	The SEALDD server acts as the NRM server and performs the multicast MBS session delete accordingly as described in clause 14.3.4A.4 of 3GPP TS 23.434 [23.434].

3.	The SEALDD server returns the Sdd_MulticastDelivery delete response indicating the success of update.

4.	The UE leave the multicast MBS session accordingly based on the dis-announcement.

6.x.2	Architecture Impacts

There are no architecture impacts.

6.x.3	Corresponding services

This solution introduces a new SEALDD data delivery service.

6.x.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.