3GPP TSG-SA WG6 Meeting #67	S6-252410

Fukuoka, Japan, 19th – 23rd May 2025	Revision of S6-252142

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0044 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Management operation of spatial anchor group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the last SA6 meeting, it was agreed to introduce the term of spatial anchor group, which refers to the collection of spatial anchors based on locations (e.g. museums in a city) or objects (e.g. items in a shop), etc. However, the procedures of spatial anchor group management had not reached any consunense. For a store owner, he/she may want to creat spatial anchor group for some items with same or similiar attributes. For example, he/she can put all breads with same expiration date into a group, so that she can delete them all at one time instead of delete them by many times when they are exspired. This maks his/her managemnt more efficent. So, it is necessary to enhance the management procedure to support group operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add an IE “group identifier” for spatial achors, in Spatial anchor creat request message - Add an IE “group identifier” for each spatial achor, in spatial anchor profile. - Update the procedure of spatial anchor create request/response to support anchor group operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management operation of spatial anchor group is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * First Change * * * *

7.3.2	Common values related to spatial anchors

7.3.2.1	Spatial anchor profile

Table 7.3.2.1-1 describes information elements for the spatial anchor profile stored in the SAn server.

Table 7.3.2.1-1: Spatial anchor profile

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| Spatial map position (NOTE) | O | Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information. |
| Spatial anchor specific information | O | A container to contain application specific information. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > VAL Service ID | M | The identifier of the application service. |
| Service area | O | The service area where the spatial anchor is accessible. |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| Spatial anchor group ID | O | Identifier of spatial anchor group that the spacital anchor associates with |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

8.3.1	Spatial anchor creation

8.3.1.1	General

Spatial anchor creation enables a SAn client or VAL server to create spatial anchor(s) with the SAn server.

8.3.1.2	Procedure

Figure 8.3.1.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.1.2-1: Spatial anchor creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor create request to the SAn server. The request includes a requestor identifier, security credentials, and list of spatial anchors to be created as described in clause 8.3.1.3.1. The request includes a position, allowed entities list defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). If the request includes spatial anchor specific information, that information remains transparent to the SAn server and can be stored with the spatial anchor. The request can indicate to create multiple spatial anchors. The requsted message may include one spatial anchor group ID for each spatial anchor to be created, if the requestor has got a available spatial anchor group ID.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a name, a URL, a media data, a type, a manufacturing company, a date, an expiry date, a ratings, and customer premise information (e.g. a residence, office or shop), a service schedule defining times when the spatial anchor(s) are active and inactive, etc. Such information may be stored in key-value pair and is transparent to the SAn server.

NOTE: The spatial anchor group ID is available with the requestor through spatial anchor group creation or through mechanism which could be implementation specific.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the spatial anchor(s), assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The SAn server sends a spatial anchor create response to the requestor. If the SAn server created spatial anchors, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and can include a reason for failure.

8.3.1.3	Information flows

8.3.1.3.1	Spatial anchor create request

Table 8.3.1.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.1.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position information | M | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format). |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| > Spatial anchor resource identifer | M | The identifier to associate the resource information with the created spatial anchors. |
| >Spatial anchor group ID | O | The identifier of spatial anchor group to be associated with the created spatial anchor |
| NOTE: One of the IEs shall be present. |  |  |

8.3.1.3.2	Spatial anchor create response

Table 8.3.1.3.2-1 describes information elements for the spatial anchor create response from the SAn service.

Table 8.3.1.3.2-1: Spatial anchor create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor create request was successful. |
| > List of spatial anchors | O | The list of created spatial anchors. |
| >> Spatial anchor ID | O | The identifier of the spatial anchor |
| >> Spatial anchor resource identifier | O | The identifier provided by the SAn client in order to associate the created spatial anchor with the resource information used for creating the spatial anchor. |
| >> Spatial anchor group ID | O | Identifier of spatial anchor group being associated with this spatial anchor. |
| Failure response | O | Indicates that the create spatial anchor request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: If spatial anchor group ID is present for one spatial anchor entry in spatial anchor creat request message, Spatial anchor group ID also should be present for the corresponding spatial anchor if it has been created successfully. |  |  |