3GPP TSG-SA WG6 Meeting #67	S6-252424

Fukuoka, Japan, 19th – 23th May 2025	(revision of 252xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0152 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to message names |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During SA6#63, new messages was introduced in CR 0102 to differentiate the regular transmission connection request sent from the server from the regualr transmission connection request sent from the client. During SA6#66, the CR 0143 was agreed to further clarify that the new clause 9.2.3.17 and 9.2.3.18 are taken out from original clause 9.2.3.3 and 9.2.3.4. However, clause 9.2.3.17 and 9.2.3.18 use a totally new message name which may cause lead implementation mistakes about SEALDD-uu and SEALDD-S service in stage-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the message names in clause 9.2.3.3, 9.2.3.4, 9.2.3.17 and 9.2.3.18 - Change the procedur and figure accoringly in clause 9.2.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leading to implementation mistakes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.2.1, 9.2.2.3, 9.2.3.3, 9.2.3.4, 9.2.3.17, 9.2.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[13]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[14]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[15]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[16]	3GPP TS 23.303: "Proximity-based Services (ProSe); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[18]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[19]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[20]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[21]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[22]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[23]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[24]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[25]	3GPP TS 24.543: "Data Delivery Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

# /******************************* Next Change *****************************/

9.2.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection (also termed SEALDD-UU flow) is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

NOTE 1:	SEALDD server and VAL server may have different behaviour when establishing the connection for signalling transmission and regular data transmission. For signalling transmission, the VAL server may allocate the same address and port to send/receive the signalling traffic of all the users. For data transmission, the VAL server may allocate different addresses and ports to send/receive the data traffic of different users. And SEALDD server may need to identify the data traffic by checking the SEALDD connection establishment request since different SEALDD clients’ application data traffic should be mapped to their specific SEALDD-S connection.

NOTE 2:	The SEALDD client and the SEALDD server can use the same service operation to request the SEALDD regular transmission connection establishment and it is specified in 3GPP TS 24.543 [25].

# /******************************* Next Change *****************************/

9.2.2.3	SEALDD enabled regular data transmission connection establishment based on policy

The SEALDD servers has Data Delivery (DD) policy being provisioned. Before the application communication between VAL client and VAL server starts, the DD policy is enforced by the SEALDD server to establish the SEALDD connection.

Pre-conditions:

1.	The SEALDD server has DD policies available.

Figure 9.2.2.3-1: Policy enforced by SEALDD server for connectivity

1.	The VAL server subscribes to SEALDD event exposure for connection status using the procedure defined in clause 9.2.2.6.

NOTE 1:	The VAL server can update/delete an existing subscription at the SEALDD Server when required.

2.	When the time for data transmission is about to start, the SEALDD server enforces the policy to trigger regular data transmission connection establishment. If spatial condition for UE is provided, the SEALDD server also ensures the UE’s location requirement is satisfied when establishing regular data transmission connection (e.g. by using NEF service for monitoring UE location or SEAL location service for UE entering area of interest).

3.	If there is a special routing requirement for SEALDD user plane traffic (e.g. running on a specific slice and DNN), the SEALDD server interacts with 3GPP CN to provision service specific parameters with NEF as described in 3GPP TS 23.502 [6], clause 4.15.6.10 and clause 4.15.6.7.

If there are QoS requirements in the DD policy, the SEALDD server also applies QoS to ensure the quality for SEALDD traffic by utilizing NEF/PCF/NRM/EES service for QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4].

If the DD policy specifies failure detection report, the SEALDD server may subscribe to CN analytics (e.g. DN performance analytics) from NEF/NWDAF and further notify data delivery status of application traffic to VAL client (via SEALDD client) and VAL server based on analytics result.

4.	The SEALDD server allocates an IP address and port for sending and receiving packet over SEALDD-S reference point, then SEALDD server sends SEALDD connection establishment notification (i.e. SEALDD connection status notification with establishment event, as described in Table 9.2.3.9-1) to the VAL server with VAL service ID, the address.

5-6.	The SEALDD server allocates a SEALDD-S connection information for sending and receiving packet over SEALDD-UU reference point, then SEALDD server sends SEALDD regular data transmission connection establishment request to the SEALDD client with SEALDD-UU flow ID, VAL service ID, the address. The request is responded by the SEALDD client. UE IP address may be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane.

NOTE 2:	Step 4 and step 5 can be done in parallel.

NOTE 3:	SEALDD server can know the UE address (e.g. via SEALDD connection establishment request initiated by SEALDD client, but rejected by SEALDD server due to policy condition not met) before step 5, if not, step 5 can be sent via application triggering.

7.	The SEALDD client further notifies the VAL client about the SEALDD connection being established.

Upon receiving application traffic from VAL client (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies application traffic based on the VAL service ID and further sends the application traffic to VAL server. The downlink application traffic sent from VAL server to VAL client is processed similarly.

# /******************************* Next Change *****************************/

9.2.3.3	SEALDD regular transmission connection establishment request (Client - Server)

Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the regular SEALDD connection establishment.

Table 9.2.3.3-1: SEALDD regular transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) or the SEALDD server side (for server side initiated request) used to establish SEALDD connection. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server side initiated request. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2) | Indicates BAT and periodicity adaptation capability of the SEALDD client side (for client side initiated request) per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 2, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for client side initiated request). It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| L4S feedback capability | O (See NOTE 3) | Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request |
| VAL UE client access capability | O | Indicates the access technologies supported by VAL UE client. E.g.,3GPP, Non-3GPP(WLAN) |
| XR Application device capability information | O (See NOTE 5) | Indicates XR Application device capability information (e.g., encoding capabilities) |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 5: This IE is used for the SEALDD enabled XR traffic data delivery. |  |  |

9.2.3.4	SEALDD regular transmission connection establishment response (Server to Client)

Table 9.2.3.4-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding to the regular SEALDD connection establishment.

Table 9.2.3.4-1: SEALDD regular transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side (for client side initiated request) or the SEALDD client side (for server side initiated request) used to establish SEALDD connection. |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD client or SEALDD server to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| Capability for BAT and periodicity adaptation | O (See NOTE 3) | Indicates BAT and periodicity adaptation capability of the SEALDD client side (for server side initiated request) per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 3, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for server side initiated request). It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure NOTE 3: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. |  |  |

# /******************************* Next Change *****************************/

9.2.3.17	SEALDD data transmission connection establishment request (Server - Client)

Table 9.2.3.17-1 describes the information flow from the SEALDD server to the SEALDD client for requesting the SEALDD data transmission connection establishment.

Table 9.2.3.17-1: SEALDD data transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server used to establish SEALDD connection. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

9.2.3.18	SEALDD data transmission connection establishment response (Client - Server)

Table 9.2.3.18-1 describes the information flow from the SEALDD client to the SEALDD server for responding to the SEALDD data transmission connection establishment.

Table 9.2.3.18-1: SEALDD data transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side used to establish SEALDD-UU data connection. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD client to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| Capability for BAT and periodicity adaptation | O (See NOTE 3) | Indicates BAT and periodicity adaptation capability for SEALDD client. |
| Capability for BAT and periodicity adaptation | O (See NOTE 3) | Indicates BAT and periodicity adaptation capability for SEALDD client. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. NOTE 2: This IE is only present if the Result is failure NOTE 3: If provided, only one of these IEs shall be present in the message. |  |  |

# /******************************* End of Changes *****************************/