3GPP TSG-SA WG6 Meeting #67	S6-252431

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252107)

Source:	InterDigital

Title:	Solution on Management of evaluation information for ML model performance and correctness evaluation

Spec:	3GPP TR 23.700-83 v0.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a solution to KI#2.

2. Reason for Change

This contribution proposes a solution to Key Issue #2: support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer.

The solution proposes procedures for management of evaluation information needed to perform ML model performance and correctness evaluation.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.1.0.

* * * First Change * * * *

## 7.x	Solution #x: Management of evaluation information for ML model performance and correctness evaluation.

### 7.x.1	Solution description

#### 7.x.1.1 Introduction

This solution proposes enhancements to address Key Issue #2 – Open Issue #2 and provides procedures required to enable evaluation of ML model performance and correctness considering application specific ML models.

This solution proposes to:

- define new procedures and information flows to enable management (e.g., storage, retrieval, removal) of ML model evaluation information in the ML model repository;

- update information flows to link the ML model evaluation information with ML model(s).

The solution allows the owner of ML model(s) to store and manage the evaluation information associated with ML model(s). The evaluation information is information that can be retrieved to perform the evaluation of performance and correctness of ML model(s). This solution can be combined with existing and/or new procedures when evaluation of a ML model performance and correctness is required.

ML models that can be stored and managed in the AIML enablement layer are application specific ML models. Such ML models can be retrieved by the AIMLE client or VAL server to perform inferencing at the VAL layer.

ML model specific evaluation information is required for a consumer of a ML model to perform evaluation of a ML model performance and correctness. Thus, this solution proposes to enable storage and retrieval of ML model evaluation information in the ML model repository.

#### 7.x.1.2 ML model evaluation information storage

Figure 7.x.1.2-1 describes procedures for storing ML model evaluation information in the ML model repository.

Figure 7.x.1.2-1. ML evaluation information storage

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information storage request to the AIMLE server. The request includes the requestor identifier and ML evaluation information.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information storage request to store the ML evaluation information in the ML repository. The request includes the requestor identifier and ML evaluation information.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository stores the ML evaluation information, creates an ML evaluation profile including a ML evaluation profile identifier and sends an ML evaluation information storage response to the AIMLE server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information storage response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

#### 7.x.1.3 ML model evaluation information retrieval

Figure 7.x.1.3-1 describes procedures for retrieving ML model evaluation information from the ML model repository.

Figure 7.x.1.3-1. ML evaluation information retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information retrieval request to the AIMLE server. The request includes the requestor identifier and ML evaluation information retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information retrieval request to obtain the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information retrieval filters.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository identifies ML evaluation profiles based on the ML evaluation information retrieval filters and sends a ML evaluation information retrieval response to the requestor. The response includes an indication of success for the request and the identified ML evaluation profiles if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information retrieval response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile(s) if the request was successful.

#### 7.x.1.4 ML model evaluation information removal

Figure 7.x.1.4-1 describes procedures for removal of ML model evaluation information from the ML model repository.

Figure 7.x.1.4-1. ML evaluation information removal

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information removal request to the AIMLE server. The request includes the requestor identifier and ML evaluation information identifier(s) to be removed.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information removal request to remove the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information identifier(s).

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository removes the ML evaluation information based on the ML evaluation information identifier(s) and sends a ML evaluation information removal response to the requestor. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information removal response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

### 7.x.2	Architecture Impacts

No architecture impact has been identified.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	ML Evaluation Profile

Table 7.x.3.1-1 describes the ML evaluation profile and Table 7.x.3.1-2 describes the ML evaluation which is used in the ML evaluation information management procedures between the AIMLE client, VAL server, AIMLE server and ML model repository.

Table 7.x.3.1-1: ML Evaluation Profile

| Information element | Status | Description |
| --- | --- | --- |
| ML evaluation profile identifier | M | The identifier of the ML evaluation profile. |
| AIMLE server identifier | M | The identifier of the AIMLE server that stored the ML evaluation information. |
| ML repository identifier | M | The identifier of the ML repository where the ML evaluation information is stored. |
| List of ML evaluations | M | The list of ML evaluation information as defined in Table 7.x.3.1-2. |

Table 7.x.3.1-2: ML Evaluation Information

| Information element | Status | Description |
| --- | --- | --- |
| Evaluation identifier | M | The identifier of the evaluation information used for evaluating ML model(s). |
| List of ML models identifiers | M | The list of ML models identifiers to which the evaluation applies. |
| Source identifier | O | The identifier of the source (e.g., VAL server ID, VAL client ID) that stored the ML evaluation information in the ML repository. |
| Description | M | The description of the ML model evaluation. |
| Metric information | M | The metrics measured by the performing the ML model evaluation. |
| Input information | M | Represents the input information which is aligned with the associated ML model input parameters. The encoding, format, and value of input information is not specified since it is specific to the ML model(s) to which this evaluation applies. |
| Output information | M | Represents the expected output information which is expected as a result of processing the input information by the ML model. The encoding, format, and value of output information is not specified since it is specific to the ML model(s) to which this evaluation applies. |
| Grading information | M | The information for evaluating and grading the performance and correctness of a ML model (e.g., grading the obtained evaluation output based on the expected output). |
| Execution information | O | The execution information which is required to evaluate performance and correctness of a ML model. The information indicates whether the evaluation is executed once, repeatedly in a time window, has its output averaged, etc. If not included, the evaluation is evaluated and graded once. |

#### 7.x.3.2	ML Evaluation Information Storage

Table 7.x.3.2-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for storing ML evaluation information in the ML repository.

Table 7.x.3.2-1: ML Evaluation Information Storage Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation information | M | The list of ML evaluation information to be stored in the ML repository as described in Table 7.x.3.1-2. |

Table 7.x.3.2-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information storage request.

Table 7.x.3.2-2: ML Evaluation Information Storage Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > ML evaluation profile identifier | M | The ML evaluation profile identifier stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.3	ML Evaluation Information Retrieval

Table 7.x.3.3-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for retrieving ML evaluation information from the ML repository.

Table 7.x.3.3-1: ML Evaluation Information Retrieval Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Evaluation information filters | M | The evaluation information filter used to identify the retrieved ML evaluation information. |
| > Evaluation profile identifier | O | Represent the filtering criteria which is a ML evaluation profile identifier. |
| > Evaluation information filter | O | Represents the filtering criteria, which can be any of the ML evaluation information as defined in Table 7.x.3.1-2. |

Table 7.x.3.3-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information retrieval request.

Table 7.x.3.3-2: ML Evaluation Information Retrieval Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profiles | M | The list of retrieved ML evaluation profiles stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.4	ML Evaluation Information Removal

Table 7.x.3.4-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for removing ML evaluation information from the ML repository.

Table 7.x.3.4-1: ML Evaluation Information Removal Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation profile identifiers | M | The list of evaluation profile identifiers to be removed from the ML model repository. |

Table 7.x.3.4-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information removal request.

Table 7.x.3.4-2: ML Evaluation Information Removal Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profile identifiers | M | The list of ML evaluation profile identifiers removed from the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.x.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

* * * End of Changes * * * *