3GPP TSG-SA6 Meeting #67	S6-252458

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025                       (revision of S6-252055)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0387 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 9.4.1 SEAL APIs for location management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table 9.4.1-1 summarizes SEAL APIs and operations of location management and some inconsistencies were found. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the third column “Known Consumer(s)” for the API operations in Table 9.4.1-1 and the descriptions in clauses 9.4.2 through 9.4.12. - Generally, a “consumer” is an entity that calls (sends a request) an API and a “producer” is an entity that processes the request message and responds. - In accordance with this definition, the column “Known Consumer(s)” needs to be modified based on the procedures specified in clause 9.3 location management. - Add “SS_LocationAreaMonitoring” API name - Change “Delete_Location_Tracing_Configuration” to “Cancel_Location_Tracing_Configuration” in “SS_LocationHistoryInfoEvent” API based on clause 9.3.21.3. - Modify the operation description of 9.4.8.5 Delete_VAL_Service_Area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in 3GPP TS 23.434. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1, 9.4.8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting | VAL server | Subscribe/Notify |
|  | Update_Trigger_Location_Reporting | VAL server |  |
|  | Cancel_Trigger_Location_Reporting | VAL server |  |
|  | Notify_Trigger_Location_Reporting | VAL server |  |
|  | Notify_Adaptive_Configuration | VAL server |  |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events | VAL server |  |
|  | Update_Location_Area_Monitoring_Subscribe | VAL server |  |
|  | Unsubscribe_Location_Area_Monitoring | VAL server |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_LocationHistoryInfoEvent | Create_Location_Tracing_Configuration | VAL server | Request/Response |
|  | Update_Location_Tracing_Configuration | VAL server |  |
|  | Cancel_Location_Tracing_Configuration | VAL server |  |
|  | Obtain_history_Location_Info | VAL server |  |
| SS_SLPositioningManagement | Subscribe_SLPositioningManagement | VAL server | Subscribe/Notify |
|  | Notify_SLPositioningManagement | VAL server |  |
| SS_SRPositioningInformation | SR_Positioning_Information | VAL server | Request/Response |
| SS_ConfirmLocation | Confirm_Location | LMC | Request/Response |
|  | ConfirmLocationServiceSubscription | VAL server | Request/Response |
|  | Notify_ConfirmLocationServiceUsage (NOTE) | VAL server | Subscribe/Notify |
| NOTE: The subscribe operation is part of ConfirmLocationServiceSubscription |  |  |  |

* * * Next Change * * * *

9.4.8.5	Delete_VAL_Service_Area

API operation name: Delete_VAL_Service_Area

Description: Delete VAL service area identifier.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.30

Outputs: See subclause 9.3.2.31

See subclause 9.3.13.5 for the details of usage of this API operation.

* * * End of Changes * * * *