3GPP TSG-SA WG6 Meeting #67	S6-252464

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252228)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0742 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Edge Enabler Using Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | R19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The following description is not correct, since the EEC is not be able to receive the EES profile in the service provisioning response message： " the EEC determines the EES where the Satellite ID in the EES profile matches the UE serving Satellite ID" Since the EEC only receive the EDN configuration informaiton in the service provisoining response message instead of EES profile. - It is not clear how EEC determine which EES to connect to when EEC receives a list of EES and the EEC is not aware of the UE serving Satellite ID, that is the EEC only provide “predicted/expected UE location, Prediction expiration time” to the ECS. Thus, the description of how the EEC determine which EES to connect to is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the EDN configuration information with "Satellite ID" and “Satellite coverage availability information” and fix the corresponding description - Add description of the EEC determines the EES based on UE current location and corresponding current time and received EES Satellite coverage availability information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EES determination based on UE location and route Procedure will remain incorrect and unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2, 11.1.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.1.2	EES determination based on UE location and route Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.5. The service provisioning procedure for satellite EDN is the same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications. When EEC sends the service request message to the ECS and the request message contains the UE location, predicted/expected UE location, Prediction expiration time, then the ECS determines a list of EES where the Satellite coverage availability information in EES profile matches the UE location, predicted/expected UE location, Prediction expiration time, and EES Satellite coverage availability information. Alternatively, the ECS may query external server with the UE current location and UE predicted/expected location to obtain a list of Satellite IDs and Trajectory IDs corresponding the UE location and the UE trajectory, then the ECS determines list of EES based on the Satellite IDs and Trajectory IDs in EES profiles by matching with the Satellite IDs and Trajectory IDs obtained from external server for the UE. The “Expected AC Geographical Service Area” in the AC profile, as specified in clause 8.2.2, is used by the ECS to request the external server (managed by the satellite service provider or operator) for such satellite IDs and trajectory IDs. When ECS sends the service provisioning response which contains a list of EES information which EES is deployed on MEO or LEO satellites, the EEC determines the EES where the Satellite ID corresponding to the EES in the EDN configuration information matches the UE serving Satellite ID.

In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage, the same satellite comes back for serving, then the EEC connects to same EES and EAS (i.e. to which the EEC had previously connected to), without needing to discover EES and EAS endpoints again.

Editor's Note: Whether and how the ACR procedure need to be enhanced is FFS.

* * * Next Change * * * *

11.1.2.X	Service provisioning response

Table 11.1.2.X-1 is same as clause 8.3.3.3.3 Table 8.3.3.3.3-2 with the following additions.

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.2-1 |  |  |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| New IEs |  |  |
| > Satellite ID | O | The satellite ID which identifies a satellite EDN. |
| IEs from Table 8.3.3.3.2-1 |  |  |

* * * End of Changes * * * *