3GPP TSG-SA WG6 Meeting #67	S6-252465

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252153)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0394 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the reference of Satellite coverage availability information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the feedback from CT3 and CT1, the definition of satellite coverage availability information is not clear in the current stage-2 specifications. In order to make it clear how to use it in SA6, it’s needed to define the contents of Satellite coverage availability information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rename and define the Satellite coverage availability information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 21.2.3.1, 21.2.3.4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21.2.3.1	Application satellite coverage availability information configuration request

Table 21.2.3.1-1 describes the information flow from the configuration management server to the configuration management client to configure the application satellite coverage availability information.

Table 21.2.3.1-1: Application satellite coverage availability information configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Application Satellite coverage availability information | M | Defines the application satellite coverage information including a list of geographical areas, corresponding available time periods and satellite RAT types. |
| > Geographical area | M | It includes a list of geographical area information (e.g. coordinates, as defined in 3GPP TS 29.572 [51]) where the satellite coverage is available. |
| > Availability time period | M | It provides the time periods when the satellite coverage is available over the indicated geographical area. |
| > Availability satellite RAT types | O | It provides the satellite RAT types information corresponding to the satellite availability in the indicated geographical area. |

* * * Next Change * * * *

21.2.3.4	Get application satellite coverage availability information response

Table 21.2.3.4-1 describes the information flow from the configuration management server to the configuration management client or VAL server to notify the application satellite coverage availability information.

Table 21.2.3.4-1: Get application satellite coverage availability information response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Application Satellite coverage availability information | M | Defines the application satellite coverage information including a list of geographical areas, corresponding available time periods and satellite RAT types. |
| > Geographical area | M | It includes a list of geographical area information (e.g. coordinates, as defined in 3GPP TS 29.572 [51]) where the satellite coverage is available. |
| > Availability time period | M | It provides the time periods when the satellite coverage is available over the indicated geographical area. |
| > Availability satellite RAT types | O | It provides the satellite RAT types information corresponding to the satellite availability in the indicated geographical area. |

* * * End of Changes * * * *