3GPP TSG-SA WG6 Meeting #67	S6-252472

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252226)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on updating Technical Solution #6

Spec:	3GPP TR 23.700-35 v0.4.0

Agenda item:	9.3

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to a new solution to the gaps identified in clause 4.1.

2. Reason for Change

The procedures, information flows and the evaluation are still missing in technical solution#6.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.4.0.

# * * * First Change * * * *

8.1.6	Technical Solution #6: Multiple NRM services for network service QoS control

8.1.6.1	Solution description

This solution proposes to address the gaps identified in clause 4.1 regarding the support various application requirements at NRM.

It is proposed to introduce a new set of NRM APIs to support the various applications per their requirements to the network resource and handling based on the following principles:

-	The new NRM APIs are designed per application layer requirements as described in clause 8.4.A.

-	The input parameters of each service API are avoiding excessive configurations and necessary to achieve the desired benefit corresponding to the application requirement(s).

-	The name of the service, service APIs, input parameters are familiar to the application community.

The list of several NRM services are as follows:

-	QoS control service for network transport with high data rate and low latency. This service enables the VAL server to request the network transport resource with high data rate and low latency. This may be used by AR/VR applications.

-	QoS control service for network transport with low latency. This service enables the VAL server to request network transport resource with low latency. Optionally, the loose requirements on bit rate is applied. This may be used by multi-modal communications.

-	QoS control service for network transport with low latency and high reliability. This service enables the VAL server to request network transport resource with low latency and high reliability. This may be used by cyber-physical control applications, V2X, rail communications.

-	QoS control service for network transport with low latency & high bandwidth & high reliability. This service enables the VAL server to request network transport resource with low latency, high bandwidth and high reliability. This may be used by AV production service, medical or gaming applications.

The new set of QoS control services at the NRM is aiming at the application stakeholders who lack the 3GPP network background but wish to and wish to start an agile development in a short cycle.

8.1.6.1.1	Procedures

Editor’s note:	The procedure and information flows to be added is FFS.

8.1.6.1.1.1		QoS control service for network transport with high data rate and low latency service

The figure 8.1.6.1.1.1-1 illustrates the high-level procedures of creating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlHighDataRateLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.1-1: Create QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the Nrm_QosCtrlHighDataRateLowLatency create service to request the network transport resource with high data rate and low latency QoS guarantee. The high data rate requirements (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth), low latency requirements (e.g., UL, DL and/or round-trip latency) are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QosCtrlHighDataRateLowLatency create response indication the result.

The figure 8.1.6.1.1.1-2 illustrates the high-level procedures of updating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlHighDataRateLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.1-2: Update QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the Nrm_QosCtrlHighDataRateLowLatency update service to update the network transport resource with high data rate and low latency QoS guarantee. The updated high data rate requirements and/or updated low latency requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QosCtrlHighDataRateLowLatency update response indication the result.

The figure 8.1.6.1.1.1-3 illustrates the high-level procedures of deleting the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlHighDataRateLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.1-3: Delete QoS control service for network transport with high data rate and low latency

1.	The VAL server invokes the Nrm_QosCtrlHighDataRateLowLatency delete service to delete the network transport resource with high data rate and low latency service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QosCtrlHighDataRateLowLatency delete response indication the result.

8.1.6.1.1.2	QoS control service for network transport with low latency service

The figure 8.1.6.1.1.2-1 illustrates the high-level procedures of creating the QoS control service for network transport with low latency service.

Pre-conditions:

The Nrm_QosCtrlLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.2-1: Create QoS control service for network transport with low latency

1.	The VAL server invokes the Nrm_QosCtrlLowLatency create service to request the network transport resource with low latency QoS guarantee. The low latency requirements (e.g., UL, DL and/or round-trip latency) are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QosCtrlLowLatency create response indication the result.

The figure 8.1.6.1.1.2-2 illustrates the high-level procedures of updating the QoS control service for network transport with high data rate and low latency service.

Pre-conditions:

The Nrm_QosCtrlLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.2-2: Update QoS control service for network transport with low latency

1.	The VAL server invokes the Nrm_QosCtrlLowLatency update service to update the network transport resource with low latency QoS guarantee. The updated low latency requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QosCtrlLowLatency update response indication the result.

The figure 8.1.6.1.1.2-3 illustrates the high-level procedures of deleting the QoS control service for network transport with low latency service.

Pre-conditions:

The Nrm_QosCtrlLowLatency service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1-3: Delete QoS control service for network transport with low latency

1.	The VAL server invokes the Nrm_QosCtrlLowLatency delete service to delete the network transport resource with low latency service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QosCtrlLowLatency delete response indication the result.

8.1.6.1.1.3		QoS control service for network transport with low latency and high reliability service

The figure 8.1.6.1.1.3-1 illustrates the high-level procedures of creating the QoS control service for network transport with low latency and high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.3-1: Create QoS control service for network transport with low latency and high reliability

1.	The VAL server invokes the Nrm_QoSCtrlLowLatencyHighReliability create service to request the network transport resource with low latency and high reliability QoS guarantee. The low latency requirements (e.g., UL, DL and/or round-trip latency) and high reliability requirements (e.g., request packet error rate) are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QoSCtrlLowLatencyHighReliability create response indication the result.

The figure 8.1.6.1.1.3-2 illustrates the high-level procedures of updating the QoS control service for network transport with low latency and high reliability service.

Pre-conditions:

The Nrm_QoSCtrlLowLatencyHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.2-2: Update QoS control service for network transport with low latency and high reliability

1.	The VAL server invokes the Nrm_QoSCtrlLowLatencyHighReliability update service to request the network transport resource with low latency and high reliability QoS guarantee. The updated low latency requirements and/or updated high reliability requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QoSCtrlLowLatencyHighReliability update response indication the result.

The figure 8.1.6.1.1.3-3 illustrates the high-level procedures of deleting the QoS control service for network transport with low latency and high reliability service.

Pre-conditions:

The Nrm_QoSCtrlLowLatencyHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.3-3: Delete QoS control service for network transport with low latency and high reliability

1.	The VAL server invokes the Nrm_QoSCtrlLowLatencyHighReliability delete service to delete the network transport resource with low latency and high reliability service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QoSCtrlLowLatencyHighReliability delete response indication the result.

8.1.6.1.1.4	QoS control service for network transport with low latency & high bandwidth & high reliability service

The figure 8.1.6.1.1.4-1 illustrates the high-level procedures of creating QoS control service for network transport with low latency & high bandwidth & high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighBWHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.4-1: Create QoS control service for network transport with low latency & high bandwidth & high reliability

1.	The VAL server invokes the Nrm_QosCtrlLowLatencyHighBWHighReliability create service to request the network transport resource with low latency and high bandwidth and high reliability QoS guarantee. The low latency requirements, high bandwidth requirements (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth) and the high reliability requirements are included. The service identifier which the high data rate requirements and the low latency requirements are applied and the UE ID are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_create request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_create response and the Npcf_PolicyAuthorization_create response.

3.	The NRM server returns the Nrm_QosCtrlLowLatencyHighBWHighReliability create response indication the result.

The figure 8.1.6.1.1.1-2 illustrates the high-level procedures of updating QoS control service for network transport with low latency & high bandwidth & high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighBWHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.4-2: Update QoS control service for network transport with low latency & high bandwidth & high reliability

1.	The VAL server invokes the Nrm_QosCtrlLowLatencyHighBWHighReliability update service to update the network transport resource with low latency and high bandwidth and high reliability service. The updated low latency requirements, updated high bandwidth requirements, and/or updated high reliability requirements are also included.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_update request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_update request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_update response and the Npcf_PolicyAuthorization_update response.

3.	The NRM server returns the Nrm_QosCtrlLowLatencyHighBWHighReliability update response indication the result.

The figure 8.1.6.1.1.4-3 illustrates the high-level procedures of deleting the QoS control service for network transport with low latency & high bandwidth & high reliability service.

Pre-conditions:

The Nrm_QosCtrlLowLatencyHighBWHighReliability service has been discovered by the ASP and programmed into the VAL server.

Figure 8.1.6.1.1.4-3: Delete QoS control service for network transport with low latency & high bandwidth & high reliability

1.	The VAL server invokes the Nrm_QosCtrlLowLatencyHighBWHighReliability delete service to delete the network transport resource with low latency and high bandwidth and high reliability service.

2.	Based on the request in step 1, the NRM server perform authorization check. If authorization is successful, the NRM server invokes the Nef_AFsessionwithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO trust domain) or Npcf_PolicyAuthorization_delete request (if the NRM server is within MNO trust domain). The NRM server selects the corresponding features and set the input parameters of the network service based on the parameters received in step 1. The 3GPP network processes the service invocation from the NRM and returns the Nef_AFsessionwithQoS_delete response and the Npcf_PolicyAuthorization_delete response.

3.	The NRM server returns the Nrm_QosCtrlLowLatencyHighBWHighReliability delete response indication the result.

8.1.6.1.1.5	Information flow

Table 8.1.6.1.1.5-1 describes the information flow Nrm_QosCtrlHighDataRateLowLatency from the consumer to the NRM server.

Table 8.1.6.1.1.5-1: Nrm_QosCtrlHighDataRateLowLatency create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| High data rate requirements | M | The requirement description of high data rate (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth) |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |

Table 8.1.6.1.1.5-2 describes the information flow Nrm_QosCtrlHighDataRateLowLatency create response from the NRM server to the consumer.

Table 8.1.6.1.1.5-2: Nrm_QosCtrlHighDataRateLowLatency create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

Table 8.1.6.1.1.5-3 describes the information flow Nrm_QosCtrlLowLatency create request from consumer to the NRM server.

Table 8.1.6.1.1.5-3: Nrm_QosCtrlLowLatency create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |

Table 8.1.6.1.1.5-4 describes the information flow Nrm_QosCtrlLowLatency create response from NRM server to the consumer.

Table 8.1.6.1.1.5-4: Nrm_QosCtrlLowLatency create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

Table 8.1.6.1.1.5-5 describes the information flow Nrm_QosCtrlLowLatencyHighReliability from the consumer to the NRM server.

Table 8.1.6.1.1.5-5: Nrm_QosCtrlLowLatencyHighReliability create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |
| High reliability | M | The requirement description of high reliability (e.g., request packet error rate) |

Table 8.1.6.1.1.5-6 describes the information flow Nrm_QosCtrlLowLatencyHighReliability create response from the NRM server to the consumer.

Table 8.1.6.1.1.5-6: Nrm_QosCtrlLowLatencyHighReliability create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

Table 8.1.6.1.1.5-7 describes the information flow Nrm_QosCtrlLowLatencyHighBWHighReliability from the consumer to the NRM server.

Table 8.1.6.1.1.5-1: Nrm_QosCtrlLowLatencyHighBWHighReliability create request

| Information element | Status | Description |
| --- | --- | --- |
| Service identifier | M | The identity of the application traffic for whom the network resource QoS control is requested |
| VAL UE ID(s) | M | One or more VAL UE IDs for whom the network resource is requested |
| Low latency requirements | M | The requirement description of low latency (e.g., UL, DL and/or round-trip latency) |
| High bandwidth requirements | M | The requirement description of high bandwidth (e.g., requested maximum bitrate, requested guaranteed bitrate, media type, media format, bandwidth) |
| High reliability | M | The requirement description of high reliability (e.g., request packet error rate) |

Table 8.1.6.1.1.5-8 describes the information flow Nrm_QosCtrlLowLatencyHighBWHighReliability create response from the NRM server to the consumer.

Table 8.1.6.1.1.5-8: Nrm_QosCtrlLowLatencyHighBWHighReliability create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource reservation with the underlying network. |

8.a.1.3	NRM services

The following new NRM service listed in table 8.a.1.3 are introduced:

Table 8.a.1.3-1: List of new NRM services for network resource control

| Service Name | Known Consumer(s) | Communication Type |
| --- | --- | --- |
| QoS control service for network transport with high data rate and low latency | AS | Request/response |
| QoS control service for network transport with low latency | AS | Request/response |
| QoS control service for network transport with low latency and high reliability | AS | Request/response |
| QoS control service for network transport with low latency & bandwidth & high reliability | AS | Request/response |

Editor’s note:	An alternative solution based on a single merged API, and the evaluation of the multi-API and single-API solutions, are FFS.

8.1.6.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

This solution creates multiple NRM services for network service QoS control at NRM.

8.1.6.3	Solution evaluation

This solution provides several new NRM services for network service QoS control to the application service providers/developers. Each of the new NRM service represents a different QoS service catalog and allow the application stakeholders to choose a proper QoS control service corresponding to their application requirements.