3GPP TSG-SA WG6 Meeting #67	S6-252473

Fukuoka, Japan 19th – 23rd May 2025                                                         was S6-252237

Source:	Nokia

Title:	Pseudo-CR on technical solution based on single API for NRM service for applications

Spec:	3GPP TR 23.700-35 v0.4.0

Agenda item:	8.9

Document for:	Approval

Contact:	Rajesh Babu Natarajan rajesh_babu.natarajan@nokia.com

- Introduction

This pCR is proposed to add a new Technical solution to address a technical gap identified in clause 4.1.

- Reason for Change

Clause 4.1 describes a technical gap related to NRM services and technical solution #6 (clause 8.1.6) is proposed to address this gap with an approach that proposes to define separate APIs based on different QoS requirements/use cases.

The technical solution presented in this pCR is an alternative method, where we can address the above identified gap, by using a single API itself for all QoS requirements/use cases related to applications, and with proposed simplification.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.4.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS)" 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[12]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[13]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[14]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[x]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[y]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

* * * Next Change * * * *

## 8.x	Technical Solution # x: Single NRM service for network service QoS control for applications

### 8.x.1	Solution description

This solution proposes to address the technical gaps identified in clause 4.1 regarding the support for various application requirements at NRM.

To achieve an API design involving different use cases/application with different QoS characteristics, the proposed solution is to consider a single API and with inputs – defining the basic QoS requirements as highlighted below:

Target QoS requirements:

- Data rate

  - Maximum Uplink Data rate

  - Maximum Downlink Data rate

- Delay Characteristics

  - Maximum Uplink Delay

  - Maximum Downlink Delay

  - Round trip Delay

  - Delay Variance

- Maximum Packet Loss Rate

NOTE: Either Maximum Uplink Delay and Maximum Downlink Delay or Round trip Delay can be provided.

#### 8.x.1.1	Procedures

Editor’s note:	The procedure and information flows to be added is FFS.

#### 8.x.1.3	NRM service

The proposal is to use a single NRM service for QoS control.

The following table 8.x.1.3-1 provides example of QoS requirements and the corresponding applications:

Table 8.x.1.3-1: List of examples QoS requirements and the corresponding applications

| Max Data rate |  | Delay Characteristics |  |  |  | Max Packet loss rate | Applications |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink | Downlink | Max Uplink Delay | Max Downlink Delay | Round trip Delay | Delay variance |  |  |
|  |  | 100 Mbps - 1 Gpbs |  |  |  |  |  | 10 ms |  | 30 ms |  | 0.01 |  | Consumption of VR content via tethered VR headset (NOTE 1) |  |
| 5 Mbps |  | 100 Kbps |  | 50 ms |  | 50 ms |  |  |  | 20 ms |  | 0.001 |  | IoT (Factory sensors) (NOTE 2) |  |
| 10 Mbps |  | 10 Mbps |  | 20 ms |  | 20 ms |  |  |  | 20 ms |  | 0.001 |  | IoT (wearables) (NOTE 2) |  |
| 600 Mbps |  | 600 Mbps |  |  |  |  |  | 300 ms |  | 30 ms |  | 0.01 |  | Immersive VR navigational application (NOTE 3) |  |
| 25 Mbps |  | 1 Mbps |  |  |  |  |  | 5 ms |  | 10 ms |  | 0.001 |  | V2X application (NOTE 4) |  |
| NOTE 1: Table 7.6.1-1 in TS 22.261 [x]. NOTE 2: Table 7.7-1 in TS 22.261 [x]. NOTE 3: Table 7.11-1 in TS 22.261 [x]. NOTE 4: Table 5.5-1 in TS 22.186 [y]. |  |  |  |  |  |  |  |

### 8.x.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

### 8.x.3	Solution evaluation

Editor’s note:	The solution evalution to be added is FFS.

* * * End of  Change * * * *